FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Tenth Working Group meeting

Virtual Meeting, 17 - 26 August 2020

Agenda Item xx

WORKING PAPER

PROPOSED AMENDMENT TO ANNEX 10, VOLUME IV, PART 2 SARPS ON

DETECT AND AVOID

(Presented by the Secretary)

SUMMARY

RPASP WG-3 has developed a draft new Part 2 to Annex 10, Volume IV, proposing SARPs on detect and avoid. With this working paper, the amendment is provided to the FSMP for review. The working paper further provides relevant information for a better understanding of the proposed amendment.

- INTRODUCTION

  - Job card RPASP.003.05, ID 1506, tasked the RPASP WG-3 with developing proposals for Standards and Recommended Practices (SARPs) relating to detect and avoid (DAA) for inclusion in Annex 10, Volume IV, as a new Part 2. The proposed amendment is included in Appendix A to this Working Paper.

- DISCUSSION

  - The proposed amendment currently only addresses the risk of a mid-air collision, while ensuring seamless integration of remotely piloted aircraft (RPA) within the current air traffic management (ATM) system. The other hazards identified in the Manual on RPAS (Doc 10019), i.e. terrain and obstacles, hazardous meteorological conditions, collision during ground operations and other airborne hazards, including wake turbulence, wind shear, birds or volcanic ash, are not covered by the proposed amendment.

  - To the extent that the other hazards can be mitigated by technical means, it is intended to address these in later amendments to Annex 10. It should however be noted that some of the hazards can be mitigated through operational procedures and/or systems that exist today, but for which no specific Annex 10 requirements exist.

  - The SARPs have been drafted within the scope of international instrument flight rules (IFR) operations. RPASP WG-3 worked under the assumption that the RPA will be able to operate on airways, standard instrument departures (SIDs) and standard terminal arrival route (STARs) and follow instructions provided by air traffic control (ATC).

  - DAA, by nature, assumes a certain predictability of the path of the intruder.

  - Current Annex 10 SARPs are often exceptionally detailed. Historically the SARPs have been based on recognized industry standards. For DAA, existing standards may still require validation or are still under development or lack validation. For legal and practical considerations, including allowing for the further development of these standards, the DAA SARPs were intentionally kept at a high, mostly objective based, level.

  - The DAA industry standards will have to be validated to ensure that they meet the performance objectives and do not cause undue disturbances to the ATM system. This process is similar to the validation of airborne collision avoidance system (ACAS) and is, or will, be performed both in the U.S. and the EU. Regrettably, this process, which is well established, has never been described in much detail. Only recently was an agreement reached with the Aircraft Collision Sub-Group (ACSG) of the Surveillance Panel on describing this validation process, which will be included in an attachment to the proposed DAA SARPs. It will take the ACSG until the third quarter of 2020 to complete this work. In the interest of timely accomplishment of coordination with the relevant ICAO expert groups, including panels, the attachment is currently included only as a placeholder.

  - In drafting the DAA SARPs, RPASP WG-3 assumed a layered approach with three levels, whereby the SARPs provide the high level, objective-based requirements. The SARPs will be complemented by a DAA Manual, which will provide context, background, guidance and references, including references to Minimum Aviation System Performance Standards (MASPS) and Minimum Operational Performance Standards (MOPS), developed by Standards Development Organizations. This approach accounts for the relatively volatile nature of current developments and allows ICAO to respond to changes more rapidly than would be possible if direct references to MOPS were made in the SARPs. The proposed amendment to the SARPs makes a few exceptions to the layered approach. One noticeable exception is the reference to EUROCAE ED-264/RTCA DO-376 - Minimum Aviation System Performance Standards (MASPS) for the Interoperability of Airborne Collision Avoidance Systems (CAS), which is referenced to ensure interoperability between the Collision Avoidance function of the DAA and ACAS systems currently in operation, as well as future versions.

  - The design of DAA systems in accordance with the interoperability requirements of EUROCAE ED-264/RTCA DO-376 and the verification and validation processes described in paragraph 2.6, requires much more effort than any single manufacturer is generally capable of performing. In addition, manufacturers are not keen to be held liable for any consequences that failures in the development of the algorithms may have. The design of DAA systems is therefore expected to follow a process very similar to the design of ACAS systems, whereby the system specifications, including the Collision Avoidance and Remain Well Clear algorithms, are developed through a Standards Development Body. The verification and validation exercise forms an integral part of this process. Due to the excessive costs, the latter process is often sponsored by States, other governing bodies and major stakeholders. The resulting MOPS include a specification of the verified and validated algorithms.

  - Manufacturers of DAA systems would be expected to design to the MOPS. Indirectly, this ensures interoperability of the system with ground infrastructure and procedures used by ATC for surveillance as well as with ACAS. It also ensures that the verification and validation processes described in the future attachment referred to in paragraph 2.6 above is complied with.

  - The proposed amendment includes provisions for automated execution of collision avoidance manoeuvres in the case of a lost C2 Link situation. In line with the decisions of the RPASP, the provisions of this proposed amendment to Annex 10, Volume IV, Part 2 SARPs recommend automated execution of collision avoidance manoeuvres, but leave room for other solutions.

  - The RPASP is submitting the proposed amendment to Annex 10, Volume IV, Part 2 to the FSMP to allow for timely formal coordination, in support of delivering the proposed Amendment in the first quarter of 2022.

- Action by the meeting:

  - The FSMP is invited to:

    - note the contents of this WP; and

    - review the attached draft of the proposed Annex 10, Volume IV, Part 2, SARPs on detect and avoid and provide comments, using the attached comment sheet, to the RPASP by 26 February 2021.

Attachments

- Annex 10, Volume IV, Part 2, proposed SARPs

- FSMP - Draft Annex 10, Volume IV, Part 2, Comment Matrix

- Slides providing background information on the draft SARPs

— END —