|  | International Civil Aviation Organization DRAFT REPORT | 25 February, 2022. |
| --- | --- | --- |

FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

THIRTEENTH MEETING OF THE WORKING GROUP OF FSMP

(FSMP-WG/13)

Virtual

21-25 February, 2022

DRAFT REPORT

1.	Opening and Working Arrangements

1.1	The electronic/virtual meeting was opened by Mr Loftur Jonasson from the ICAO Secretariat, Montreal and Mr Mike Biggs, the Rapporteur of Working Group FSMP (FSMP-WG).  Mr Jonasson acted as the Secretary of the meeting. Mr Biggs welcomed the group and provided introductory remarks and meeting information. Lastly, the Rapporteur of the Working Group expressed his appreciation for membership providing input contributions in a timely manner, allowing the development of a more consistent meeting schedule.

1.2	The meeting was held in English. After the opening of the meeting the agenda was approved by the group and it is contained in Appendix A.

1.3	The list of papers submitted for consideration by FSMP-WG/13 is contained in Appendix B. Any papers denoted with an asterisk (“*”) are available on the closed website(s). The list of participants is in Appendix C.

1.4	The material in this report is organized by meeting agenda item number, and does not necessarily reflect the order of discussions.  The meeting conducted a review of the actions from the last meeting. Actions captured during discussions are shown in Appendix D, together with status of prior-meeting(s) actions.

2.	Agenda Item 2 –ICAO WRC-23 Position

2.1	Identified conflicts between administration preliminary proposals and ICAO Position

2.1.1	No contributions were received on this topic

2.2	Discussion of possible future agenda item proposals from administrations

2.2.1	No contributions were received on this topic

2.3	Modifications/Updates to the ICAO WRC-23 Position

2.3.1	WP06 and WP11 were both dealing with the ICAO WRC-23 AI1.8 Position, so both documents were introduced and a single discussion was held.  WP11 raised concerns on how some topics are addressed, however proposed not to change the ICAO position at this time. In contrast, WP06, in line with points raised in WP05, proposed significant changes to the ICAO position for WRC-23, and suggested there is a significant risk from the perspective of the writer of WP06 that no agreement will be reached on AI 1.8 and then Resolution 155 might be withdrawn during WRC-23. After in-depth discussion the meeting agreed to keep the current ICAO Position as it was for the time being. Depending on continuing offline work, the changes proposed in WP06 may be reconsidered at FSMP-WG/14.

3.	Agenda Item 3 – Development of (planned) material for ITU-R studies

3.1	WRC-23 AI 1.6 Suborbital Vehicles

3.1.1	WP02 provided a possible Resolution as a way to resolve WRC-23 Agenda Item 1.6. The purpose of the paper was to socialize the approach and get comments. The meeting provided comments, and in particular that resolves 3 may be seen to deal with air traffic management business that is not under the responsibility of spectrum regulation, and so it may need further considerations. The proponent indicated that the paper would be updated for submission to the upcoming ITU-R Working Party 5B (WP5B) meeting.

3.2	WRC-23 AI 1.7 VHF AMS(R)S

3.2.1	IP06 and its attachment presented updated information regarding the European initiative called VOICE. The object of that project is to perform a proof of concept for a Satellite-Based CNS technology in a real operational environment, to include satellite relay of VHF voice and data. The project is to be completed by the end of 2022. Some initial outcomes of the technical studies and test/validations performed by VOICE were presented as well as a section on frequently asked questions (FAQs).

3.2.2	WP01 was intended to trigger a discussion on some technical considerations related to the potential relay via satellite of VDL Mode 2 communications in the frequency band 136-137 MHz.  In particular the contribution suggested some elements to be discussed and to further consolidate in FSMP in if ICAO wanted to send a reply to ITU-R WP5B.

3.2.3	WP13 reported that progress had been made at the recent meeting of ITU-R WP 5B by including technical interference studies on several technologies (voice, data-link) and covering the whole band 117.975-137 MHz towards a potential AMS(R)S allocation. Also, CPM text (regulatory text for the WRC 23 to take decisions) has been drafted. This text includes several alternative methods to satisfy the WRC-23 agenda item.

3.2.4	WP15 discussed several areas of consideration for the coordination of any new AMS(R)S frequencies with existing and future AM(R)S assignments.  It presented several questions for the work of the FSMP, PT-T, and FVSG, together with implications for the ICAO AI 1.7 position for WRC-23.

3.2.5  	All of the papers were introduced and a general discussion was held. While the meeting appreciated the updates on the ongoing studies, a number of questions were raised. After extensive conversation, the following points were agreed:

- At this point the meeting would not be able to reach agreement as to the extent of the AMS(R)S allocation (e.g., should 136-137 MHz be included). As a result, the aviation goal for the March/April 2022 WP5B meeting should be to ensure that any CPM text developed includes all options;

- A satellite VHF coordination group (CG-SV) should be formed to allow for continued information exchange and to allow for new questions to be considered and agreed responses developed. Goal would be a comprehensive agreed set of FAQs. The group will be co-coordinated by Manuel Garcia Martín and Olivier Pellay. [AI 13-04]

3.3	WRC-23 AI 1.8 Resolution 155/FSS for UAS

3.3.1	WP04 noted that the links between the UAS control station and the satellites meet the definition of FSS and as such do not need to be included in Resolution 155 (rev. WRC-19). In addition, since similar links are already used as feeder links for AMS(R)S applications, the question of the safety aspects has already been addressed. As a result, the paper suggested removing those links from the Resolution. During discussion it was noted that frequency bands other than those listed in resolves 1 might be used for the control station-to-satellite links, providing another possible benefit of accepting the paper suggestion. The meeting asked to see the specific changes proposed, and the author agreed to circulate them to interested parties offline for consideration before the next FSMP WG.

3.3.2	 WP05 noted that, at its 3rd meeting, CPG-23 updated the preliminary CEPT position on agenda item 1.8 of WRC-23. In addition, several contributions were received at the November-December 2021 WP 5B that are reflecting a common approach on the way to deal with Article 4.10 of the RR and other aspects on ensuring the safety aspects of using FSS for UAS control. Finally, in parallel, Annex 6 of the draft RPAS SARPs has identified that the State providing oversight of the UAS operator is the only responsible State for ensuring the safety of the link. The WP was also seeking to get confirmation that the approach described in it on the future revision of Resolution 155 in regards with the safety of the UA CNPC links would be acceptable by ICAO for operating CNPC links with regular FSS. When considering the proposed terminology “deactivation” for Article 4.10, discussions with different views took place about the status of Article 4.10 of the Radio Regulations for the FSS. The WP also suggested that ICAO liaise to WP5B that this approach, including that “RR No. 4.10 does not apply”, could be accepted by ICAO. There was no agreement by the meeting on the overall approach, or to provide such a liaison to WP 5B.

3.3.3	IP01 reported on the status of remotely piloted aircraft systems (RPAS) Panel work regarding the SARPs and Manual for RPAS C2 links. In particular it was noted that the SARPs are high-level and technology-neutral, and that any specific technologies would be explained in the the supporting ICAO Manual. The SARPs are undergoing informal and formal coordination within RPASP and with other Panels, with the goal of being completed by the end of September 2022.

3.4	WRC-23 AI 1.9 Wideband HF

3.4.1	WP07 provided an analysis of Appendix 27 of the Radio Regulations (RRs). That analysis concluded that wideband HF systems that were implemented by “stitching together” a number of standard 3 KHz channels, each of which met the requirements of Appendix 27, would be accommodated under the existing Appendix 27 without requiring text modification. During discussion the meeting raised issues which needed to be explored such as whether the current  allowed emission classes would accommodate digital transmissions, whether the channel mask would accommodate digital modulations, and whether digital signals would have more impact on existing analog systems than the current analog-on-analog situation. The proponent noted that they would review the inputs and provide an updated paper to WP5B.

3.5	WRC-23 AI1.10 Non-safety AMS

3.5.1	WP08 with its attachment provided an outline of the current studies being performed in ITU-R on Agenda Item 1.10. The meeting appreciated the information.

3.6	WRC-23 AI9.2 Radio Regulations clean-up

3.6.1	No contributions were received on this topic.

3.7	Other

3.7.1	WP09 provided a comparison of the spurious emissions requirements contained in Appendix 3 of the RR and those contained in Recommendation ITU-R SM.1541. The conclusion was that the requirements do not match, and that the issue should be raised within ITU-R either through WP5B or directly to Study Group 1. The meeting supported the idea, but cautioned that it needed to be done in a manner that did not impact already-fielded aeronautical systems.

4.	Agenda Item 4 – Radio Altimeter issues

4.1	Report from correspondence group on radio altimeters (CG-RA)

4.1.1	IP03 provided the report of the chairman of the correspondence group –radio altimeters (CG-RA), and it included a comprehensive listing of 5G implementation activities in different countries. The report generated considerable discussion and the meeting appreciated the information and agreed that the work of the CG-RA should continue.

4.2 	National efforts to implement broadband mobile near 4200-4400 MHz

4.2.1	WP03 provided information on efforts in France to ensure compatibility of 5G with radio altimeters (RAs). This included a report on testing of an RA on a helicopter and on plans to develop a 5G roadmap to inform development of MOPS for next-generation RAs. The paper generated considerable discussion, in particular on the concept of what is “good enough” rejection of adjacent-band signals for a receiver. The discussion resulted in an action item to national spectrum regulators that were contributors to the work of the FSMP to provide examples of standards/systems for adjacent-band signal rejection that could be provided to the next FSMP meeting (AI13-01)

4.2.2	WP16 discussed recent lessons-learned concerning the deployment of 5G telecommunication networks and outlined the need for ICAO material to assist States in mitigating any potential risks to flight safety by 5G deployments, and to support safe co-existence between 5G and radio altimeters. The paper generated considerable discussion including the following;

- groups such as the European PT-1 could be utilized to bring aviation and wireless proponents together, and as a result the meeting was encouraged to participate in such groups to the extent possible;

- the meeting asked that a dedicated site on the FSMP website be defined to place important reference material such as complete studies submitted to State regulators, or material explaining RA operation; (AI13-02)

- the CG-RA was tasked with developing an “Information for States” document that contained material such as explaining the 5G/RA issue and providing guidance as to where additional information can be found. (AI13-03)

4.2.3	IP02 provided the results of trials to measure interference path loss (IPL) between a radio altimeter receiver and a 5G emitter for a medium sized helicopter. The measurements were carried out based on RTCA DO-307A procedures with the 5G emitter assumed to operate across the frequency range 3.5 GHz–4.7 GHz to cover both the 4.2-4.4 GHz band used for radio altimeters and Wireless Avionics Intra-Communication (WAIC), and the sub-6 5G mobile communication bands in Japan (3.5–4.1 GHz, 4.5–4.7 GHz). The transmitting antenna was tested both inside and outside the cabin, resulting in minimum IPL values of 49.2 dB and 55.1 dB, respectively. The minimum IPL are obtained when the transmitting antenna and the receiving antenna are in line-of-sight conditions, and when the 5G is operating inside the 4.2-4.4 GHz band. When the 5G is operating outside of that band, an addition attenuation (up to approximately 13 dB) is measured. The meeting greatly appreciated the information and asked to be kept informed if additional measurements are made.

4.2.4	IP04 contained informal briefing material, as developed for use by the Secretariat in January 2022, to further supplement the information provided in State Letter 21/22 regarding compatibility between 5G and radio altimeters. The meeting appreciated having the reference material

5.	Agenda Item 5 – Any other business

5.1	WP10 informed the FSMP on a change in the European regulations with respect to placing aeronautical equipment on the market within Europe. In particular the meeting was asked to consider the impact of these regulations on the development of standards for aeronautical radio equipment and endorse that this paper be submitted to the RPASP for further consideration due to potential impacts on SARPs for UAS command and non-payload communication as well as detect and avoid systems operating in non-aeronautical frequency bands. The meeting appreciated the information.

5.2	WP12 proposed that new aeronautical radio equipment should be designed to meet or be better than current radio regulatory requirements and guidance material provided within the ITU or other recognised body. In particular the meeting was asked to support the principle that standards for new and revised aeronautical systems should be designed to push the economic/performance trade-off boundaries.  The meeting agreed in principle and the author agreed to develop a list of existing aviation equipment transmit/receive masks to progress and inform the effort [AI 13-05]. It was also noted in this regard that a new RTCA/EUROCAE group was being stood up to try and (a) catalog existing equipment interference rejection, and (b) develop guidelines for interference rejection requirements for future aviation equipment. (see https://www.rtca.org/wp-content/uploads/2022/02/SC242__WG124__2022_March-Calling-Notice_Final_Publlc.pdf)

5.3	WP14 contained an update of the draft Standards and Recommended Practices (SARPs) for WAIC systems currently under preparation by the FSMP WG. The paper notes that following the review at FSMP-WG/12 of WAIC implementations under agreeable power limits, the WAIC SARPs Correspondence Group (CG-WAIC) was directed to further consider remaining concerns with the content of the SARPs and to prepare an updated draft containing the consensus recommendations of the CG-WAIC (see AI12-01). The draft SARPs text contained in the Annex to the document updated the material concerning unwanted emissions and out-of-band susceptibility to bring the SARPs in compliance with existing ITU-R guidance, and as a result the CG-WAIC recommends that this draft, along with the related Impact and Implementation Statement and Validation Report, be approved by the Panel and promoted for incorporation in the Convention. After discussion the meeting agreed that the material was acceptable for formal approval by the Working Group and promotion to the Panel at the next meeting, pending any final comments provided at FSMP-WG/14. The WAIC Correspondence Group (CG) will continue to meet to address any remaining concerns prior to that meeting. All FSMP/WG members are encouraged to provide comments to the CG no later than 4 weeks prior to that meeting so that the final WP paper providing formal approval can be prepared. FMSP/WG members are encouraged to attend CG meetings and contact David Redman (dredman@tamu.edu) to be added to the correspondence. [AI13-06]

5.4	IP05 noted the recent discussions within the FSMP with respect to the spectral mask that should be applied to WAIC. After considering the regulatory requirements, the paper identifies the minimum and two alternative masks that could be applied, and that the proposed mask in WP14 provides the best performance of the three.

5.5        At the request of the Communications Panel (CP), the draft revisions Annex 10, Volume III, Section 4.3.1.1 (see Appendix I of the FSMP-WG/12 Report) were considered. The meeting agreed to “Option 2” and deletion of the existing Note, with the suggestion to the CP to make sure the referenced “AMS(R)S Communications” were adequately defined somewhere in the Annex.

OPTION 2: Modify the text to read:

When providing AMS(R)S communications, an AMS(R)S system shall operate only in frequency bands which are appropriately allocated on a primary basis to the :

- AMS(R)S,

or

- the MSS under the condition that

- in at least some portion of the operational frequency band of the system there is a clear indication in the ITU Radio Regulations that priority is given to accommodating the spectrum requirements of the AMS(R)S through priority access and immediate availability, by pre-emption, if necessary, over all other mobile-satellite communications operating within a network in that portion of the frequency band,

and

- there is the capability in  the case of interference for the system to fall back to operating only in that portion of the frequency band that meets the provisions defined in a. above.

Delete the existing note:

Note.— ITU Radio Regulations permit systems providing mobile-satellite service to use the same spectrum as AMS(R)S without requiring such systems to offer safety services. This situation has the potential to reduce the spectrum available for AMS(R)S. It is critical that States consider this issue in frequency planning and in the establishment of national or regional spectrum requirements.

6.	Date of next meeting and administrative issues

6.1	The FSMP-WG/14 meeting is scheduled for April 25-29, 2022 in virtual format. It was also noted that a third FSMP 2022 meeting is planned for August/September 2022, with dates and location to be determined. [15-26 Aug (10 days), 22 Aug-1 Sept (9 days), 12-23 Sept (10 days) are the current options under consideration. Location is tentatively Montreal, but rooms have not been secured yet.]

6.2 	The meeting discussed the best way to hold the regional WRC Workshops. One suggestion was to have a virtual Workshop for a 4 hour period tied to the local time for the intended regional office. In general the meeting agreed, however finding an acceptable schedule would be key.

6.3	The meeting was reminded that as agreed by FSMP-WG/12, papers for FSMP-WG/14 are due to the ICAO Secretariat one week prior to the meeting’s first day. In exceptional circumstances, a framework detailing at minimum the paper summary and introduction will be provided by the deadline, with the full paper provided no later than the day before the start of the meeting. Any papers received after this deadline will not be accepted unless agreed by the meeting on the first day. No attendee contributions received after the first day of the meeting will be accepted.

APPENDICES

Appendix A – Agenda

Appendix B – List of Working Papers, Information Papers and Flimsies

Appendix C – List of Participants

Appendix D – Action Item List

Appendix E – Draft WAIC SARPs

APPENDIX A

INTERNATIONAL CIVIL AVIATION ORGANIZATION

thirteeth working Group Meeting of the

Frequency Spectrum Management Panel

(FSMP-WG/13)

(Virtual, 21-25 February, 2022)

DRAFT AGENDA

Agenda Item 1		Opening and Working Arrangements

Agenda Item 2		ICAO WRC-23 Position

a) 	Identified conflicts between administration preliminary proposals and ICAO Position

b)	Discussion of possible future agenda item proposals from administrations

c)	Modifications/Updates to the ICAO WRC-23 Position

Agenda Item 3		Development of (planned) Material for ITU-R Studies on:

a)	WRC-23 AI1.6 Suborbital Vehicles

b)	WRC-23 AI1.7 VHF AMS(R)S

c)	WRC-23 AI1.8 FSS for UAS

d)	WRC-23 AI1.9 Wideband HF

e)	WRC-23 AI1.10 Non-safety AMS

f)	WRC-23 AI9.2 Radio Regulations clean-up

g)	Other

Agenda Item 4		Radio Altimeter issues

b)	  Report from correspondence group on radio altimeters (CG-RA)

a)	  National efforts to implement broadband mobile near 4200-4400 MHz

- Mitigation measures

- Safety Cases/Compatibility Analyses

Agenda Item 5		Any Other Business

APPENDIX B

List of Papers

“*” papers available on closed website(s)

List of Working Papers

| Paper # | Source | Title | Agenda Item |
| --- | --- | --- | --- |
| 1 | C. Fleury and O. Pellay | ELEMENTS OF DISCUSSION ON WRC-23 AGENDA ITEM 1.7 | 3b |
| 2 | C. Fleury and J. André | Draft new Resolution to respond to the WRC-23 AI 1.6 on suborbital vehicles | 3a |
| 3 | C. Fleury and J. André | NATIONAL EFFORTS ON COEXISTENCE WITH RADIOALTIMETERS WITHIN FRANCE | 4a |
| 4 | C. Fleury and J. André | ELEMENTS OF DISCUSSION ON SAFETY OF FSS FOR LINKS BETWEEN UA CONTROL EARTH STATIONS ON THE GROUND AND SATELLITES | 3c |
| 5 | J. André | ELEMENTS OF DISCUSSION ON WRC-23 AGENDA ITEM 1.8 | 3c |
| 6 | J. André | Proposal to update the draft ICAO WRC-23 Position on AI 1.8 | 2c |
| 7 | C. Fleury and J. André | ELEMENTS OF DISCUSSION ON PROGRESS OF THE WORK ON AGENDA ITEM 1.9 | 3d |
| 8 | A. Marquet and J. André | ELEMENTS OF FSMP WRC-23 WORKSHOP AND FOR DISCUSSION ON PROGRESS OF THE WORK ON AGENDA ITEM 1.10 | 3e |
| 9 | J. Mettrop | Anomalies Between the Spectral Mask for Aeronautical Mobile Systems as Defined by the Radio Regulations and Recommendation SM.1541 | 5 |
| 10 | J. Mettrop | Radio Equipment Directive and it’s Implications for Aeronautical Standards | 5 |
| 11 | J. Mettrop | Support for Maintaining the Position as Currently Stated and Concerns as to Whether Current Proposals Resolve the Issues of Concern | 2c |
| 12 | J. Mettrop | Specification of New or Revised Transmitter and Receiver Standards | 5 |
| 13 | ENAIRE, Indra, SITA and EUROCONTROL | ELEMENTS OF DISCUSSION ON WRC-23 AGENDA ITEM 1.7 | 3b |
| 14 | D. Redman | Update on Draft SARPs for Wireless Avionics Intra-Communications (WAIC) | 5 |
| 15 | A. Roy | CONSIDERATIONS FOR COORDINATION BETWEEN EXISTING AM(R)S AND PROPOSED AMS(R)S SYSTEMS | 3b |
| 16 | N. Pringvanich | ICAO Material to Support Safe Co-existence between 5G and Radio Altimeters | 4a |
| 17 |  |  |  |
| 18 |  |  |  |
| 19 |  |  |  |
| 20 |  |  |  |
| 21 |  |  |  |
| 22 |  |  |  |
| 23 |  |  |  |
| 24 |  |  |  |
| 25 |  |  |  |
| 26 |  |  |  |
| 27 |  |  |  |
| 28 |  |  |  |
| 29 |  |  |  |
| 30 |  |  |  |

List of Information Papers, Presentations and Flimsies

| Paper # | Source | Topic | Agenda Item |
| --- | --- | --- | --- |
| 1 | RPASP WG2 | SARPs and Manual on C2 Links for RPAS, Status Update | 3c |
| 2 | S. Futatsumori (etal) | Radio altimeter interference path loss measurements of helicopter | 4a |
| 3 | C. Fleury | Update of the CG-RA Report FSMP-WG/12 IP08 | 4b |
| 4 | Secretary | Briefing material on the Radio Altimeter and 5G rollout issues | 4a |
| 5 | J. Mettrop | Derivation of the WAIC Spectral Mask | 5 |
| 6 | ENAIRE, Indra, SITA and EUROCONTROL | Technical studies and laboratory tests performed by VOICE project implementing Satellite-Based CNS concept | 3b |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |

| PRESENTATIONS |  |  |  |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |

| FLIMSIES |  |  |  |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |

APPENDIX C

Thirteenth meeting of the Working Group of the Frequency Spectrum Management Panel

(FSMP-WG/13)

Virtual meeting (e-meeting),    21-25 February, 2022

LIST OF PARTICIPANTS

| COUNTRY/ ORGANIZATION | NO. | NAME | DESIGNATION-JOB TITLE/ TELEPHONE | EMAIL |
| --- | --- | --- | --- | --- |
| AUSTRALIA |  | Matthew Kelly | Air Services Australia | matthew.kelly@airservicesaustralia.com |
| BRAZIL |  | Vahe Antoine Yaghdjian | Telecommunications Engineer Department of Air Space Control – DECEA Technical Subdepartment – Technical Coordination Division Rio de Janeiro - Brazil Tel: +21- 2101–6487/21-99955–3305 | vahevay@decea.gov.br vahe.antoine@gmail.com |
|  |  | Gerson Monteiro Siqueira | CNS Advisor Department of Air Space Control – DECEA Technical Subdepartment Rio de Janeiro - Brazil Tel: +55 21- 2101–6665 | monteirogms@decea.gov.br |
|  |  | Wallace Gutemberg Medeiros Luz | CNS Advisor Department of Air Space Control – DECEA Technical Subdepartment Rio de Janeiro - Brazil Tel: +55 21- 2101–6579 | gutembergwgml@decea.gov.br |
|  |  | Carlos Evangelista da Silva Junior | Specialist on Regulation of Telecommunications Services ANATEL - Agência Nacional de Telecomunicações | carlosjunior@anatel.gov.br |
|  |  | Agustinho Linhares de Souza Filho | Manager of Spectrum, Orbit and Broadcasting ANATEL Brazil | linhares@anatel.gov.br |
| CANADA |  | Michel Drolet | Air Navigation Standards Inspector Transport Canada Tel: +1 613-991-3325 | michel.drolet@tc.gc.ca |
|  |  | John Taylor | Aeronautical Spectrum Regulations Specialist Canada Tel: +1-613-993-4061 | john@johntaylorconsulting.ca |
|  |  | Milivoje Jevtovic | Manager, Spectrum Management Nav Canada Tel. +1 613 248 7275 | Milivoje.Jevtovic@navcanada.ca |
|  |  | Seyed Rastaghi | Nav Canada | Seyed.Rastaghi@navcanada.ca |
| CHINA |  | Zhao Liang | FSMP Member Aviation Data Communication Corporation of CAAC | zhaoliang@adcc.com.cn |
|  |  | Cui Kaitao | Advisor Civil Aviation Administration of China | cuikaitao@caacsri.com |
|  |  | Liu Rui | Aviation Data Communication Corporation of CAAC | liur@adcc.com.cn |
| FRANCE |  | Christian Fleury | IESSA - Spectrum Manager France DGAC/ Direction des services de la Navigation Aérienne Toulouse – France Tel: +33-0-562-14-55-41 Cell: +330625164420 | christian.fleury@aviation-civile.gouv.fr |
|  |  | Ivan Martin | Thales SIX | ivan.martin@thalesgroup.com |
|  |  | Josep GINE i CORTIELLA | ATR | JOSEP.GINE-I-CORTIELLA@atr-aircraft.com |
|  |  | Jérome ANDRE | ANFR | Jerome.ANDRE@anfr.fr |
|  |  | Benoit LOUVET | Thales Alenia SPACE | Benoit.Louvet@thalesaleniaspace.com |
|  |  | Christine Mengelle | Thales Alenia Space | christine.mengelle@thalesaleniaspace.com |
|  |  | Olivier Pellay | Airbus | olivier.pellay@airbus.com |
|  |  | Alexandre MARQUET | ANFR | alexandre.marquet@anfr.fr |
| GERMANY |  | Dr. Felix Butsch | Head of Frequency Management DFS Deutsche Flugsicherung GmbH, (German Air Navigation Services) SIS/MF (Spectrum and Frequency Management Office) - Am DFS-Campus 10 D-63225 Langen, GERMANY Tel: +496103707-1533 Cell: +49-172 6677682 | Felix.Butsch@DFS.DE |
|  |  | Martin Weber | Federal Network Agency, Germany | Martin.weber@bnetza.de |
|  |  | René Woerfel | AUDENS Telecommunications, Germany | Rene.woerfel@audens.de |
|  |  | Christian Beitz | Federal Supervisory Authority for Air Navigation Services, Germany | Christian.Beitz@baf.bund.de |
|  |  | Sergio Bovelli | Airbus, Germany | sergio.bovelli@airbus.com |
| INDIA |  | Ajay Kumar Kapur | General Manager (CNS-Com) India / Airports Authority of India Tel: +91 9868815045 | akkapur@aai.aero |
|  |  | A. S. Yadav | Jt GM (CNS-Com) | ​asyadav@aai.aero |
|  |  | Umesh Kumar | AGM (CNS-Com) | umeshkc@aai.aero |
|  |  | Aklesh Kumar | AM (CNS-Com) | akikr@aai.aero |
| INDONESIA |  | Abdul Aziz | Directorate of Air Navigation (DGCA) Indonesia | azizsabdul@gmail.com |
| JAPAN |  | Kazuya Kawamura | Special Assistant to the Director JAPAN Civil Aviation Bureau Phone: +81-3-5253-8756 | kawamura-k2v8@mlit.go.jp |
|  |  | Takashi ITO | Special Assistant to the Director of the Division JAPAN Civil Aviation Bureau Phone: +81-3-5253-8735 | itoh-t2qm@mlit.go.jp |
|  |  | Kenji Watanabe | Special Assistant to the Director JAPAN Civil Aviation Bureau Phone: +81-3-5253-8735 | watanabe-k97iv@mlit.go.jp |
|  |  | Mitsuhiro Kawatsuhara | Chief Ministry of Internal Affairs and Communications Phone: +81-3-5253-5902 | m.kawatsuhara@soumu.go.jp |
|  |  | Naruto Yonemoto | Principal Researcher Japan Electronic Navigation Research Institute (ENRI) Tel: +81-422-41-3165 | yonemoto@mpat.go.jp |
|  |  | Hiroyuki Tsuji | Research Manager Japan National Institute of Information and Communications Technology (NICT) Tel: +81-42-327-6034 | tsuji@nict.go.jp |
|  |  | Shunichi Futatsumori | Japan Electronic Navigation Research Institute (ENRI) | futatsumori@mpat.go.jp |
|  |  | Toshiyasu Watanabe | Partner NTT DATA INSTITUTE OF MANAGEMENT CONSULTING, Inc. Phone: +81-70-6658-4695 | watanabet@nttdata-strategy.com |
|  |  | Akira Kurihara | Manager NTT DATA INSTITUTE OF MANAGEMENT CONSULTING, Inc. Phone: +81-80-8825-3744 | kuriharaa@nttdata-strategy.com |
|  |  | Naoya Takizawa | Consultant NTT DATA INSTITUTE OF MANAGEMENT CONSULTING, Inc. Phone: +81-90-6943-6566 | takizawan@nttdata-strategy.com |
|  |  | Sachiko Harada | Chief Information Researcher NTT DATA INSTITUTE OF MANAGEMENT CONSULTING, Inc. | haradas@nttdata-strategy.com |
|  |  | Atsushi Ozu | Director, Fixed and Satellite Radio Communications Division, Radio Department, Telecommunications Bureau Ministry of Internal Affairs and Communications TEL: +81-3-5253-5902 | a.ozu@soumu.go.jp |
|  |  | Toshio Nomi | Senior Engineer Japan Radio Air Navigation Systems Association (JRANSA) Tel: +81-3-5214-1353 | nomi.toshio@jransa.or.jp |
| NETHERLANDS |  | Gerlof Osinga | Director GSM Europe and MENA Boeing Netherlands BV Tel: +31 6 1534 0982 | gerlof.osinga@boeing.com gerlof.osinga@planet.nl |
|  |  | Maaike Jans | Netherlands | maaike.jans@ILenT.nl |
| PHILIPPINES |  | Charlemagne P. Gilo | Division Chief III Aeronautical Telecommunications Division AICD, ATS, CAAP | ats_atd@caap.gov.ph charlemagne.gilo@gmail.com |
|  |  | Jaime C. Solomon | Asst. Division Chief for Aeronautical Telecommunications | ats_mfss@caap.gov.ph |
| ROMANIA |  | Aurelian Sorinel Calinciuc | Spectrum Manager Vice-Chair ITU-R SG5 Tel: +40721291274 | aurelian.calinciuc@ancom.org.ro |
| SINGAPORE |  | Cheng Nam YEO | Consultant Aeronautical Telecommunications and Engineering Civil Aviation Authority of Singapore Tel: +65-9776-9376 | yeo_cheng_nam@caas.gov.sg |
|  |  | Henry FOO | Deputy Director (Aviation Technology and Development) Civil Aviation Authority of Singapore Tel: +65-8161-1958 | henry_foo@caas.gov.sg |
|  |  | John CHONG | Head (Masterplan & Enterprise Architecture) Civil Aviation Authority of Singapore Tel: +65-9369-6482 | john_chong@caas.gov.sg |
|  |  | Kok Pin PUAH | Head (Aviation Technology and Development) Civil Aviation Authority of Singapore Tel: +65-9116-0541 | puah_kok_pin@caas.gov.sg |
| SOUTH AFRICA |  | Lisa Tele | Senior Engineer/Project Manager ATNS Head Office Johannesburg - South Africa Tel: +2711 6071134 Cell: +2783 779 3920 | lisat@atns.co.za |
|  |  | Takalani Tshikalaha | ATNS | TakalaniT@atns.co.za |
|  |  | Thomas Matabane | Frequency Spectrum, CNS CAA South Africa Tel: +27 11 545 1066 Cell: +27 71 801 1507 | Matabanet@caa.co.za |
| SPAIN |  | Manuel Garcia Martín | División de Comunicaciones Jefe de División Tel: +34 913 213 261 | mangarcia@enaire.es |
|  |  | José Luis Chinchilla Garcia | INDRA | jlchinchilla@indra.es |
|  |  | Miguel Muñoz | INDRA | mmunozm@indra.es |
|  |  | Joan Manuel Cebrian | INDRA | jcebrian@indra.es |
|  |  | Nicolas Van Wambeke | SITA | Nicolas.Van_Wambeke@sita.aero |
| THAILAND |  | Chavalit Ithiapa | Communication Navigation and Surveillance Services Standard The Civil Aviation Authority of Thailand | chavalit.i@caat.or.th |
| UGANDA |  | David Macho | UGANDA Civil Aviation Authority | adtmacho@gmail.com |
| UNITED KINGDOM |  | John Ernest Mettrop | Spectrum Policy Specialist UK Civil Aviation Authority Tel: +44-0-1293 573477 | John.Mettrop@caa.co.uk |
|  |  | Stephen Parry | Spectrum Manager, UK NATS Tel: +44 1489 616 454 | stephen.parry@nats.co.uk |
|  |  | Kamlesh Masrani | Manager, Spectrum Inmarsat Global Limited Tel: +44 20 7728 1338 | Kamlesh.Masrani@inmarsat.com |
|  |  | Stephen Limb | UK Ofcom | stephen.limb@ofcom.org.uk |
| UNITED STATES |  | Michael Biggs | Senior Electronics Engineer Federal Aviation Administration USA Tel:+1-202-267-8241 | michael.biggs@faa.gov |
|  |  | Chris Tourigny | Federal Aviation Administration USA | chris.tourigny@faa.gov |
|  |  | Miles Bellman | General Engineer AJW-1C3, Spectrum Planning & International Team Tel +1 617 838-4620 | miles.e.bellman@faa.gov |
|  |  | Damon Ladson | Technology Policy Advisor United States of America Tel: +1 301 728 7253 | dladson@hwglaw.com |
|  |  | Michael Neale | Aces Inc. | michael.neale@aces-inc.com |
|  |  | Louis Bell | Electronics Engineer Federal Communications Commission Tel. +1 202 418 1641 | louis.bell@fcc.gov |
|  |  | Don Jansky | Jansky-Barmat Telecommunications Inc. USA Tel: +1 202-415-1834 | don@jansky-barmat.com |
|  |  | Dante Ibarra | WRC Director Federal Communications Commission USA Tel: +1- 202-230-1983 | Dante.ibarra@fcc.gov |
|  |  | Scott Kotler | Director, Technical Regulatory Affairs Lockheed Martin Arlington, VA 22202 USA Tel: +1-703-789-3923 | scott.kotler@lmco.com |
|  |  | Bruce Lamb | Telecoms Specialist NTIA Tel: +1 202-317-0036 | blamb@ntia.gov |
|  |  | Donald Nellis | Electronics Engineer Federal Aviation Administraton USA Tel. +1 202 267 9779 | Donald.Nellis@faa.gov |
|  |  | Gregory Baker | Federal Communications Commission USA | Gregory.Baker@fcc.gov |
|  |  | Eric P.Y. Lee | Electronics Engineer Terrestrial Team Lead (Acting), International Spectrum Policy Division NTIA - Office of Spectrum Management\] Mobile: +1 (202) 360-0836 | elee@ntia.gov |
|  |  | Sanjay Bajekal | Collins Aerospace | Sanjay.bajekal@collins.com |
|  |  | John Nelsen | John A Nelsen NELSEN Advisors LLC P.O. Box 553 Wayne, PA 19087-0553 USA Tel: +1 610-613-2058 | nelsenj@gmail.com |
|  |  | Michael Lewis | DLA Piper LLC Tel: +1-202-799-4042 | michael.a.lewis@dlapiper.com |
| ASECNA |  | Eric Damiba | CNS and MET Senior Manager ASECNA | damibaeri@asecna.org |
| ASRI |  | Andrew Roy | Director of Engineering Aviations Spectrum Resources Inc. Tel: +1-443-951-0340 | ACR@asri.aero |
|  |  | Kris Hutchison | Aviations Spectrum Resources Inc. Tel: +1 443 951 0322 | KEH@asri.aero |
|  |  | Nic Shrout | Aviations Spectrum Resources Inc. | NJS@asri.aero |
| EUROCAE |  | Anna Guégan | Technical Programme Manager EUROCAE Le Triangle 9-23 rue Paul Lafargue 93200 Saint-Denis France | anna.guegan@eurocae.net |
| EUROCONTROL |  | Raffi Khatcherian | Head of International Spectrum Management EUROCONTROL Tel: +32 478241421 | raffi.khatcherian@eurocontrol.int |
|  |  | John Micallef | EUROCONTROL | john.micallef@eurocontrol.int |
| IATA |  | Noppadol Pringvanich | Global Head Air Traffic Management Engineering and Aviation Radio Spectrum International Air Transport Association Montreal, QC - Canada Tel: +1-438- 258-4714 | pringvanin@iata.org |
|  |  | Abhishes Dhakal | IATA | dhakala@iata.org |
| ICCAIA |  | Joseph Cramer | Regional Director The Boeing Company Tel: +1-703-465-3486 | Joseph.Cramer@Boeing.com |
|  |  | Claude Pichavant | ICCAIA Advisor Senior Expert – COM SUR Tel: +33622452389 | claude.pichavant@airbus.com |
|  |  | Dr. David Redman | Director, AVSI USA Tel: +1-979-218-2272 | dredman@tamu.edu |
|  |  | Sai Kalyanaraman | Ph.D. Technical Fellow, R&T, Avionics COLLINS AEROSPACE 855 35th Street NE, Cedar Rapids, Iowa, 52498 U.S.A Tel: +1 319-263-8152 | Sai.Kalyanaraman@collins.com |
|  |  | Uwe Schwark | AIRBUS Operations GmbH Kreetslag 10 21129 Hamburg Germany Tel: +49 (0) 40 743-72908 Mobile: +49 (0) 151 14081929 | uwe.schwark@airbus.com |
|  |  | Thomas Meyerhoff | Airbus | Thomas.meyerhoff@airbus.com |
|  |  | Marcella Ost | Marcella Ost Dir., Spectrum Policy & Regulatory Affairs The Boeing Company 45 O’Connor Street, Suite 1220 Ottawa, K1P1A4 Tel: +1 613-790-2270 | marcella.s.ost@boeing.com |
|  |  | Steven H Rines | Aircraft Systems Architect Safran Aerospace Tel: +1 714 592-9359 | steven.rines@safrangroup.com |
| ITU |  | Nikolai Vassiliev | International Telecommunication Union (ITU) Place des Nations CH-1211 Geneva Switzerland | nikolai.vassiliev@itu.int |
|  |  | Nelson Malaguti | ITU International Telecommunication Union (ITU) Place des Nations CH-1211 Geneva Switzerland Tel: +41 022 73051 98 | nelson.malaguti@itu.int |
|  |  | Xingguo Zhou | International Telecommunication Union (ITU) Place des Nations CH-1211 Geneva Switzerland Tel: +41 022 73050 68 | xingguo.zhou@itu.int |
|  |  | Karlis Bogens | International Telecommunication Union (ITU) Place des Nations CH-1211 Geneva Switzerland | karlis.bogens@itu.int |
| ICAO |  | Loftur E. Jónasson | International Civil Aviation Organization (ICAO) Tel: +1-514-954-8219 Ext. 7130 | ljonasson@icao.int |
|  |  | Mie Utsunomiya | International Civil Aviation Organization (ICAO) Tel: +1-514-954-8219 Ext. 6082 | mutsunomiya@icao.int |

-END-

APPENDIX D

ACTION ITEM LIST

| Number | Description | Actionee | Due Date | Status |
| --- | --- | --- | --- | --- |
| 03-03 | Provide comment on the spectrum sharing approach between terrestrial and satellite RPAS C2 systems for the 5 030-5 091 MHz as proposed in FSMP-WG/3 WP10 and FSMP-WG/4 WP17 | All | FSMP-WG/14 | Ongoing FSMP-WG/12 IP01 and WP11 provide status information |
| 04-05 | Provide input to complete the equipment physical characteristics (e.g., weight) table shown in the Annex of FSMP-WG4/WP26. | All | FSMP-WG/14 |  |
| 04-06 | With regard to the action to “Conduct an aircraft fleet equipage impact analysis and develop detailed transition plans based on industry input and expected safety benefit” in the radio altimeter job card, initiate outreach to the airlines and aircraft manufacturers. | IATA and ICCAIA | FSMP-WG14 | Ongoing |
| 05-06 | Explore restructuring of Doc 9718 Volume 1. | Secretary/A. Roy/correspondence group | 2022 (complete) | FSMP-WG/12 WP05 provides updates |
| 09-05 | Provide material for the relevant structures specified in WG/09 WP17 and WP20 to the spectrum handbook correspondence group before the next FSMP meeting. | All | FSMP-WG/14 |  |
| 09-06 | Review system characteristics structure in WG/09 WP20, and provide necessary system information to complete the section for each system. | K. Masrani (Inmarsat), D. Ladson (Iridium), U. Schwark (WAIC), A. Roy (HF and VHF), and Secretary (for other panels to review) | FSMP-WG/14 | Partially addressed by FSMP-WG/10 WP14, FMSP-WG/11 WP25. |
| 09-09 | Provide updated/expanded example outlining the approach for aviation system protection suggested in FSMP-WG/2 WP24. | J. Mettrop | FSMP-WG/14 |  |
| 09-10 | Provide comparison of aviation system emission masks with respect to relevant ITU requirements. | J. Mettrop | FSMP-WG/14 | FSMP-WG/12 WP18 and WP19 & FSMP-WG/13 WP10 and WP12 provide information. |
| 10-01 | Catalogue aviation concerns with the current language in Resolution 155 (Rev. WRC-19). Participants to send their email address to John Mettrop (john.mettrop@caa.co.uk) by 28 August, 2020 if they want to be part of the FSMP correspondence group. | All | FSMP-WG/14 | Termed the group CG-155. FSMP-WG/12 WP13 progressed questions to help delineate ITU tasks from tasks for ICAO/Administration |
| 10-02 | Provide comments on increased role for ICAO proposed in the longer-term HF frequency coordination approach contained in FSMP-WG/10 WP17. | All | FSMP-WG/14 |  |
| 10-03 | Report on current use in States of the 1300-1350 MHz frequency band for radars and longer term plans. | All | FSMP-WG/14 |  |
| 10-06 | Provide contributions describing how ultrawideband systems are being authorized in your State. | All | FSMP-WG/13 | FSMP-WG/11 IP01 |
| 10-07 | Monitor ITU-R WP5A deliberations on radiocommunications systems for train and trackside (RSTT) as frequency bands being considered are adjacent to those used by aeronautical safety systems | All | Ongoing |  |
| 10-08 | Develop contributions on how to ensure protection of aeronautical systems that are not included in the ITU MIFR. | All | FSMP-WG/14 |  |
| 11-04 | Participate in the radio altimeter correspondence group (CG-RA) | All | Ongoing | Send Email to christian.fleury@aviation-civile.gouv.fr |
| 11-05 | Participate in ITU-R WP 3K-3M-9 correspondence group which is modelling air-ground propagation paths | All | Ongoing |  |
| 11-06 | Consider RPAS use of more than just the “2x10 MHz” for AMS(R)S communications in the 1.5/1.6 GHz L-Band | All | FSMP-WG/13 |  |
| 11-08 | Review Recommedation ITU-R SM.1535 and determine if updates are required | All | FSMP-WG/14 | Noted that any identified changes should be input to ITU-R by State(s) and not ICAO. |
| 12-01 | Provide agreed draft SARPS 2 weeks prior to FSMP-WG/13 to allow review and action by that meeting | CG-WAIC | 2 weeks before FSMP-WG/13 | FSMP-WG/13 WP14 and IP05 apply CLOSED |
| 12-02 | Provide comments and updates to the national power levels contained in Figure 1 of FSMP-WG/12 WP17 to Sai.Kalyanaraman@collins.com | All | ASAP |  |
| 12-03 | Provide contributions to FSMP-WG/13 regarding the proposals on SM.1541 contained in FSMP-WG/12 WP18 | All | ASAP to author or to FSMP-WG/14 | FSMP-WG/13 WP09 applies |
| 12-04 | Provide contributions to FSMP-WG/13 regarding the proposals on adding SARPS spectral characteristics requirements contained in FSMP-WG/12 WP19 | All | ASAP to author or to FSMP-WG/13 |  |
| 12-05 | Consider the options for Annex 10, Vol III, 4.3.1.1 contained in Appendix I of the FSMP-WG/12 meeting report and come to a decision at FSMP-WG/13 | All | FSMP-WG/13 | CLOSED |
| 12-06 | Provide contributions to WG/13 to further develop and answer outstanding questions on the aviation RF sharing framework in Flimsy08 | All | FSMP-WG/13 |  |
| 12-07 | Provide contributions to WG/13 to provide material on a methodology to develop aviation system protection criteria and spectral masks. | All, Lead: J. Mettrop | FSMP-WG/14 |  |
| 12-08 | Review and provide comments on the “ITU Res 155 tasks” material contained in Appendix G to the report of FSMP-WG/12. | All | FSMP-WG/14 |  |
| 12-09 | Review the text of the Job Card FSMP-006-001 and determine if changes are needed.. | All | FSMP-WG/14 |  |
| 12-10 | Determine if generic guidance can be developed to help aviation authorities analyse potential 5G impacts on radio altimeters given their specific State implementation |  | FSMP-WG/14 |  |
| 13-01 | Provide examples of national specifications or regulations on adjacent-band signal rejection. | FSMP participants who are also spectrum regulators in their country | FSMP-WG/14 |  |
| 13-02 | Develop repository on the FSMP website for storage of reference material on 5G/Radio altimeter issue | Secretary | ASAP or by FSMP-WG/14 |  |
| 13-03 | Develop educational material for States explaining the 5G/RA issue. | CG-RA | ASAP, but not later than FSMP-WG/14 |  |
| 13-04 | Develop a set of frequently asked questions (FAQs) regarding satellite-VHF. | CG-SV | FSMP-WG/15 | To be part of the CG-SV send an Email to the coordinators at: mangarcia@enaire.es and olivier.pellay@airbus.com |
| 13-05 | Catalog existing aviation system transmit and receive masks | J. Mettrop | FSMP-WG/14 |  |
| 13-06 | Provide any comments on the draft WAIC SARPs (see Appendix E) to CG-WAIC in sufficient time for them to be considered prior to FSMP-WG/14 | All | Prior to FSMP-WG/14 |  |

APPENDIX E