|  | International Civil Aviation Organization REPORT | 13 September, 2018. |
| --- | --- | --- |

FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

SEVENTH MEETING OF THE WORKING GROUP OF FSMP

(FSMP-WG/7)

Johannesburg, South Africa

6-13 September, 2018

DRAFT REPORT

1.	Introduction

1.1	The meeting was opened by Mr Loftur Jonasson from the ICAO Secretariat, Montreal and Mr Mike Biggs, the Rapporteur of Working Group FSMP (FSMP-WG).  Mr Jonasson acted as the Secretary of the meeting assisted by Mr Francois-Xavier Salambanga of the ICAO/Dakar Office and Mr Harvey Lekamisy of the ICAO/Nairobi Office. Following introductions, Mr Biggs thanked South Africa Air Traffic Navigation Services (ATNS), and in particular Mr. Thabani Mthiyane and Ms. Lisa Tele for hosting the meeting. He also welcomed the group and provided introductory remarks, meeting information and housekeeping details. Mr Biggs also noted with sorrow that it was a shame that long-time South African CAA colleague Mr Koos Pretorius had passed away before being able to host an ICAO meeting in his beautiful home country, which had been a long-term goal of his. The meeting noted its appreciation for his extensive contributions.

1.2	The meeting was held in English. After the opening of the meeting the agenda was approved by the group. The agenda is contained in Appendix A.

1.3	The list of papers submitted for consideration by FSMP-WG/7 is contained in Appendix B. The list of participants is in Appendix C.

1.4	The material in this report is organized by meeting agenda item number, and does not necessarily reflect the order of discussions.  The meeting conducted a review of the actions from the last meeting. Actions captured during discussions are shown in Appendix D, together with status of prior-meeting(s) actions.

2.	Agenda Item 2 – ICAO WRC-19 Position

2.1	WP05 presented the draft Conference Preparatory Meeting (CPM) text for the 2019 World Radiocommunication Conference (WRC-19) Agenda Item 1.10 on the Global Aeronautical Distress and Safety System (GADSS) as developed by International Telecommunication Union – Radiocommunication Sector (ITU-R) Working Party 5B (WP5B) at its meeting in May/June 2015. That text summarized the results of studies on GADSS and outlined two methods to satisfy the agenda item. The paper further suggested that ICAO should support minimal recognition of GADSS in the Radio Regulations (RR), and in particular oppose language that will result in requiring RR changes as GADSS evolves (e.g. oppose identification of specific functions of GADSS; specific elements of GADSS; or specific operational frequency bands of GADSS).

2.2	WP09 also considered the draft CPM text developed for GADSS. It noted that internally the national regulator proposed developing a single method combining the two existing proposed CPM methods. This would only reference the relevant ITU-R Recommendations for the safety systems used during the automated distress tracking function. There was no general meeting support for this approach.

2.3	WP06 presented the draft CPM text for Agenda Item 9.1, Issue 9.1.4 (dealing with suborbital vehicles) as developed by ITU-R WP5B at its meeting in May/June 2015. That text summarized the results of ITU-R studies and suggested a way forward. In particular no change to the Radio Regulations is proposed for WRC-19; rather further operational, technical and regulatory issues require continuing studies, in particular of the status of the station aboard suborbital vehicles and type of applications. A future Conference agenda item to address study outputs may be needed.

2.4	WP04 and WP18 provided proposed updates to the ICAO Position on issues of critical concern to aviation which are on the agenda of WRC-19, based on the results of studies in the ITU-R.

2.5	The meeting considered each of the inputs and the agreed initial updates to the ICAO position are contained in Appendix E. One member expressed concern that the ICAO position on agenda item 1.7 includes opposition of the use of existing allocations unless agreed ITU-R studies have demonstrated compatibility with aeronautical systems.

2.6	WP26 noted that while space-based ADS-B provides a means of surveillance over oceanic and remote continental areas, there are no similar ready solutions for the case of VHF voice communication to support radar-like separation minima in these areas.  The paper suggested one potential solution using VHF radio relays installed on satellites or high altitude platforms.  The meeting highlighted the need for ITU allocation of VHF frequency band if satellite solutions were to be selected. The meeting expressed general support for the concept, but raised a number of concerns that need to be addressed. Those included: (1) ensuring the approach would entail no change to existing aircraft VHF avionics/installations; (2) more detail was required on potential interference to terrestrial VHF usage; and (3) while any necessary ITU allocations could be on a global basis, the idea of a global set of VHF channel assignments would not be supportable due to severe congestion in areas of the world; however regional allotments might be possible. In addition, it was stressed that the concept should be considered as part of an overall aviation future strategy and as part of that other options (e.g., fielding of VHF repeaters on platforms such as oil rigs or stratospheric vehicles) should be examined. While the paper was submitted under agenda item 2, the meeting agreed that at this point there was no related change required to the ICAO Position. It was however added to the running FSMP “laundry list” of possible future agenda items (see paragraph 9.12) and a short explanation was developed and is contained in Appendix G.

3.	Agenda Item 3 – Radio Altimeter and WAIC issues

3.1	IP15 reported the intermediate results of Radio Altimeter (RA) interference susceptibility testing and status of continued testing. Testing of RA susceptibility to a single representative in-band Wireless Avionics Intra-Communication (WAIC) signal interference source has been completed for all altimeters and altitudes considered in this campaign. Results indicate sufficient margin for safe operation of RAs in the presence of WAIC signals. Additionally, the paper provides an overview and preliminary results from continued testing to characterize altimeters in an aggregate interference environment. Based on discussion within the meeting it was agreed to add a test case where RA performance is considered in concert with both an in-band WAIC environment and representative out of band interference signals.

3.2	WP27 contained an update of the draft Standards and Recommended Practices (SARPs) for WAIC systems currently under preparation by the FSMP WG. The draft SARPs text contained in the Annex is the result of the feedback received regarding Working Paper 18rev1 presented at the sixth meeting of FSMP WG, together with input from ongoing standardization efforts within the joint RTCA / EUROCAE committee SC-236 / WG-96. This group is currently developing a Minimum Aviation System Performance Standard (MASPS) and a Minimum Operational Performance Standard (MOPS) for WAIC systems. The meeting provided some suggestions on the wording, in particular with regard to interference from out of band sources. The meeting also suggested that it might not be appropriate to reference outside documents (e.g., RTCA MASPS) directly in a SARPs provision. For other systems (e.g., AeroMACS) the reference is given in a Note.

3.3	IP11 was considered together with a draft of the RTCA WAIC MASPS that was provided to interested FSMP membership for their review in their capacity as independent aviation experts. The proposed approach was extensively discussed by the meeting, and reviewers were asked to provide their comments directly to RTCA using the comment matrix provided.

3.4 	IP06 provided the results of a study undertaken to evaluate adjacent frequency band compatibility between radio altimeters and 5G mobile communication systems using the 3.7 GHz and 4.5 GHz band.  The study took into account the positional relation between a radio altimeter (RA) and base stations (BSs) or land mobile stations (MSs) of 5G systems in the vicinity of an airport or heliport, and used the ITU-R characteristics for radio altimeters as its basis. The meeting greatly appreciated the material, and asked a number of questions about the approach and conclusions. The primary conclusion was that under the parameters of the study, compatibility could be assured only with the institution of geographic separation, a frequency guardband, and additional filtering on the 5G equipment.

3.5	IP08 presented the results of an investigation into the impacts of an adjacent-band LTE-Advanced base station on four typical aircraft radio altimeters using the methodology of RTCA DO-155. The paper focused on helicopter take-off and landing at a heliport, and in particular on the interference which causes receiver front-end overload, i.e., blocking. Considering LTE-Advanced base station deployment in an urban area, aggregate interference signal strength produced by multiple base station was calculated, and such interference is considered in the experiment. When the guard band is less than 10 MHz, a pulsed radio altimeter system is found to be affected by adjacent-band interference, resulting in a maximum altitude error of 4.3% at an altitude of 50 ft. Meanwhile, three frequency modulated continuous wave (FMCW) radio altimeter systems examined in this study were found to be unaffected by the considered adjacent-band interference. The results show that the pulsed radio altimeter system is more sensitive to the interference and a 60-MHz guard band is required to maintain an altitude error of < 0.5%. The meeting appreciated the information.

3.6	IP16 provided information on testing of radio altimeter susceptibility to emissions outside, but in proximity to, the 4 200 – 4 400 MHz frequency band. A description of the experimental setup was presented along with examples of data obtained from preliminary testing. The paper concludes with a status of this testing.  The paper did not recommend conclusions concerning the general susceptibility of radio altimeters to out of band interference as the data acquired to date is insufficient to support such conclusions. The meeting appreciated the information and encouraged continued dialog between the entities performing altimeter testing to ensure consistency in approach.

3.7	Given the different results from IP’s 6, 8 and 16, it was noted that all testing should be completed before any definitive statements on radio altimeter compatibility with potential adjacent band IMT/5G are made.

3.8	Flimsy5 was presented as an outgrowth of previous discussions on SARPS text for WAIC tolerance of interference from sources operating outside of the 4200-4400 MHz band. After discussion the meeting agreed on proposed phrasing as contained in Appendix K. In addition, the meeting agreed to form a correspondence group to assist in completing WAIC SARPS by FSMP-WG/8 (see Action Item 07-04). The meeting was encouraged to Email Mr. Radek Zakrzewski  (radek.zakrzewski@utas.utc.com) in order to become part of the correspondence group.

4.	Agenda Item 4 – Development of (planned) material for ITU-R studies on:

4.1	FSS for UAS

4.1.1	IP04 provided information regarding the propagation characteristics of Ka-band links intended to provide communications between satellites and unmanned aircraft systems (UAS). The meeting expressed appreciation for the material noting that it would help in studies to assess rain fade which had been raised as a potential issue for Ku/Ka band use in Africa. The meeting also asked to be kept informed as studies progress.

4.1.2	IP07 provided, for information, three letters containing correspondence between the Directors of the ITU Radiocommunication Bureau (BR) and the ICAO Air Navigation Bureau on progress of SARPS development regarding UAS/RPAS use of FSS links.  The meeting noted with appreciation that the Director BR had indicated he would address the difficulty of progress within the ITU-R which had resulted in ICAO progressing the work in parallel.

4.2	GADSS

4.2.1	WP20 set out a proposal on the introduction of a return space-to-Earth link in the 1559-1610 MHz frequency band in the GADSS concept being developed by ICAO and on the inclusion of provisions for the use of such a link in the ITU Radio Regulations.  It was clarified that this proposal was for a potential agenda item for WRC-23. The meeting discussed the topic extensively and noted that States had rejected including a requirement for remote trigger of the automatic distress tracking function in GADSS-related SARPS. In addition, regulatory protection of the postulated signal would require addition of a mobile-satellite service allocation to the RNSS band(s), which was seen as risky to RNSS. As such the meeting agreed that aviation should not be active proponent of the actions proposed in the working paper.

4.3	Status of proposed update to Recommendation ITU-R SM.1009

  - 4.3.1	WP07 noted that in ITU-R Working Party 1A a number of options had been explored regarding possible updates to Recommendation ITU-R SM.1009 “Compatibility between the sound-broadcasting service in the band of about 87-108 MHz and the aeronautical services in the band 108-137 MHz” including: developing a stand-alone ITU-R Report on alternate approaches used in certain states, developing a new ITU-R Recommendation on digital sound broadcasting, making the new material an annex to the existing SM.1009, or even beginning a complete new revision of SM.1009.  At its June 2018 meeting however, based on an input from the proponent, WP1A agreed to cease work on possible modification. The meeting supported the decision.

4.4	Aeronautical Protection Criteria

4.4.1	WP01 was a liaison statement from WP5B noting that, in response to Question ITU-R 235/5, they have begun development of material on protection criteria for aeronautical and maritime systems, including a working document toward a preliminary draft new report on protection criteria for aeronautical and maritime systems found in the previous study period (2012-2015) Doc. 5B/475 Annex 26. WP5B further noted that that draft material was quite old and that WP5B was considering whether the effort should be continued. As a result ICAO was asked for any information that would assist WP 5B in developing such a new report, new recommendation, or alternatively a response that will inform WP 5B that development should be halted (see 4.4.2 below).

4.4.2	WP08, WP13, WP14 and WP18 all noted that recently ITU-R has been considering the introduction of short durations satellite systems in frequency bands above 137 MHz, for example as part of studies for Agenda Item 1.7 (AI1.7) for WRC-19. The frequency band 117.975 – 137 MHz is allocated worldwide to the aeronautical mobile (route) service (AM(R)S) and is the main aviation communications band for air-ground, air-to-air, and ground-air communications.  It is used at all airports and for all phases of flight. As a result the papers suggested protection criteria for the AM(R)S systems and/or studied the potential interference from non-geostationary orbit short duration (NGSO-SD) satellite communications links, using the proposed ITU-R characteristics for such links. The meeting considered all four papers together. The agreed elements for an ICAO response to WP5B (see para 4.4.1) are contained in Appendix F. The meeting also suggested the response be copied for information to WP7B.

5.	Agenda Item 5 – 5 GHz Band Planning

5.1	AeroMACS Status

5.1.1	No papers were presented on this topic.

5.2	Global UAS/RPAS channel plan

5.2.1	WP10 presented an analysis regarding the number of Unmanned Aircraft Systems (UAS) over an arbitrary 3000x2000 km territory that could receive C2 link frequency channel allocations using the terrestrial communication system, assumed to be given 10 MHz within the 5030-5091 MHz band. The performance of two types of channel allocation methodologies are compared (Static or Dynamic), depending on the type communication architecture considered (standalone UAS or centralized network), and conclusions and recommendations are drawn. The assumptions concerning the C2 link signal and C2 radio systems were based on the conclusions of Phase 1 of the Command and Control Communications Subproject of NASA’s UAS Integration in the National Airspace System Project.  The meeting appreciated the in-depth and extensive information and provided suggestions for the follow-on efforts.

5.2.2	WP16 noted that as part of its work to implement AeroMACS system, Japan is starting studies of potential interactions with CNPC links for RPAS in the 5030-5091 MHz band. In particular the paper notes that the allocation to the communication services from the satellite to the earth should be carefully discussed to establish the mitigation criteria between the service and other terrestrial and aeronautical services. As a result, studies versus AeroMACS in the band 5091-5150 MHz are planned. After discussion, one member noted that they had also performed some CNPC/AeroMACS studies and would be presenting the results to FSMP-WG/8. That member agreed to provide Japan with characteristics of the CNPC links to facilitate the work envisioned in WP16.

6.	Agenda Item 6 – New provisions to support aeronautical radiocommunications

6.1	L-band Digital Aeronautical Communication System (LDACS)

6.1.1	IP10 included an attached paper that was developed by the Communication Panel - Infrastructure Working Group – Project Team “Terrestrial” (CP/IWG/PT-T).  The paper provided an overview on the current technical characteristics of LDACS and the selected frequency bands:

- Forward Link (Ground-to-Air): 1110–1125 MHz

- Reverse Link (Air-to-Ground): 964–979 MHz

It also provided information on testing envisaged to demonstrate the compatibility with the navigation systems operated in that band, i.e. DME and TACAN. The paper still needs to be finalized, and it was noted that comments could be provided via E-mail to:  Felix.Butsch@DFS.de. (See action item 07-01). Once the paper is mature, it will be submitted to the meeting of ICAO NSP in November 2018 for final adoption. The meeting discussed the paper extensively and raised a number of questions and concerns, including how to ensure that protection criteria for aviation system interference to other aviation systems were not misused to address non-aviation interference. The author asked for comments to be provided in writing, including rationale for the comment. A question was also asked about a statement in the PT-T minutes where the possibility was raised of relaxing LDACS emission requirements and addressing through different channel assignment criteria. The meeting expressed the opinion that new aviation systems should be designed to be as spectrally efficient and resistant to interference as possible.

6.2	RPAS Panel Discussion of Direct Ground-Ground VHF Communications

6.2.1	No papers were presented on this agenda item, however it was reported that the RPASP Secretary had stated that there are no active discussions on the issue and it would be flagged to the FMSP if it was raised again. AI 05-03 was closed.

7.	Agenda Item 7:  Interference from non-aeronautical sources

7.1	Programme Making and Special Events (PMSE)

7.1.1	IP12 gave an update on the current situation with respect to the implementation of wireless microphones in the 960-1164 MHz frequency band within the UK. The meeting appreciated the information but noted that a number of the concerns raised by previous FSMP WG meetings had not been addressed. The concern was expressed by a number of participants that in order to ensure aviation concerns were adequately addressed, more support from the European aeronautical spectrum management community was required to assist the on-going work within the CEPT framework. Toward this end it was noted that a video was available (see  https://youtu.be/Gq-E-xBRuiA ) to help aviation experts explain those concerns to their radio regulators.

7.2	LTE introduction adjacent to 1518-1559 MHz SATCOM band

7.2.1	WP02 provided a liaison from ITU-R WP4C to the International Maritime Organization (IMO) that was also copied to ICAO.  That liaison noted that Resolution 223 (Rev.WRC-15) invites ITU-R to conduct compatibility studies between IMT in 1 492-1 518 MHz and MSS in 1 518-1 525 MHz and also invites ITU-R to develop frequency arrangements for IMT in 1 427-1 518 MHz taking into account the results of those studies. As a result, WP 4C and WP5D are working on studies related to IMT/MSS compatibility and developing a new ITU-R Recommendation and a new ITU-R Report to address compatibility measures. Once the results of the studies become available, WP 4C will share the results with IMO/IMSO. The meeting noted the information.

7.2.2	WP24 was a liaison from CEPT ECC Project Team 1 (PT1) thanking ICAO for their letters addressing potential blocking of current mobile earth stations in adjacent bands to 1518 MHz (including 1525-1559 MHz) in seaports and airports. It noted that the next ECC PT1 meeting would be in July 2018 and that they will keep ICAO informed on the progress of this work.  The meeting noted the information and in response to a question the Secretary stated that no further information from the July 2018 PT1 meeting had been received by ICAO.

7.2.3	PRES52 from the WRC-19 Workshop was briefly presented to highlight the issues being addressed in the ECC PT1 forum. Then WP29 was presented to explain the current status on each of those issues in PT1. In particular it was noted that a report was expected to be finalized at the PT1 meeting starting September 17, 2018. The majority of that report was drafted by an “Administration Group” comprised solely of administration representative. Input on equipment availability and timelines was developed by a “Stakeholders Group” made up of the wider PT1 membership. It is envisioned that power flux density (PFD) limits will be placed on the LTE equipment in two phases, where phase 1 limits are more restrictive than phase 2. What those limits will be, and the time period between phase 1 and phase 2 are not yet defined. The meeting highlighted that it was imperative that phase 1 limits protect existing aviation equipment and the transition to phase 2 be done on a timeline that is supportable by aviation equipage schedules. Questions were also raised about who would pay for the necessary equipment changes.

7.2.4	IP13 provided the meeting, for information, with a copy of the IMO response to the WP4C liaison discussed in WP02 above (see paragraph 7.2.1).  The meeting noted the information.

7.3	Other

7.3.1	IP02 provided results of emission characterization tests performed on two Light Emitting Diode (LED) bulbs which malfunctioned and led to recent VHF airport interference cases. The meeting noted the information with interest and reminded participant to preserve any LED bulbs/fixtures that should cause interference to aviation systems in the future. Study of those failed equipment may facilitate design of fail-safe LED equipment. During the discussion it was noted that an LED manufacturer had provided a presentation on LED equipment to the Navigation Systems Panel (NSP). That presentation was made available to participants as FSMP-WG/7 IP14. Of particular note from that material was that (1) installation of LED fixtures needed to be closely controlled at simply crossing power cables during installation could greatly influence interference characteristics, and (2) some LED devices use pulsing to provide a data delivery service. For this data delivery purpose, future versions of that service were stated to be pulsed at a rate of 108 MHz, a parameter flagged by NSP participants as being a very bad idea. Participants were asked to provide further information if possible. The meeting was also notified that the maritime community had reported similar LED interference to their systems. Finally participants were asked to consider if there were better ways to ensure the LED concerns were distributed to airports and ANSPs, not to stop/slow the installation of LED lighting, rather to inform the aviation community to consider LED as a source if cases of VHF interference are encountered.

7.3.2	IP09 provided a comparison of standards between VDLM2 and Plain old ACARS (POA) to explain why LEDs interference had been experienced by the former and not the latter. In summary, while both systems rely on carrier sensing to determine if a channel is in use, POA does additional check to ensure any measured signal is an actual POA signal. As a result, while wideband noise (such as from LED interference) could cause VDLM2 to consider a channel “in use”, POA would ignore the noise and continue to use the channel. The meeting appreciated the information.

8.	Agenda Item 8: Revision of the ICAO Frequency Spectrum Handbook (Doc 9718)

8.1	A number of papers were received regarding the on-going revision of Doc 9718. In particular: Overview (WP21); Volume A – Spectrum Policy, Vision and Strategy (WP32, WP23); Volume B – Spectrum Management for ICAO (WP28); Volume C – ICAO Band Position and Aviation System Parameters (WP22, WP31); and Volume D – Frequency Management of ICAO Systems (WP15).

8.2	WP21 provided the FSMP membership an overview of the current activities of the correspondence group created to revise the ICAO Spectrum Handbook.  Particular note was made of the need to raise issues with the proposed restructuring as soon as possible so they can be addressed before the draft is finalized.

8.3	WP32 proposed a structure for the new Spectrum Handbook Volume A and asked for guidance from the FSMP meeting. The meeting discussed the proposals and provided some suggestions for the author.

8.4	WP23 explored how the ICAO handbook may encourage better use of spectrum by new aviation systems being developed.  The meeting was supportive of the concept in general, however it was indicated that caution needed to be taken regarding the specific wording. The concept will be further developed via correspondence and at FSMP-WG/8.

8.5	WP28 proposed a structure for Volume B (Spectrum) of the restructured handbook and solicited comments on that structure and content. The meeting agreed with the overall structure, and provided some comments to the author.

8.6	WP22 proposed a structure for the new Spectrum Handbook Volume C, focusing on elements that need to be resolved to update the existing material. The meeting liked the overall structure and agreed it would be a useful reference when complete. Concerns were raised about including “protection criteria”, with a need to balance the text to avoid possible incorrect use of any listed parameters/limits.

8.7	WP31 provided an update on the aviation use of VSAT networks in the AFI region. The Aviation usage of VSAT has increased in the AFI region during the ten year period since 2008. An upgrade has been done recently in 2017 to improve the networks and enhance them for future developments. In particular, a section for doc 9718 on the VSAT was proposed. The meeting agreed that the material would be folded into the reformatted Volume C.

8.8	WP15 suggested that minimum requirements/technical parameters and the frequency channelling plan(s) of surveillance radar systems be included in the proposed Volume-D-Frequency Management of Doc 9718 to support aviation stakeholders for getting regulatory clearances from national spectrum regulators. The particular difficulty was regarding channelling plans for 2.7-2.9 GHz radars, which while aeronautical systems, are not ICAO-standard systems. While channel plans for ICAO standard systems are contained in SARPS (e.g., Annex 10 Volume V), non ICAO-standard systems are left to national approaches. Given this situation, the meeting agreed it would be appropriate to include such information in the Handbook for non-ICAO aeronautical systems.

8.9	Comments and discussion on all of the papers will be taken on-board by the Doc 9718 restructuring correspondence group in its work, and updates will be presented to FSMP-WG/8. Particular note was made of the goal of making the handbook, and in particular Volume A, clear and complete, but concise (short). In fact, one suggestion was to make Volume A (Overview) very short to make it more useful as a tool for educating senior managers from spectrum and other disciplines. The other volumes could then provide the detail necessary for spectrum management experts. While the current focus is on structure, in the near future there will be work on specific content areas. The meeting was reminded of action item 05-06, and active participation in the correspondence group was again encouraged.

9.	Agenda Item 9:  Any other business

9.1	WP03 provided two contributions from the Radio Spectrum Administration of Greece to a recent meeting of the European Conference of Postal and Telecommunications Administrations (CEPT) Electronic Communications Committee (ECC) Working Group on Frequency Management (WGFM), sub-group FM44.  FM44 deals with Satellite Communications.  Those two papers were liaised informally to the ICAO Secretariat by the chairman of FM44, with the request to help define a worst case area that could cover any typical European aerodromes to help with licensing of satellite earth stations. The Secretary noted that the request had been provided to the Aerodrome experts in ICAO resulting in an exchange of information. The current status of the discussions is shown in Attachment H, and FMSP members are invited to provide any comments to the FSMP Secretary.

9.2	IP03 contained the outcome of an avionics equipment weight survey performed in Japan for generic short haul (B737-800) and long haul (B777-200/300) aircraft in response to action item 04-05. The material may be used to start populating the table contained in the attachment to FSMP-WG/4 WP/26. The meeting appreciated the information and encouraged others to provide information as it becomes available.

9.3	IP05 provided information on the results of an air-to-air communication experiment regarding location information sharing and traffic management for low-altitude small unmanned aircraft and manned aircraft in uncontrolled airspace of Japan.  It was noted that the required data amount was approximately 40 bytes per transmission per tracked aircraft. The meeting appreciated the presentation and asked to be kept informed as studies progress.

9.4	WP11 proposed to modify the provisions of ICAO Annex 10, Vol. V,  chapter 4 to change the upper limit for the use of 8.33 kHz voice MHz from 136.500 MHz to 136.875. It also suggested inserting a note stating that the regulation for the exact use of channels within the band 136.500 – 136.875 MHz be determined by means of regional air navigation agreements.  The meeting was not supportive of the proposal at this time, rather it was suggested that more information was required regarding the rationale for the current SARPS provisions before they could be considered for change. The Secretary was asked to liaise the proposal with the Secretary of the Communications Panel to also get their input (Action Item 07-02).

9.5	WP12 provided draft Standards and Recommended Practices (SARPs) that are intended for potential inclusion in Annex 10, all Volumes except for Volume II. Those new draft high-level provisions are intended to facilitate the evaluation of a States’ capacity to monitor the effective implementation of the relevant Annex 10 provisions, under the Universal Safety Oversight Programme (USOAP). The meeting supported the general idea and looked forward to the updated version at the next meeting. It was suggested that reference material from existing ICAO documents be included, especially regarding proposed language that appeared directed at the State radio regulators.

9.6	WP17 provides information on the UK’s plan to deliver its preferred option for improving electronic conspicuity nationally through the implementation of ADS-B ‘in/out’ and requested information on any testing or simulation work that has been undertaken in other States to support the implementation of ADS-B. The meeting noted that there had been previous studies on 1090 MHz capacity both as part of the Technical Link Assessment Team (TLAT) in the early 2000s, and continuing work within the ICAO Surveillance Panel (SP). It was suggested that the author reach out to the SP to get information on input parameters for modeling, and to ensure there was no duplication of efforts.

9.7 	WP19 reported that the dual frequency multi-constellation (DFMC) satellite-based augmentation system (SBAS) SARPs Sub-group (DS2) of the navigation systems panel (NSP) is developing SARPs for DFMC SBAS. Those SARPs will include interference masks to be developed for SBAS L5 signal.  The paper also included first contributions towards establishing that mask, based on the interference mask and methodology included in RTCA DO-292. It was noted that Revision A of RTCA DO-292 containing a new L5/E5a interference mask is to be released by RTCA SC-159 WG6 in March 2019 taking into account the new radio-frequency environment under which GNSS system transmitting L5/E5a signals are operating. The work on the interference mask carried out by DS2 is tightly linked to the work to be done by RTCA SC-159 WG6 and coordination of work is ongoing. As a result, DS2 was looking for feedback on the described methodology.  The meeting asked a number of questions and agreed to provide any comments through the NSP Spectrum Working Group (SWG) chairman Mr Felix Butsch (Felix.Butsch@DFS.de)

9.8	WP25 highlighted the need for aviation as a whole to be aware of the potential dangers that can result from publishing information about a topic without considering the wider implications and seeks support to raise the awareness of our colleagues to the potential dangers of not considering the implications of how documents are worded.  The meeting discussed the topic and agreed that vigilance was required. It was also suggested that the paper be distributed to the Navigation Systems Panel and Communications Panel to raise their awareness.

9.9	IP01 was the report of the sixth meeting of the ICAO NSP SWG, which was held at ICAO Headquarters, Montreal from 17th to 20th April 2018. The most relevant topics for the ICAO FSMP-WG were the preparation of frequency coordination criteria for ILS and VOR in the context of moving these criteria from Annex 10, Vol. I into “Handbook on radio frequency spectrum requirements” (Doc. 9718), and that a new non-aviation detection and collision avoidance application had been introduced in the frequency band 442.2-450 kHz (see ECC Report 284). Regarding the former, the latest drafts are provided in Appendix I, and the meeting was requested to provide comments to the SWG Rapporteur at Felix.Butsch@DFS.de by the end of October, 2018. Regarding the latter it was noted that while that system had not proven problematic to aviation, another proposal may be an issue (see Appendix J).

9.10	The meeting agreed that it was nearly impossible with the available resources to keep track of all the new proposals that could impact aviation. Participants were asked to remain vigilant, and if they see something that may be a problem to share it with the group.

9.11	WP30 noted that there is an interest in operating Upper Altitude Platforms at altitudes as low as 18km in quasi stationary positions, overlapping with the Upper Traffic Area (FL195 to FL660), where air traffic control services are provided fully to IFR flights, and partially to other VFR flights. The Secretary was given an action (see Action Item 07-05) to liaise with the appropriate body in ICAO and provide information to FSMP-WG/8 as to whether this planned approach would require special coordination.

9.12	As an outgrowth of other discussions and FSMP-WG/6, the meeting was aware of the following items being suggested as possible aviation-related proposals for WRC-23 agenda items:

- To assess whether non-GADSS-related regulatory changes to the Radio Regulation are necessary to facilitate introduction of Remotely Piloted Aircraft Systems (RPAS) and/or as general clean-up.

- Support for a space planes agenda item (WRC-19 agenda item 9.1, issue 4).

- Response to notional idea to have satellite reception/transmission of VHF to be used by aviation (see Appendix G).

- An allocation to support airport foreign object detection and/or wildlife detection radars

No FSMP endorsement for the items in this list is implied. All participants are invited to provide additional input to FSMP-WG/8 (see AI 06-04).

10.	Date of next meeting

10.1	The FSMP-WG/8 meeting is scheduled for 21-29 January, 2019 at ICAO Headquarters in Montreal, Canada.  Papers for FSMP-WG/8 are due one week prior to the meeting. For planning purposes it was also noted that FSMP/3 and FSMP-WG/9 are scheduled for 22-30 August, 2019 also at ICAO Headquarters in Montreal, Canada.

APPENDICES

Appendix A – Agenda

Appendix B – List of Working Papers, Information Papers and Flimsies

Appendix C – List of Participants

Appendix D – Action Item List

Appendix E – Initial Updates to ICAO WRC-19 Position

Appendix F – Elements for ICAO response to ITU WPP5B regarding protection criteria for VHF AM(R)S

Appendix G – Elements for consideration of a possible future WRC Agenda item under AI10 for the next FSMP meeting

Appendix H– Correspondence between Secretary of FSMP and Chair of CEPT ECC FM44, leveraging the draft responses contained in FSMP-WG/07 WP/03

Appendix I– Frequency coordination criteria for ILS and VOR

Appendix J– Ongoing consultation in Europe that may prove problematic to aviation

Appendix K- Proposed phrasing of WAIC SARP regarding tolerance to interference from sources outside the 4200-4400 MHz band

APPENDIX A

INTERNATIONAL CIVIL AVIATION ORGANIZATION

seventh working Group Meeting of the

Frequency Spectrum Management Panel

(FSMP-WG/7)

(Johannesburg, South Africa, 6-13 September, 2018)

AGENDA

Agenda Item 1	Opening and Working Arrangements

    - Action Item Review

    - Status of Tasks Identified on Job Cards

Agenda Item 2	ICAO WRC-19 Position

- Updates to draft Position

- Future Agenda Item proposals (WRC-19 Agenda Item 10 items)

Agenda Item 3	Radio Altimeter and Wireless Aircraft Intra-Communications (WAIC) issues

  - Status of Compatibility Testing

  - Radio altimeter out-of-band selectivity mask

Agenda Item 4	Development of (planned) Material for ITU-R Studies on:

- Fixed Satellite Services (FSS) for Unmanned aircraft system (UAS)

- Global Aeronautical Distress and Safety System (GADSS)

- Status of proposed update to Recommendation ITU-R SM.1009

- Aeronautical protection criteria (e.g., VHF Comm)

Agenda Item 5	5 GHz Band Planning

- AeroMACS status

- Global UAS/ Remote Piloted Aircraft System (RPAS) channel plan

Agenda Item 6	New Provisions to Support Aeronautical Radiocommunications

  - L-band Digital Aeronautical Communication System (LDACS), status update CP WG-T

  - RPASP Discussion of Direct Ground-Ground Very High Frequency (VHF) Communications

Agenda Item 7	Interference From Non-Aeronautical Sources

- Programme Making and Special Events (PMSE)

- Long Term Evolution (LTE) introduction adjacent to 1518-1559 MHz Satellite Communication (SATCOM) Band

Agenda Item 8	Revision of the ICAO Frequency Spectrum Handbook (Doc 9718)

Agenda Item 9	Any Other Business

APPENDIX B

List of Papers

List of Working Papers

| Paper # | Source | Title | Agenda Item |
| --- | --- | --- | --- |
| 1 | Secretary | Liaison Statement from WP5B regarding characteristics and protection criteria | 2,4 |
| 2 | Secretary | Liaison Statement from WP4C regarding IMT in a frequency band adjacent to 1518-1559 MHz | 4 |
| 3 | Secretary | Generic wedge shaped area, for airports and helipads, for the needs of CEPT ECC Report 272 | 9 |
| 4 | M. Biggs | Proposed Modifications to ICAO WRC-19 Position | 2 |
| 5 | M. Biggs | GADSS Methods in Draft CPM text | 2 |
| 6 | M. Biggs | Draft CPM for WRC-19 Agenda Item 9.1, Issue 9.1.4 on suborbital vehicles | 2 |
| 7 | M. Biggs | Status of proposal to modify SM.1009 | 4 |
| 8 | C. Fleury | Study on the impact of non-GSO satellites operating under the space operation service in the band 137-138 MHz on ICAO system operating in the band 136-137 MHz (VDL MOD2) | 4 |
| 9 | C. Fleury | Consideration of ICAO Position regarding the Russian proposal at ITU and CEPT levels | 2 |
| 10 | F. Ribaud | On the methodologies to allocate terrestrial C2 link frequency channels within the 5030-5091 MHz frequency band to a realistic UAS traffic | 5b |
| 11 | F. Butsch | Future use of the band 136.500 – 136.875 MHz | 9 |
| 12 | Secretary | Draft SARPs for Annex 10, Volumes I, III, IV and V | 9 |
| 13 | M. Biggs | Protection Criteria for Aeronautical Mobile (R) Service Systems Operating in the Frequency Band 117.975-137 MHz | 4 |
| 14 | M. Biggs | Study of Compatibility Between Aeronautical Mobile (R) Service Systems Operating in the Frequency Band 117.975-137 MHz and Possible Space Operations Service Systems Operating in the Frequency Bands 137-138 MHz (space-to-Earth) and/or 148-149.9 MHz (Earth-to-space) | 4 |
| 15 | Airports Authority of India | Rationale for inclusion of materials on frequency channeling plan of aeronautical bands and minimum technical parameters of ICAO systems in Doc 9718 (Volume- D) | 8 |
| 16 | N. Yonemoto | Consideration to allocate to the satellite service (Space to Earth) from 5030 MHz to 5090 MHz | 5b |
| 17 | J. Mettrop | Electronic Conspicuity in the UK | 9 |
| 18 | J. Mettrop | WRC-19 Agenda item 1.7 Short Duration Non-Geostationary Satellites | 2 |
| 19 | NSP SWG Papporteur | Methodology for the L5/E5a Aggregate Non-Aeronautical Interference Mask | 9 |
| 20 | Dmitriy Muzychenko | Introduction of a return link in 1559-1610 MHz frequency band in GADSS | 4b |
| 21 | A. Roy | UPDATE ON ICAO SPECTRUM HANDBOOK CORRESPONDENCE GROUP’S ACTIVITIES | 8 |
| 22 | A. Roy | PROPOSED STRUCTURE AND CHANGES TO ICAO HANDBOOK – SYSTEM CHARACTERISTICS AND BAND POLICY | 8 |
| 23 | A. Roy | SPECTRUM GUIDANCE IN AVIATION STANDARDS DEVELOPMENT | 8 |
| 24 | Secretary | Liaison Statement from CEPT ECC PT1 to the International Civil Aviation Organization – L-Band aeronautical MSS | 7 |
| 25 | J. Mettrop | Inadvertent Own Goals | 9 |
| 26 | Singapore | ALLOCATION OF VHF FREQUENCY BAND FOR SPACE-BASED VERY HIGH FREQUENCY (VHF) VOICE SERVICES | 2 |
| 27 | U. Schwark | Update on Draft SARPs for Wireless Avionics Intra-Communications (WAIC) | 3 |
| 28 | J. Mettrop | Volume B – Spectrum Draft Structure | 8 |
| 29 | K. Masrani | Current status of work within CEPT ECC PT1 on the development of the draft ECC Report on Proportionate Measures to address the potential blocking from IMT/LTE base stations below 1 518 MHz on MES receivers operating in bands adjacent to 1 518 MHz (including 1 525-1 559 MHz) at sea ports and airports | 7b |
| 30 | C. Fleury | LIAISON STATEMENT REGARDING THE IMPACT OF UPPER ALTITUDE PLATFORMS ON UPPER AIRSPACES | 9 |
| 31 | L. Tele | Update on the Aviation use of VSAT in AFI Region | 8 |
| 32 | R. Khatcherian | PROPOSED STRUCTURE AND CHANGES TO ICAO HANDBOOK VOLUME A – ICAO SPECTRUM POLICY, VISION and STRATEGY | 8 |

List of Information Papers, Presentations and Flimsies

| Paper # | Source | Topic | Agenda Item |
| --- | --- | --- | --- |
| 1 | F. Butsch | Report of the ICAO NSP Spectrum Working Group | 9 |
| 2 | C. Pichavant | Radiated and conducted emissions test results performed on LED bulbs disturbing VHF frequency band | 7 |
| 3 | H. Watanabe | FSMP WG/6 Report, Action Item 04-05, Weight Survey | 9 |
| 4 | NICT | Propagation Measurement of Ka-band for Unmanned Aircraft Systems (UAS) | 4a |
| 5 | NICT | An Experiment on Air-to-Air Communication Toward Location Information Sharing and Traffic Management for Drones and Manned Helicopters in the Same Airspace | 9 |
| 6 | H. Watanabe | The study of frequency sharing compatibility between aircraft radio altimeter and 5G mobile communication systems | 3 |
| 7 | N. Vassiliev | Correspondence between ITU and ICAO concerning Resolution 155 (WRC-15) and the development of SARPs for RPAS | 4 |
| 8 | N. Yonemoto | Experimental investigation of adjacent-band interference into radio altimeter due to LTE-Advanced base stations | 3 |
| 9 | A. Roy | COMPARISON OF VDL STANDARDS ON THEIR SUSCEPTIBILITY TO LED INTERFERENCE | 7 |
| 10 | F. Butsch | LDACS (L-band Digital Aeronautical Communications System) versus DME/TACAN Compatibility Testing Plan | 6a |
| 11 | R. Zakrzewski | Update on Development of MASPS for WAIC | 3 |
| 12 | J. Mettrop | Operation of Low Power Programme Making and Special Event wireless Microphones in the Frequency Band 960-1164 MHz | 7 |
| 13 | K. Masrani | Draft Liaison Statement from IMO to ITU-R WP 4C AND WP 5D AND CEPT ECC PT 1 on possible interference of terrestrial mobile communications with L-band maritime satellite communications | 7 |
| 14 | Secretary | LED RF devices update | 7 |
| 15 | D. Redman | RADIO ALTIMETER INTERFERENCE SUSCEPTIBILITY TESTING STATUS UPDATE | 3 |

| FLIMSIES |  |  |  |
| --- | --- | --- | --- |
| 1 | Rapporteur | Initial updates to ICAO WRC-19 Position | 2 |
| 2 | Rapporteur | Elements for response to WP5B on protection criteria for VHF AM(R)S | 4 |
| 3 | J. Taylor | Elements for consideration of a possible future WRC Agenda item under AI10 for the next FSMP meeting | 2 |
| 4 | Secretary | Correspondence between Secretary of FSMP and Chair of CEPT ECC FM44, leveraging the draft responses contained in FSMP-WG/07 WP/03 | 9 |
| 5 | R. Zakrzewski | Proposed WAIC SARPs text for section xx.4.5 concerning Out-of-Band Interference Tolerance | 3 |

APPENDIX C

Seventh meeting of the Working Group of the Frequency Spectrum Management Panel (FSMP-WG/6)

Johannesburg, South Africa    6-13 September, 2018

Attendance List

| COUNTRY/ ORGANIZATION | NO. | NAME | DESIGNATION-JOB TITLE/ TELEPHONE | EMAIL |
| --- | --- | --- | --- | --- |
| AUSTRALIA |  | Edmondo D’Amico | Senior Engineering Specialist (RF Spectrum) Air Services Australia Tel: +61-2-6268-5443 | eddy.damico@airservicesaustralia.com |
| BOTSWANA |  | Tebogo Maketlo | CNS Inspector Civil Aviation Authority of Botswana Tel: +267 73236191 | tmaketlo@caab.co.bw |
| BOTSWANA |  | Samuel Chiliwa Mbaakanyi | Chief Inspector – ANS Civil Aviation Authority of Botswana Tel: +Office:+267 368836 Cell: +267 72277905 | smbaakanyi@caab.co.bw smbaakanyi@gmail.com |
| BRAZIL |  | Vahe Antoine Yaghdjian | Telecommunications Engineer Department of Air Space Control – DECEA Technical Subdepartment – Technical Coordination Division Rio de Janeiro - Brazil Tel: +21- 2101–6487/21-99955–3305 | vahevay@decea.gov.br vahe.antoine@gmail.com |
| CAMEROON |  | Pierre Olivier Ntongmo | CNS Inspetor, Sous-directeur de la Navigation Aérienne et des Aérodromes Cameroon Civil Aviation Authority CAMEROON Tel: +237 222 3030 90 | Olivier.ntongmo@ccaa.aero contact@ccaa.aero |
| CAMEROON |  | Robert Ngongang | CNS Inspector Cameroon Civil Aviation Authority CAMEROON Tel: +237 222 3030 90 | contact@ccaa.aero |
| CAMEROON |  | Chantal Mimbang Lombol | Cadre au Service CNS CNS Inspector Cameroon Civil Aviation Authority CAMEROON Tel: +237 222 3030 90 | contact@ccaa.aero |
| CANADA |  | John Taylor | Aeronautical Spectrum Regulations Specialist Canada Tel: +1-613-993-4061 | john.taylor@tc.gc.ca |
| DEMOCRATIC REPUBLIC OF CONGO |  | Alberto Elombosila Ayeganagato | CNS Technical Controller Civil Aviation Authority Republic Démocratique du Congo Tel: +243 998816527/+243 810187194 | elombosila@aacrdc.org |
| DEMOCRATIC REPUBLIC OF CONGO |  | Shaby Shamba Mbiya | CNS Technical Controller Civil Aviation Authority Republic Démocratique du Congo Tel: +243 903241130/+243 819229181 | shabyshamba@aacrdc.org |
| DEMOCRATIC REPUBLIC OF CONGO |  | Yamba Bienge | CNS Service Chief Civil Aviation Authority Republic Démocratique du Congo Tel: +243 999925549/+243 818125916 | nehemieyambabienge@gmail.com |
| FRANCE |  | Christian Fleury | IESSA - Spectrum Manager France DGAC/ Direction des services de la Navigation Aérienne Toulouse – France Tel: +33-0-562-14-55-41 Cell: +330625164420 | christian.fleury@aviation-civile.gouv.fr |
| FRANCE |  | Dr. Christophe MACABIAU | Researcher/Lecturer, Head of ENAC TELECOM Research Team FRANCE, ENAC Tel: +33562174277 | macabiau@recherche.enac.fr; |
| FRANCE |  | Dr. Florian Ribaud | Researcher (Post. Doctorate) at ENAC TELECOM Research Team FRANCE, ENAC Tel: +33562174298 | florian.ribaud@recherche.enac.fr; |
| FRANCE |  | Pierre Ladoux | Head of Frequency and Spectrum Management Unit at DSNA/DTI France Tel: +33612503517 | pierre.ladoux@aviation-civile.gouv.fr |
| GERMANY |  | Dr. Felix Butsch | Head of Frequency Management DFS Deutsche Flugsicherung GmbH, (German Air Navigation Services) SIS/MF (Spectrum and Frequency Management Office) - Am DFS-Campus 10 D-63225 Langen, GERMANY Tel: +496103707-1533 Cell: +49-172 6677682 | Felix.Butsch@DFS.DE |
| GERMANY |  | Martin Weber | Tel: +49 4626230 | martin.weber@bnetza.de |
| GERMANY |  | Dominik Meyer | Head of Frequency Management Federal Supervisory Authority for Air Navigation Services, Germany Tel: +49 6103 8043 320 | Dominik.meyer@baf.bund.de |
| GHANA |  | Mr. Frank Kofi Apeagyei |  | fkapeagyei@gmail.com |
| GHANA |  | Mr. Isaiah Kofi Tefutor | Communications Manager Ghana Civil Aviation Authority | itefutor@gcaa.com.gh istefutor@yahoo.com |
| INDIA |  | Ajay Kumar Kapur | General Manager (CNS) India / Airports Authority of India Tel: +91 9868815045 | akkapur@aai.aero |
| INDIA |  | Arvind Singh Yadav | Joint General Manager (CNS) India/Airports Authority of India Tel: +91 11 24643247/+91 9599186558 | asyadav@aai.aero |
| JAPAN |  | Hiroshi Watanabe | Special Assistant to the Director Japan CNS planning Office, Air Navigation Services Department Civil Aviation Bureau, Ministry of Land, Infrastructure, Transport and Tourism Tel: +81-3-5253-8111 | watanabe-h48t8@mlit.go.jp |
| JAPAN |  | Toshio Nomi | Senior Engineer Japan Radio Air Navigation Systems Association (JRANSA) Tel: +81-3-5214-1353 | nomi.toshio@jransa.or.jp |
| JAPAN |  | Naruto Yonemoto | Principal Researcher Japan Electronic Navigation Research Institute (ENRI) Tel: +81-422-42-3174 | yonemoto@mpat.go.jp |
| JAPAN |  | Hiroyuki Tsuji | Research Manager Japan National Institute of Information and Communications Technology (NICT) Tel: +81-42-327-6034 | tsuji@nict.go.jp |
| JAPAN |  | Takeshi Tomoda | Researcher Japan Mitsubishi Research Institute, Inc. (MRI) Tel: +81-3-6705-6039 | tatomoda@mri.co.jp |
| JAPAN |  | Fumie Ono | Senior Researcher Japan National Institute of Information and Communications Technology (NICT) Tel: +81-46-847-5094 | fumie@nict.go.jp |
| KENYA |  | Erick Meli | Chief Technical Officer Kenya Civil Aviation Authority Tel: +254206827470 Fax: +254206822300 Cell: +0722791817 | erickmeli@kcaa.or.ke; |
| KENYA |  | Portas O. Oganga | Chief CNS Inspector Kenya Civil Aviation Authority Nairobi-Kenya. Tel: +254-20-827470 Cell: +254-722-834666 | pooganga@kcaa.or.ke; |
| KENYA |  | Nicholas Odwar | Acting Manager Engineering Services Kenya Civil Aviation Authority | nodwar@kcaa.or.ke; |
| KOREA |  | Yongsup Shim | Ministry of Science and ICT National Radio Research Agency Republic of Korea Tel: +82-61-338-4623 | Wave0908@korea.kr |
| MALAWI |  | Rutherford Chayima Kanunkha | Chief Aeronautical Telecommunications Engineer Department Of Civil Aviation – Malawi Tel: +265 888 743 533 | Rutherfordkanunkha@yahoo.co.uk |
| NETHERLANDS |  | Gerlof Osinga | Senior Manager Netherlands Tel: +31653548495 | Gerlof.osinga@agentschaptelecom.nl |
| ROMANIA |  | Aurelian Sorinel Calinciuc | Spectrum Manager Vice-Chair ITU-R SG5 Tel: +40721291274 | aurelian.calinciuc@ancom.org.ro |
| RUSSIAN FEDERATION |  | Dmitriy Muzychenko | Institute оf Air Navigation of GosNII GA Russia/Branch Tel: +7-903-174-82-24 | muzychenko@atminst.ru |
| SENEGAL |  | Souleymane FALL | Chef Service CNS Senegal/ANACIM Tel: +221-77536-35 66 | souleymane.fall@anacim.sn |
| SEYCHELLES |  | Sherille Fathiya Quatre | ANS Inspectorate (CNS) Seychelles Civil Aviation Authority Tel: +248 4384288 | fpillay@scaa.sc |
| SEYCHELLES |  | Christian Ng Ping Cheun | CNS Engineer Seychelles Civil Aviation Authority Tel: +2482619684 | ccheun@scaa.sc |
| SEYCHELLES |  | Pitter Elizabeth | CNS Manager Seychelles Civil Aviation Authority Tel: +2484384202 | pelizabeth@scaa.sc |
| SINGAPORE |  | John Chong | Principal Engineer Communications NAVAIDS Projects Civil Aviation Authority of Singapore Tel: +65-9369-6482 | John_chong@caas.gov.sg |
| SINGAPORE |  | Cheng Nam Yeo | Director Aeronautical Telecommunications and Engineering Civil Aviation Authority of Singapore Tel: +65-9776-9376 | yeo_Cheng_Nam@caas.gov.sg |
| SOUTH AFRICA |  | Lisa Tele | Senior Engineer/Project Manager ATNS Head Office Johannesburg - South Africa Tel: +2711 6071134 Cell: +2783 779 3920 | LisaT@atns.co.za |
| SOUTH AFRICA |  | Bhuttie Keagile | Manager CNS South African Civil Aviation Authority South Africa Tel: +0115451089 | keagileb@caa.co.za |
| SOUTH AFRICA |  | Thomas Machobane Matabane | Inspector Frequency Spectrum South African Civil Aviation Authority South Africa Tel: +0115451066 | matabanet@caa.co.za |
| SOUTH AFRICA |  | Takalani Tshikalaha | Senior Engineer South Africa: ATNS Tel: +011-607-1140 | TakalaniT@atns.co.za |
| SOUTH SUDAN |  | Cook Kabbash Morris Badigo | Senior Inspector Aeronautical Engineering Department South Sudan Civil Aviation Authorit Juba International Airport-RSS Tel: +211926089455/+211916306760 | kabasmorris@gmail.com |
| SOUTH SUDAN |  | Thomas Rizik Hasanen Rajab | Avionics Engineer Aeronutical Engineering Department SSCAA-JIA - South Sudan Civil Aviation Authority - Juba International Airport Republic of South Sudan Tel: +211926664663 | Thomas.laweya@gmail.com |
| SUDAN |  | Khedir Elemam Ahmed | Head of Installation & Configuration Section Sudan Civil Aviation Authority Tel: +249912163666 | Khadeercaa@gmail.com |
| UGANDA |  | Andrew Mulindwa Mwesige | Principal Technical Officer-SMS Uganda Civil Aviation Authority UGANDA Tel: +256-778-976402 | amwesige@caa.co.ug |
| UGANDA |  | Peter Okello | Principal Technical Officer-Planning Uganda Civil Aviation Authority UGANDA Tel: +256-772-963515 | pokello@caa.co.ug pokello@usa.com |
| UNITED REPUBLIC OF TANZANIA |  | Clara Rangimoto Mpili | Chief Air Navigation Inspector Tanzania Civil Aviation Authority Tel: +255 754 692984 | cmpili@tcaa.go.tz |
| UNITED REPUBLIC OF TANZANIA |  | Zawadi Juma Maalim | Principal Air Navigation Engineer Tel: +255-755-447414 | zmaalim@tcaa.go.tz |
| UNITED KINGDOM |  | John Ernest Mettrop | Spectrum Policy Specialist UK Civil Aviation Authority Tel: +44-0-1293 573477 | John.Mettrop@caa.co.uk |
| UNITED KINGDOM |  | Kamlesh Masrani | Manager, Spectrum Inmarsat Global Limited Tel: +44 20 7728 1338 | Kamlesh.Masrani@inmarsat.com |
| UNITED KINGDOM |  | Richard Leithall | UTC Aerospace Systems +44 78017 15347 | richard.leithall@utas.utc.com |
| USA |  | Michael Biggs | Senior Electronics Engineer USA Tel: +1-202-267-8241 | MICHAEL.BIGGS@FAA.GOV |
| USA |  | Dr. David Redman | Director, AVSI USA Tel: +1-979-218-2272 | dredman@tamu.edu |
| USA |  | Michelle Jackson | Electronics Engineer Federal Communications Commission Tel: +1-202-418-0360 | Michelle.Jackson@fcc.gov |
| USA |  | Scott Kotler | Director, Technical Regulatory Affairs United States of America Tel: +1-703-789-3923 | SCOTT.KOTLER@LMCO.COM |
| USA |  | Dr. Radoslaw Zakrzewski | Associate Director, Research Engineering USA/UTC Aerospace Systems Tel: +1-802-458-2610 | radek.zakrzewski@utas.utc.com |
| USA |  | Damon Ladson | Technology Policy Advisor United States of America Tel: +1 301 728 7253 | dladson@hwglaw.com |
| USA |  | Robert Denny | Electronics Engineer U.S Department of Commerce National Telecommunications & Information Administration Tel: +1-202-430-0879 | rdenny@ntia.doc.gov |
| ZAMBIA |  | Lt. Col. Emmanuel Chitembo | Director Air Navigation Services Zambia Civil Aviation Authority Tel: +260 211 251248 | Emmanuel.Chitembo@caa.co.zm |
| ZAMBIA |  | Harrison John Banda | Chief Inspector - CNS Zambia/CAA Tel: +260 966880699 | Harrison.banda@caa.co.zm |
| ASECNA |  | Mr. Sougue Bissa | CNS/ATM Officer Tel: +221776542355 | souguebis@asecna.org souguebiss@gmail.com |
| ASRI |  | Andrew Roy | Director of Engineering Aviations Spectrum Resources Inc. Tel: +1-443-951-0340 | ACR@asri.aero |
| ASRI |  | Kris Hutchison | Aviations Spectrum Resources Inc. Tel: +1 443 951 0322 | KEH@asri.aero |
| EUROCONTROL |  | Raffi Khatcherian | Head of International Spectrum Management EUROCONTROL Tel: +32 478241421 | raffi.khatcherian@eurocontrol.int |
| IATA |  | Noppadol Pringvanich | Global Head Air Traffic Management Engineering and Aviation Radio Spectrum International Air Transport Association Montreal, QC - Canada Tel: +1-438- 258-4714 | pringvanin@iata.org |
| IATA |  | Tim Totten | UPS Tel: +1-502-419-5694 | ttotten@ups.com |
| ICCAIA |  | Mr. Joseph Cramer | Regional Director ICCAIA - The Boeing Company Tel: +1-703-465-3486 | Joseph.Cramer@Boeing.com |
| ICCAIA |  | Claude Pichavant | ICCAIA Advisor Senior Expert – COM SUR Tel: +33622452389 | claude.pichavant@airbus.com |
| ICCAIA |  | Luiz Fernando de Souza | Flight Test Instrumentation EMBRAER- Gavião Peixoto-SP Tel: +55 16 33389173 Cell: +55 16 981672275 | lfsouza@embraer.com.br |
| ICCAIA |  | Uwe Schwark | AIRBUS Operations GmbH Kreetslag 10 21129 Hamburg Germany Tel: +49 (0) 40 743-72908 Mobile: +49 (0) 151 14081929 | uwe.schwark@airbus.com |
| ICAO |  | Loftur E. Jónasson | International Civil Aviation Organization (ICAO) Tel: +1-514-954-8219 Ext. 7130 | ljonasson@icao.int |
| ICAO |  | François Xavier Salambanga | ICAO Western and Central African (WACAF) Office Tel: +221 338692415 | Fsalambanga@icao.int |
| ICAO |  | Harvey G. Lekamisy | ICAO Eastern and Southern African (ESAF) Office Tel: +254-20-7622370 | Hlekamisy@icao.int |

APPENDIX D

ACTION ITEM LIST

| Number | Description | Actionee | Due Date | Status |
| --- | --- | --- | --- | --- |
| 32-8 | Review the proposal to develop a definition of “aviation safety system” and provide draft inputs as appropriate | All | FSMP-WG/6 | Still open. Input provided in WG-FSMP/1 Flimsy 1 for Comm systems |
| 02-6 | Develop a timeline for FSMP actions regarding resolves 18 of Resolution 155 (WRC-15) | All (working with RPASP) | FSMP-WG/7 | CLOSED (IP07) |
| 02-11 | Develop a simple example outlining the approach for aviation system protection suggested in FSMP-WG/2 WP24. | J. Mettrop | FSMP-WG/7 |  |
| 03-03 | Provide comment on the spectrum sharing approach between terrestrial and satellite RPAS C2 systems for the 5 030-5 091 MHz as proposed in FSMP-WG/3 WP10 and FSMP-WG/4 WP17 | All | FSMP-WG/8 | Ongoing but meeting noted WP10 and the European FMG action to remove most of the MLS assignments in Europe. |
| 04-03 | Develop material on aviation use of VSAT for Chapter 7 of doc 9718, Volume I, for the next update. | Lisa Tele, Bissa Sougue | FSMP-WG8 | CLOSED (WP31) |
| 04-05 | Provide input to complete the equipment physical characteristics (e.g., weight) table shown in the Annex of FSMP-WG4/WP26. | All | FSMP-WG8 | IP03 provides information |
| 04-06 | With regard to the action to “Conduct an aircraft fleet equipage impact analysis and develop detailed transition plans based on industry input and expected safety benefit” in the radio altimeter job card, initiate outreach to the airlines and aircraft manufacturers. | IATA and ICCAIA | FSMP-WG7 | Ongoing testing in FSMP-WG/7 IP16 covered the majority of radio altimeter models being used by airline operators. See also Presentations WG06-WRC19Wrkshp25 and WG07-WRC19WrkShp5 for examples of outreach activities. |
| 05-03 | Review the proposed ground-ground VHF communications scheme detailed in FSMP-WG/5 IP07 and provide comments to be included in a liaison to the RPASP. | All | FSMP-WG/7 | CLOSED RPASP Section chief has stated that there are no active discussions on the issue and it will be flagged to the FMSP if it is raised again. |
| 05-06 | Explore restructuring of Doc 9718 Volume 1. | Secretary/A. Roy/correspondence group | FSMP-WG/8 (status) Q3 2020 (complete) | FSMP-WG/7 WP15,WP21, WP22, WP23WP28, WP31 & WP32 progressed the effort |
| 06-01 | Based on studies within CEPT PT1 and ITU-R, develop general guidelines that can be used by spectrum regulators to ensure protection of aeronautical systems from the introduction of IMT/LTE in the frequency band below 1518 MHz. | ALL | FSMP-WG/8 | FSMP-WG/7 WP02, WP24, WP29 and IP13 provided information. |
| 06-02 | Liaise material for response liaison to WG SE regarding a new detection and collision avoidance application in the frequency band 446-457.1 kHz to NSP SWG for review. | Secretary | March 2018 | CLOSED |
| 06-03 | Begin review and update of ICAO WRC-19 Position | ALL | FSMP-WG/8 | See FSMP-WG/7 Flimsy 1 (Appendix E of this report) |
| 06-04 | Provide any proposals for aviation-related WRC-23 agenda items | ALL | FSMP-WG/7 |  |
| 06-05 | Provide to L. Jonasson and R. Khatcherian any comments on the draft report “Preliminary investigations on regulatory and legal issues on the feasibility of introducing low power audio PMSE in the band 960-1164 MHz” in time for the next FM51 meeting. | ALL | Feb 23, 2018 | CLOSED |
| 06-06 | Provide L. Jonasson with any comments on FSMP-WG/6 WP05 (draft ICAO Secretariat Working Paper on Integrated CNS and Spectrum Strategy for the 13th Air Navigation Conference). | ALL | March 1, 2018 | CLOSED |
| 07-01 | Provide comments on the LDACS test plan contained in FSMP-WG/7 IP10. Comments can be provided via E-mail to: Felix.Butsch@DFS.de | ALL | September 30, 2018 |  |
| 07-02 | Liaise FSMP-WG/7 WP11 with the Secretary of the CP-DCIWG to get their view on the proposal. | Secretary | FSMP-WG/8 |  |
| 07-03 | Liaise with the Secretary of the SP to get their view on the capacity of 1090ES and/or UAT to handle surveillance of small drones operating outside controlled airspace. | Secretary | FSMP-WG/8 |  |
| 07-04 | Participate in correspondence group to complete WAIC SARPS. | All | FMSP-WG/8 |  |
| 07-05 | Liaise FSMP-WG/7 WP30 with the appropriate body in ICAO and report back to FSMP. | Secretary | FSMP-WG/8 |  |

APPENDIX E

Initial Updates to the ICAO WRC-19 Position

APPENDIX F

Elements for ICAO response to ITU WPP5B regarding protection criteria for VHF AM(R)S

APPENDIX G

Elements for consideration of a possible future WRC Agenda item under AI10 for the next FSMP meeting

APPENDIX H

Correspondence between Secretary of FSMP and Chair of CEPT ECC FM44, leveraging the draft responses contained in FSMP-WG/07 WP/03

[See Flimsy4 on restricted site of FSMP-WG/7 documents]

Hyperlink to password protected site

APPENDIX I

Frequency coordination criteria for ILS and VOR

APPENDIX J

Ongoing consultation in Europe that may prove problematic to aviation

The following wireless power transfer (WPT) report is out for Public Consultation:

https://cept.org/Documents/wg-se/45971/se-18-072a1_draft-ecc-report-on-wpt-ev-wi60

In this report, it can be seen that the spurious emission is quite strong, approximately 84dBµA/m @10m at 80 kHz.  Also, the spurious emission between 130 kHz and 535 kHz goes up to 20 dBµA/m @10meters.

In order to protect NDB (where aviation is secondary) at 450 kHz against the SaveX system, aviation analyses applied a protection distance of 75 meters for aggregated emissions of SaveX system (power of 7dBµA/m @10meters).

The WPT system emissions are 13 dB higher. Therefore a protection distance will also need to be applied.

In addition, in the band 130-535 kHz, the aeronautical radionavigation service is primary. As a result, Unlike the SaveX compatibility studies, aviation may also apply an additional safety margin in order to protect our primary service.

APPENDIX K

Proposed phrasing of WAIC SARP regarding tolerance to interference from sources outside the 4200-4400 MHz band

    - WAIC receivers shall be capable of tolerating interference from sources operating outside of the frequency band 4 200 – 4 400 MHz under the conditions specified as follows:

- The total combined power as measured in the WAIC receiver from those sources shall not exceed -30 dBm assuming a 40 dB/decade rejection roll-off from the band edges.

- The aggregate unwanted emissions into the 4200-4400 MHz band as measured at the WAIC receiver shall not exceed a power spectral density of -101 dBm / MHz