INTERNATIONAL CIVIL AVIATION ORGANIZATION (ICAO)

SURVEILLANCE PANEL (SP)

AERONAUTICAL SURVEILLANCE WORKING GROUP (aswg)

TECHNICAL SUB GROUP (TSG) MEETING 03

Paris, France, 27 June to 1 July 2016

Reply Liaison Statement (LS) to the European Telecommunications Standards Institute (ETSI)

(Prepared by Chris Tourigny, FAA)

SUMMARY

This working paper documents the ICAO ASWG TSG discussion on the ETSI LS presented in ASWG TSG WP3-30 and proposes a response for the SP Secretary.  Additional information may be necessary to respond to the questions in the LS.  The TSG request for additional information will be provided to the SP Secretary to coordinate with the Frequency Spectrum Management Panel (FSMP) Secretary to assist in formally responding to the ETSI LS.  This work is related to Action Item ASWG/3-26.

  - That the ETSI radar sub-group is correct in assuming that there are no regulatory provisions within ICAO relevant to the definition of transmit and receive parameters for primary radar?

ICAO response:

ICAO Standards and Recommended Practices (SARPS) are designed to maintain interoperability such that equipment on an aircraft from one country can communicate with similar equipment in another country. Such interoperability is not required for primary radar functions, and as a result ICAO doesn't maintain equipment requirements in the SARPs for primary radar. ICAO does, however, maintain guidance materials, such as ICAO Doc 9924, Aeronautical Surveillance Manual, and ICAO Doc 9718, Handbook on Radio Frequency Spectrum Requirements for Civil Aviation. In addition, while various radars have differing design requirements based on their operational applications, parameters for aeronautical radars are contained in a number of ITU-R Recommendations.

  - That the transmitter parameters for Mode A/C transmitters are contained in section 3.1.1 whilst those for Mode S are contained in Section 3.1.2 of Volume IV of Annex 10?

ICAO response:

Yes, those are the appropriate sections of the SARPs for the secondary surveillance radar (SSR) and transponder.

  - The reason as to why the frequency tolerance defined by ICAO for 1090 MHz transponder (3MHz Mode A/C & 1 MHz Mode S) is different to that required by the ITU Radio Regulatory requirements (500 parts in 106 or 545 kHz) and whether there are any plans to update these requirements?

ICAO response:

Appendix 2 of Volume II of the ITU RRs does, indeed, indicate a maximum tolerance of 500 parts per million for radiodetermination systems operating in the frequency range used by civil aviation SSR systems. This could appear to indicate differences in the frequency tolerance defined by ICAO and ITU; however it is apparent that there are differences within ITU-R as well for SSR systems.  Recommendation ITU-R SM.1045-1 Note 9 indicates that the 1090 MHz transponder may operate within a 3 MHz frequency tolerance, the ICAO SARPs Mode A/C transponder tolerance.

RR No. 35.1 of the ITU RRs indicates that the ICAO SARPs is an intergovernmental agreement that governs the use of aeronautical services, provided that the implementation doesn’t cause harmful interference to other countries.  The frequency band that the Mode S interrogator (1030 MHz) and transponder (1090 MHz) operate in only contains aeronautical services and the use of these services by international civil aviation is harmonized by ICAO.

ICAO develops aeronautical standards in an international, consensus-based process.  Any change in ICAO standards would be an ICAO-led effort following that same approach.

ICAO recommends that the frequency tolerance defined in the ICAO SARPs should be considered the international standard for civil aviation SSR systems.

  - The reason as to why the spectral masks defined by ICAO within Volume 1V of Annex 10 for the Mode S Interrogator (Figure 3-2) and Transponder (Figure 3-5) are such that they would appear not to conform with the ITU Radio Regulatory requirements and whether there are any plans to update these requirements?

ICAO Response:

References to the SARPs are provided, but it’s not clear to which ITU RR they are being compared to make the determination of non-conformity.  In order to properly evaluate the need for changes to the SARPs, please provide additional information, such as which RRs appear to be different than those defined in the SARPs.  ICAO  will then be able to evaluate any difference and any potential need to update the SARPs.

ICAO is of the view that this evaluation should also take into consideration RR No. 35.1, as pointed to in the last question.    In addition numerous references in ITU (e.g., Resolution 417 (Rev. WRC-15)) recognize ICAO Annex 10 as the international standards document used by civil aviation.

  - In the context of aircraft fitted devices, is there any other information that the ETSI Radar sub-group should take into account when preparing the European Harmonised Standards for secondary radars?

ICAO Response:

The following list contains the EUROCAE avionics standards for ATCRBS, Mode S, ADS-B, and TCAS.

·         Last Light Aviation Secondary Surveillance Radar Transponders MOPS:  Ed-115

·         Mode S transponder MOPS: ED-73E

·         1090 ADS-B MOPS: ED-102A

·         TCAS MOPS: ED-143

  - If in answer to question 1, ICAO does define transmit and receive parameters for primary radars:

a.           Do you have any comments on the information contained in the liaison statement attached to the ITU?

b.           Is there any other information that the ETSI Radar sub-group should take into account when preparing the European Standards for primary radars?

ICAO Response:

ICAO doesn’t define equipment specifications for primary radar.