	[image: image2.png]



	[image: image2.png]
International Civil Aviation Organization

WORKING PAPER
	FSMP-WG/17-WP/13
2023-08-30 




ATMRPP-WG/WHL/4-WP/


.././06
FSMP-WG/17-WP/13
- 2 -

- 3 -
FSMP-WG/17-WP/13

FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Seventeenth Working Group Meeting 
Cairo, Egypt – 30 August – 7 September 2023
Agenda Item 2b:
WRC-23 Preparations

                                   b) Discussion of expected future agenda item proposals from administrations
Brazilian Proposal for CITEL meeting on WRC-23 Agenda Item 10 
(Presented by Vahe Yaghdjian)

(Prepared by DECEA)

	SUMMARY

	Attached to this document is the contribution of Brazil on preliminary agenda item 2.9 for the 2027 World Radiocommunication Conference (WRC-27), which will be submitted at the 42nd CITEL PCC.II meeting to be held between August 28th and September 1st.
This contribution was approved by ANATEL during the last national meeting on WRC-23 agenda items, therefore, it is Brazil’s official position on this agenda item.



1. INTRODUCTION

1.1 ITU-R Resolution 812 (WRC-19) contains the preliminary agenda for WRC-27 and the AI 10, topic 2.9 is: “to consider possible additional spectrum allocations to the mobile service in the frequency band 1 300-1 350 MHz to facilitate the future development of mobile-service applications, in accordance with Resolution 250 (WRC-19).”
1.2 On a global basis, the band 1 300–1 350 MHz (and in many countries also the band 1 215–1 300 MHz) is extensively used for primary surveillance radar (PSR), for both en-route and terminal surveillance tasks, mainly providing long-range independent non-cooperative airspace surveillance for air traffic control (ATC) and national defense.
1.3 Although Resolution 250 (WRC-19) stablishes that the sharing and compatibility studies in the frequency band 1 300-1 350 MHz should ensure protection of existing services to which the band is allocated on a primary basis, the studies performed to date have not shown any potential compatibility with the systems operating in this band, therefore, there is a significant concern of a new WRC-27 Agenda Item considering a new mobile service allocation to the 1 300-1 350 MHz frequency band that could cause harmful interference to these incumbent radar systems and has the potential to harm the public safety. 
2. DISCUSSION

2.1 Resolution 250 (WRC-19) calls for “Studies on possible allocations to the land mobile service (excluding International Mobile Telecommunications) in the frequency band 1 300-1 350 MHz for use by administrations for the future development of terrestrial mobile-service applications”.1
2.2 Several studies under WRC-15 Agenda Item 1.1 dealing with International Mobile Telecommunication (IMT) and the same incumbent radar systems have been carried out with respect to the frequency band 1 300-1 400 MHz. All the studies show, based on the parameters provided by the relevant working parties, that within the same geographical area co-frequency operation of mobile broadband systems and radar is not feasible. As a result, globally harmonized usage of the 1 300-1 400 MHz frequency range or a portion thereof by the mobile service for the implementation of IMT may not be possible.2
2.3 It should also be noted that even the studies which concluded that it is feasible to introduce IMT systems in the 1 300-1 400 MHz frequency band require modification of the IMT and radar equipment.  Such studies also suggest frequency segmentation in accordance with Recommendation ITU-R SM.1132 which may involve replanning radar systems as necessary to remove radars from a portion of the frequency range to provide sufficient spectrum to accommodate the IMT channel, plus the frequency offset. Any consideration of radar replanning must consider that some administrations make use of radars that operate across the frequency range between 1 300-1 400 MHz.2
2.4 The ICAO Spectrum Strategy for long-term aeronautical surveillance systems (Handbook 9718 chapter 8, 8-10) establishes the continuous availability of the frequency band 1 215-1 350 MHz, which is allocated to the radionavigation and aeronautical radionavigation service, for use by primary surveillance radar on a global basis.3
2.5 Taking into account the potential risks for aviation, DECEA proposed the suppression of the preliminary WRC-27 Agenda Item 2.9 (more infornation in the attached file).
3. ACTION BY THE MEETING

3.1 The meeting is invited to:

a) Note the proposal  of this working paper and consider in coordination with the regulators of your administrations to support the proposed position stated by Brazil.
 _________________________________                           

https://www.itu.int/dms_pub/itu-r/oth/0C/0A/R0C0A00000F0088PDFE.pdf.
2https://www.itu.int/dms_ties/itu-r/md/12/jtg4567/c/R12-JTG4567-C-0715!N25!MSW-E.docx.
3https://www.icao.int/safety/FSMP/Documents/Doc9718/Doc.9718%20Vol.%20I%20(AdvanceUneditedVersion%202021).pdf.


[image: image1.emf]CCPII-2023-42-5883 _i.pdf


(2 pages)

FSMP-WG17-WP13_Brazilian Proposal for CITEL meeting on WRC-23 Agenda Item 10 .doc

_1754047874.pdf


 


ORGANIZACIÓN DE LOS ESTADOS AMERICANOS  
ORGANIZATION OF AMERICAN STATES  


 
Comisión Interamericana de Telecomunicaciones 
Inter-American Telecommunication Commission 


 


CITEL, 1889 F ST. NW., WASHINGTON, D.C. 20006, U.S.A. 


TEL: +1 202 370 4713   FAX: +1 202 458 6854    e-mail: citel@oas.org 


Web page: http://citel.oas.org 


42 MEETING OF PERMANENT 


CONSULTATIVE COMMITTEE II: 


RADIOCOMMUNICATIONS 


August 28 to September 01, 2023 


Ottawa, Canada 


OEA/Ser.L/XVII.4.2.42 


CCP.II-RADIO /doc. 5883/23 


06 August 2023 


Original: English 


 


 


 PRELIMINARY PROPOSAL FOR WRC-23 


AGENDA ITEM 10 (NO MS 1.3 GHz) 


 


 (Item on the Agenda: 3.1 (SGT-5))  


 (Document submitted by the delegation of Brazil)  


 
 


Impact on the sector: 
 


WRC-23 Agenda Item 10 is to recommend to the ITU Council items for inclusion in the agenda for the 


next world radiocommunication conference, and items for the preliminary agenda of future conferences, 


in accordance with Article 7 of the ITU Convention and Resolution 804 (Rev. WRC 19). 


 


Executive Summary: 


Brazil proposes the suppression of item 2.9 from the preliminary WRC-27 agenda which considers 


possible additional spectrum allocations to the mobile service in the frequency band 1 300-1 350 MHz to 
facilitate the future development of mobile-service applications, in accordance with Resolution 250 


(WRC-19). 


In our view, such proposal impacts the aviation long-range surveillance radar systems operating in the 


frequency band 1 300-1 350 MHz, used for air traffic control (ATC). 


Brazil invites other countries to join this preliminary position. 


 


  



mailto:citel@oas.org





 


CCPII-2023-42-5883_i                    06.08.23 2 


Agenda Item 10: to recommend to the ITU Council items for inclusion in the agenda for the next world 
radiocommunication conference, and items for the preliminary agenda of future conferences, in accordance 


with Article7 of the ITU Convention and Resolution 804 (Rev. WRC-19). 


 


BACKGROUND: 


ITU-R Resolution 812 (WRC-19) contains the preliminary agenda for the 2027 World 


Radiocommunication Conference (WRC-27) and item 2.9 is “to consider possible additional spectrum 


allocations to the mobile service in the frequency band 1 300-1 350 MHz to facilitate the future 


development of mobile-service applications, in accordance with Resolution 250 (WRC-19).” 


Resolution 250 (WRC-19) calls for “Studies on possible allocations to the land mobile service (excluding 


International Mobile Telecommunications) in the frequency band 1 300-1 350 MHz for use by 


administrations for the future development of terrestrial mobile-service applications1”. 


The frequency band 1 300-1 350 MHz is allocated to the radiolocation service (RLS), the aeronautical 


radionavigation service (ARNS) and the radionavigation-satellite service (E-s)(RNSS) on a primary basis, 


in all three Regions. 


Two relevant footnotes are applicable in the frequency band 1 300-1 350 MHz, 5.337 and 5.337A.  


No. 5.337 states “The use of the bands 1 300-1 350 MHz, 2 700-2 900 MHz and 9 000-9 200 MHz by the 


aeronautical radionavigation service is restricted to ground-based radars and to associated airborne 


transponders which transmit only on frequencies in these bands and only when actuated by radars operating 


in the same band”.  


No. 5.337A states “The use of the band 1 300-1 350 MHz by earth stations in the radionavigation-satellite 


service and by stations in the radiolocation service shall not cause harmful interference to, nor constrain the 


operation and development of, the aeronautical-radionavigation service. (WRC-2000)”. 


The frequency band 1 300-1 350 MHz is used by ICAO Member States for operating various types of long-


range radar systems that measures range, bearing, and velocity of aircraft, and perform missions critical to 


safe and reliable air traffic control (ATC) in the national airspace, border surveillance and drug interdiction. 
These radar systems ensure the safe transportation of people and goods, encourage the flow of commerce, 


and provide for national defense. Long-range radars are operated in this portion of the radio frequency 


spectrum because the effects of rain and fog on radar target detection are very low and the external 
background noise levels are low. These factors are important to achieve the long-range detection of different 


size aircraft as well as other targets2. 


Several studies under WRC-15 Agenda Item 1.1 dealing with International Mobile Telecommunication 


(IMT) and the same incumbent radar systems have been carried out with respect to the frequency range 
1 300-1 400 MHz All the studies show, based on the parameters provided by the relevant working parties, 


that within the same geographical area co-frequency operation of mobile broadband systems and radar is 


not feasible. As a result, globally harmonized usage of the 1 300-1 400 MHz frequency range or a portion 


thereof by the mobile service (MS) for the implementation of IMT may not be possible3. 


It should also be noted that all the studies which concluded that it is feasible to introduce IMT systems in 


the 1 300-1 400 MHz frequency range require modification of the IMT and radar equipment.  Such studies 
also suggest frequency segmentation in accordance with Recommendation ITU-R SM.1132 which may 


involve replanning radar systems as necessary to remove radars from a portion of the frequency range to 


 
1 https://www.itu.int/dms_pub/itu-r/oth/0C/0A/R0C0A00000F0088PDFE.pdf 
2 https://www.ntia.doc.gov/files/ntia/publications/compendium/1300.00-1350.00_01DEC15.pdf 
3 https://www.itu.int/dms_ties/itu-r/md/12/jtg4567/c/R12-JTG4567-C-0715!N25!MSW-E.docx 



https://www.itu.int/dms_pub/itu-r/oth/0C/0A/R0C0A00000F0088PDFE.pdf

https://www.ntia.doc.gov/files/ntia/publications/compendium/1300.00-1350.00_01DEC15.pdf

https://www.itu.int/dms_ties/itu-r/md/12/jtg4567/c/R12-JTG4567-C-0715!N25!MSW-E.docx





 


CCPII-2023-42-5883_i                    06.08.23 3 


provide sufficient spectrum to accommodate the IMT channel, plus the frequency offset. Any consideration 
of radar replanning must consider that some administrations make use of radars that operate across the 


frequency range between 1 300-1 400 MHz3. 


Although Resolution 250 (WRC-19) stablishes that the sharing and compatibility studies in the frequency 


band 1 300- 1 350 MHz should ensure protection of existing services to which the frequency band is 


allocated on a primary basis, the studies performed to date have not shown any potential compatibility with 
the systems operating in this band. Therefore, there is a significant concern of a new WRC-27 Agenda Item 


considering a new mobile service allocation to the 1 300-1 350 MHz frequency band that could provoke 


harmful interference to these incumbent radar systems and has the potential to harm the public safety4. 


The ICAO Spectrum Strategy for long-term aeronautical surveillance systems establishes the continuous 


availability of the frequency band 1215-1350 MHz, which is allocated to the radionavigation and 


aeronautical radionavigation service, for use by primary surveillance radar on a global basis5. 


Therefore, Brazil proposes the suppression of the preliminary WRC-27 Agenda Item 2.9. 


PRELIMINARY PROPOSAL 


MOD B/10 (NO MS 1.3 GHz)/1 


RESOLUTION 812 (WRC-19) 


Preliminary agenda for the 2027 World Radiocommunication Conference 


 


The World Radiocommunication Conference (Sharm el-Sheikh, 2019),  


(…) 


resolves to give the view 


(…) 


2.9 to consider possible additional spectrum allocations to the mobile service in the frequency band 1 


300-1 350 MHz to facilitate the future development of mobile-service applications, in accordance with 


Resolution 250 (WRC-19); 


(…) 


Reasons: While Resolution 250 (WRC-19) resolves to conduct sharing and compatibility studies to ensure 


protection of existing services to which the frequency band is allocated on a primary basis, the studies 
performed to date have not shown any potential for compatibility with the systems operated in this band. 


Furthermore, studies under WRC-15 Agenda Item 1.1 with IMT and the same incumbent radar systems 


demonstrated that co-frequency sharing was not possible. Therefore, there is significant concern of a new 


WRC-27 Agenda Item considering a new mobile service allocation to the 1 300-1 350 MHz frequency band 
that could provoke harmful interference to these incumbent radar systems and has the potential to harm the 


public safety. 


SUP B/10 (NO MS 1.3 GHz)/2 


 


 
4 https://www.icao.int/safety/FSMP/Documents/ITU-WRC23/SL.2023.60.english.pdf 



https://www.icao.int/safety/FSMP/Documents/ITU-WRC23/SL.2023.60.english.pdf





 


CCPII-2023-42-5883_i                    06.08.23 4 


RESOLUTION 250 (WRC-19)  


Studies on possible allocations to the land mobile service (excluding International Mobile 


Telecommunications) in the frequency band 1 300-1 350 MHz for use by administrations for the 


future development of terrestrial mobile-service applications 


Reasons: Consequential to non-inclusion of resolves 2.9 from the preliminary WRC-27 agenda on the 
WRC-27 agenda adopted by WRC-23. 


 


________________________ 


 
1 https://www.itu.int/dms_pub/itu-r/oth/0C/0A/R0C0A00000F0088PDFE.pdf 
2 https://www.ntia.doc.gov/files/ntia/publications/compendium/1300.00-1350.00_01DEC15.pdf 
3  https://www.itu.int/dms_ties/itu-r/md/12/jtg4567/c/R12-JTG4567-C-0715!N25!MSW-E.docx 
4 https://www.icao.int/safety/FSMP/Documents/ITU-WRC23/SL.2023.60.english.pdf 
5  https://www.icao.int/safety/FSMP/Documents/ITU-WRC23/SL.2023.60.english.pdf 


 



https://www.itu.int/dms_pub/itu-r/oth/0C/0A/R0C0A00000F0088PDFE.pdf

https://www.ntia.doc.gov/files/ntia/publications/compendium/1300.00-1350.00_01DEC15.pdf

https://www.itu.int/dms_ties/itu-r/md/12/jtg4567/c/R12-JTG4567-C-0715!N25!MSW-E.docx

https://www.icao.int/safety/FSMP/Documents/ITU-WRC23/SL.2023.60.english.pdf

https://www.icao.int/safety/FSMP/Documents/ITU-WRC23/SL.2023.60.english.pdf




