# WRC-27 Agenda Item 1.12

Agenda Item Title:

To consider, based on the results of studies, possible new allocations to the mobile-satellite

service and possible regulatory actions in the frequency bands 1 427-1 432 MHz (space-to-Earth), 1 645.5-1 646.5 MHz (space-to-Earth) (Earth-to-space), 1 880-1 920 MHz (space-to-Earth) (Earth-to-space) and 2 010-2 025 MHz (space-to-Earth) (Earth-to-space) required for the future development of low-data-rate non-geostationary mobile-satellite systems, in accordance with Resolution 252 (WRC-23);

# Discussion:

This agenda item considers  low data-rate non-GSO MSS systems in the following frequency bands:

- 1 427-1 432 MHz (space-to-Earth),

- 1 645.5-1 646.5 MHz (space-to-Earth and Earth-to-space),

- 1 880-1 920 MHz (space-to-Earth and Earth-to-space)

- 2 010-2 025 MHz (space-to-Earth and Earth-to-space)

According to RR No. 5.343, in Region 2 use of the radio frequency band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service.  Aeronautical Mobile Telemetry is used by the aviation industry to certify commercial aircraft.

This position is focused only on the possible use of the frequency band 1 645.5-1 646.5 MHz for low data-rate non-GSO MSS systems, in the Earth-to-space and space-to-Earth directions.  The potential future use of the band 1 645.5-1 646.5 MHz by low data-rate non-GSO MSS systems should consider the adjacent band co-existence with current and future aviation safety systems.

The frequency band 1 626.5-1 660.5 MHz is allocated to the MSS (Earth-to-space) where RR No. 5.357A provides priority in the frequency band 1 646.5-1656.5 MHz to accommodate the spectrum requirements of AMS(R)S. The frequency band 1 626.5-1 660.5 MHz is used for aviation SATCOM air-ground communications for ATC and other regularity of flight functions,

# ICAO Position:

To ensure that the results of this agenda item do not reduce the protection of, or impose additional regulatory or technical constraints on, aeronautical mobile-satellite (R) communications provided under the mobile satellite service allocations in the frequency band 1 646.5-1 656.5 MHz where, under No. 5.357A, priority shall be given to accommodate the spectrum requirements of the aeronautical mobile-satellite (R) service providing transmission of messages with priority 1 to 6 in Article 44.