|  | International Civil Aviation Organization INFORMATION PAPER | FSMP WG-F/33 IP10 2015-08-21 |  |
| --- | --- | --- | --- |

FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

THIRTY-THIRD MEETING OF WORKING GROUP F

Montreal, 25 - 28 August 2015

| Agenda Item XX |  |
| --- | --- |

Wireless Avionics Intra-Communications

Proposed Standardization Activities

(Prepared by Aerospace Vehicle Systems Institute (AVSI))

SUMMARY

This Information Paper puts forward for ICAO’s consideration a proposal that the Air Navigation Commission should establish a Working Group for development of Standards and Recommended Practices (SARPs) for Wireless Avionics Intra-Communications systems operated in the frequency band 4 200 – 4 400 MHz. Further, ICAO should support creation of an RTCA Special Committee and a EUROCAE Working Group that will subsequently develop a Minimum Operational Performance Standard to provide the means of compliance with the future SARPs.

ACTION The meeting is invited to take into account the information contained in this document and provide feedback as to the steps forward regarding WAIC.

# 1		INTRODUCTION

The civil aviation industry is currently developing the regulatory framework for using wireless communications onboard aircraft for implementing safety-related aircraft functions. Such applications are known under the term “Wireless Avionics Intra-Communications (WAIC)” [1] [5]. The regulatory framework for allowing operating WAIC-enabled avionics must cover both, radio regulatory aspects as well as aviation regulations. This paper discusses the current situation regarding the radio regulatory process and explores the steps to be pursued for putting proper aviation regulations in place after radio regulatory provisions have been made by World Radiocommunication Conference 2015 (WRC-15) [7]. Specifically, the relationship between the Radio Regulations (RRs) [8], ICAO Standards and Recommended Practices (SARPs) [9], EUROCAE/RTCA Minimum Operational Performance Standards (MOPSs) and industry standards are discussed. Furthermore, the linkage to activities related to the development of airworthiness certification guidance materials such as the ongoing activities within EUROCAE WG-96 are considered in this paper. EUROCAE is currently developing a generic process applicable for the certification of wireless systems for on-board aircraft communications within its Working Group 96 (WG-96).

# 2		RADIO REGULATORY AND STANDARDIZATION EFFORTS ON WAIC

2.1	RADIO REGULATORY EFFORTS

It is expected that WRC-15 will introduce provisions into the ITU Radio Regulations (RRs) including an allocation to the aeronautical mobile (route) service (AM(R)S) in the frequency band 4 200 – 4 400 MHz supporting the introduction of WAIC systems on a global scale. These provisions will also call ICAO for the development of Standards And Recommended Practices (SARPs) addressing the necessary technical requirements imposed on WAIC systems. SARPs are technical specifications adopted by the Council of ICAO in accordance with Article 37 of the Convention on International Civil Aviation (the “ICAO Convention”). SARPs are published by ICAO in form of Annexes to this convention. ICAO defines a standard as:

“any specification for physical characteristics, configuration, material, performance, personnel or procedure, the uniform application of which is recognized as necessary for the safety or regularity of international air navigation and to which Contracting States will conform in accordance with the Convention”.

The requirement for interoperability between equipment on-board one aircraft and those on-board another aircraft or on the ground is the main motivation for SARPs. With regards to WAIC, there is not necessarily the need for SARPs from this viewpoint since WAIC transmitters and receivers are both situated on-board the same aircraft and hence under the full control of the aircraft manufacturer or system integrator who ensures this interoperability. However, the frequency band 4 200 - 4 400 MHz (which will very likely be allocated to the AM(R)S in support of WAIC) is also used by radio altimeters on-board civil and state aircraft. The continued use of the frequency band for this essential safety application needs to be properly protected. Furthermore, it is a fundamental requirement that multiple WAIC systems on-board different aircraft which are in mutual radio range have to be able to coexist, i.e. to arrange their transmissions such that their performance requirements can be met. For these reasons the regulatory provisions which will be made by WRC-15 assume that ICAO will develop corresponding SARPs. These SARPs should provide “high level guidance” for the design and certification of WAIC systems. Beyond this, the group of WAIC proponents is of the view that more detailed Minimum Operation Performance Standards (MOPSs) are required to address the essential technical elements of WAIC systems to assure that they are able to fulfil their intended function under all operational conditions. Figure 1 below summarizes these necessary standardization steps. Section 2 of this document describes these steps in more detail.

Figure 1 Necessary standardization steps towards WAIC system certification

2.2	STANDARDIZATION EFFORTS

The international civil aviation industry is of the view that the required standard(s) for WAIC systems should be developed by the appropriate aeronautical standardization bod(y)/ies. The choice of the standards body will depend on the level of detail which the WAIC standard should address. It must address at least technical means to fulfil:

- the radio regulatory obligation to ensure safe coexistence between WAIC systems and radio altimeters in the frequency band 4 200 – 4 400 MHz,

- the WAIC-immanent requirement of multiple WAIC systems being able to coexist in the same frequency band, whilst

- reaching the intended Quality-of-Service requirements of WAIC systems.

These requirements are depicted in Figure 2 below.

Figure 2 Motivation for WAIC Systems standardizing WAIC systems to ensure coexistence between WAIC systems and radio altimeters as well as coexistence among WAIC systems

2.2.1	ICAO Standards And Recommended Practices

SARPs are seen as the proper means to address the essential technical requirements of WAIC systems on a “high level”. With regards to technical details it is expected that these SARPs make reference to the appropriate MOPS. Therefore, SARPs are seen to serve in the case of WAIC as formal binding element between the RRs and the aeronautical regulations.

In order to start the process of developing SARPs a corresponding proposal would have to be submitted to the ICAO Aeronautical Communication Panel (ACP). The ACP would have to formally task an existing Working Group or to launch a new Working Group for addressing the topic. In any case the ACP would have to develop a “Job Card” which is similar to Terms of References describing the task and scope of the corresponding Working Group.

2.2.2	RTCA/EUROCAE Minimum Operational Performance Standard

It is expected that at least a set of principles and rules of how WAIC systems use the frequency band (i.e. a channel plan and principles of multiple access techniques and other techniques supporting coexistence) has to be laid down in a standard. Such a standard could be in the form of a Minimum Operational Performance Standard (MOPS). Such standards specify the minimum requirements that an aeronautical system has to fulfill in order to meet its intended objectives. The Aerospace Vehicle Systems Institute (AVSI) project AFE 76 [10] is already tasked with developing WAIC system requirements (including co-existence requirements) and with making recommendations on communication protocols. The outputs of this activity can be used to shape the MOPS for WAIC systems.

2.2.3 Timeline for ICAO & RTCA/EUROCAE activities

Figure 3 below provides an overview over the planned activities within the AVSI project AFE 76 with regards to supporting the development of a MOPS for WAIC systems.

Figure 3 Timeline for ICAO & RTCA/EUROCAE activities

2.3	AIRWORTHINESS CERTIFICATION GUIDANCE MATERIAL – EUROCAE WG-96

EUROCAE WG-96 was established by the Technical Advisory Committee of EUROCAE late 2012 and its initial Terms of Reference were approved by the EUROCAE Council in January 2013. EUROCAE WG-96 aims at developing certification guidance material in form of a Process Specification (PS). A PS is a documents which

“specifies generic methods which are not specific to individual  components,  e.g. software or  hardware development, environmental testing ...”.

The scope of WG-96 currently covers general intra-aircraft wireless communication not necessarily limited to WAIC. The PS in its 1st version will be non-specific such that it does not differentiate between specific wireless standards or technologies and regulatory regimes under which the wireless system is operated. Essentially it means that the PS can be used for WAIC as well as for wireless implementations of aircraft system functions using legacy technology, such as IEEE 802.11x or similar standards operated in license-exempted frequency bands. During the work of WG 96 it became evident that for a quantitative assessment of communication performance an aviation industry wide standard is essential for defining expected worst-case conditions under which the wireless system needs to be capable to operate and provide its intended performance. Which standardization body will be involved in defining such as standard will be determined after the broad requirements for the standard are developed as part of the AVSI project AFE 76.

3	PROPOSAL

3.1	ICAO

It is proposed that the Air Navigation Commission of ICAO undertakes the necessary steps for establishing a Working Group tasked with the development of Standards and Recommended Practices (SARPs) for Wireless Avionics Intra-Communications systems operated in the frequency band 4 200 – 4 400 MHz. These SARPs shall cover the following essential requirements:

•	the general requirement for a WAIC system to comply with the ITU Radio Regulations

•	safe coexistence of Radio Altimeters and WAIC Systems onboard different aircraft and

•	safe coexistence of (a) WAIC System(s) onboard one aircraft and (b) WAIC System(s) onboard other aircraft.

3.2	RTCA / EUROCAE

It is proposed that RTCA and EUROCAE set up a Special Committee (SC) and a working group, respectively (ideally concerted in a joint effort) dealing with the development of a Minimum Operational Performance Standard providing the means of compliance to cover the requirements provided by SARPs expected to be developed for WAIC systems.

4	REFERERNCES

- Report ITU-R M.2283 “Technical characteristics and spectrum requirements of Wireless Avionics Intra-Communications systems to support their safe operation”, approved Dec. 2013

- Recommendation ITU-R M.2059 “Operational and technical characteristics and protection criteria of radio altimeters utilizing the band 4 200-4 400 MHz”, approved Feb. 2014

- Report ITU-R M.2318 “Technical characteristics and protection criteria for Wireless Avionics Intra-Communications systems”, approved Nov. 2014

- Report ITU-R M.2319 “Compatibility analysis between wireless avionics intra-communication systems and systems in the existing services in the frequency band 4 200-4 400 MHz”, approved Nov. 2014

- Recommendation ITU-R M.2067 “Technical characteristics and protection criteria for Wireless Avionics Intra-Communication systems”, approved in Nov. 2014

- Recommendation ITU-R M.[WAIC_CONDITIONS] “Technical conditions for the use of the aeronautical mobile (R) service  in the frequency band 4 200- 4 400 MHz to support wireless avionics 99999intra-communication systems”

- http://www.itu.int/en/ITU-R/conferences/wrc/2015/

- International Telecommunication Union, Radio Regulations, Edition of 2012

- International Civil Aviation Organization, Annex 10 to the Convention of International Civil Aviation “Aeronautical Telecommunications”

- Aerospace Vehicle Systems Institute, http://www.avsi.aero/