FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

THIRD MEETING OF THE WORKING GROUP

Montreal, Canada, 6-14 September 2016

Agenda Item 10: Any other business

Operation of Low Power Programme Making and Special Event wireless Microphones in the Frequency Band 960-1164 MHz

(Presented by John METTROP)

SUMMARY

This paper provides information on the current situation within the UK with respect to the use of low power wireless microphones in the frequency band 960-1164 MHz.

INTRODUCTION

In 2013 the UK radio regulator, Ofcom, initiated a strategic review of the Programme Making & Special Event (PMSE) sector spectrum requirements with the aim of ensuring continued access to sufficient spectrum to deliver an appropriate level of service.  This review addressed both video (wireless cameras, video links etc) and audio (wireless microphones, in ear monitors etc) requirements and took into account the UK decision, in line with European strategies and decisions, to make the 700 MHz frequency band available for mobile data.  Therefore, alternative spectrum would be required for the PMSE systems currently accommodated in the 700MHz frequency band

Following a review of initial candidate frequency bands for low power (<17dBm/200kHz) PMSE (wireless microphones and in-ear monitors), two frequency bands,960-1164 MHz & 1525-1529 MHz, were identified for further study by Ofcom.  They then approached the Civil Aviation Authority (CAA) to seek their view and advice as what would need to be achieved in order to demonstrate compatibility with existing aeronautical systems.  The CAA, recognising that spectrum is a scarce resource with an evolving environment which needs to be carefully managed in a way that ensures the continued viability/safety of the aviation sector in the long term but also serves the wider spectrum community agreed to consider the proposal

DISCUSSION

- 2.1	In early 2015, working with CAA, Ofcom let a study contract with JCSys to test the compatibility of low power PMSE devices (using CW as an acceptable representation of an analogue PMSE signal) with existing CNS systems operating in the frequency band 960-1164 MHz. The testing was based on what, in the opinion of the the project team, was the worst case scenario e.g. tested against agreed minimum performance criteria (e.g. 70% reply efficiency) in the presence of the maximum JTIDS pulse density permitted under the UK JTIDS frequency clearance agreement. The testing used the same set of aeronautical equipment that was used when establishing the UK JTIDS frequency clearance agreement.

- DME Ground Beacons

  - Fernau 2020

  - Thales 415

  - Fernau 1117

- DME Interrogators

  - Bendix/King KN64

  - Collins 860E-3

  - King KDM 705

- SSR

  - Trig TT21

  - Quadrant Mode S

- 2.2	The final test report was delivered in September 2015 that provided the following results:

- DME Beacon Selectivity

| Offset [MHz] | PMSE Power [dBm] at which System Fails |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fernau 1117 |  | Fernau 202 |  |  | Thales 415 |  |  |
|  | X MODE | Y MODE | X MODE |  | Y MODE | X MODE |  | Y MODE |
| -8 |  | -45 |  |  |  |  |  |  |
| -7 |  | -46 |  |  |  |  |  |  |
| -6 | -52 | -46 |  |  |  |  |  |  |
| -5 | -64 | -55 | -56 |  | -50 | -35 |  | -40 |
| -4 | -66 | -70 | -59 |  | -52 | -45 |  | -55 |
| -3 | -70 | -75 | -70 |  | -69 | -58 |  | -70 |
| -2 | -79 | -87 | -89 |  | -82 | -70 |  | -80 |
| -1 | -89 | -97 | -100 |  | -102 | -88 |  | -95 |
| 0 | -106 | -107 | -111 |  | -111 | -108 |  | -115 |
| 1 | -89 | -97 | -100 |  | -100 | -92 |  | -91 |
| 2 | -79 | -89 | -87 |  | -89 | -77 |  | -80 |
| 3 | -67 | -81 | -74 |  | -82 | -71 |  | -65 |
| 4 | -57 | -77 | -59 |  | -74 | -60 |  | -50 |
| 5 | -47 | -74 | -41 |  | -63 | -45 |  | -40 |
| 6 | -39 | -68 |  |  |  |  |  |  |
| 7 |  | -55 |  |  |  |  |  |  |
| 8 |  | -50 |  |  |  |  |  |  |

- DME Interrogator Selectivity

| Offset [MHz] | PMSE Power [dBm] at which System Fails |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KN64 |  | Collins 860E-3 |  | KDM705A |  |
|  | X MODE | Y MODE | X MODE | Y MODE | X MODE | Y MODE |
| -6 | -32 |  |  |  |  |  |
| -5 | -20 | -35 | -20 | -25 | -20 | -20 |
| -4 | -29 | -41 | -23 | -21 | -20 | -20 |
| -3 | -38 | -48 | -25 | -21 | -20 | -20 |
| -2 | -47 | -58 | -25 | -22 | -20 | -20 |
| -1 | -61 | -75 | -35 | -36 | -25 | -27 |
| 0 | -85 | -95 | -95 | -98 | -87 | -90 |
| 1 | -61 | -69 | -25 | -40 | -23 | -23 |
| 2 | -46 | -55 | -24 | -28 | -20 | -20 |
| 3 | -37 | -48 | -20 | -27 | -20 | -20 |
| 4 | -29 | -40 | -20 | -25 | -20 | -20 |
| 5 | -20 | -36 | -20 | -20 | -20 | -20 |
| 6 |  | -29 |  |  |  |  |

- SSR 1090 MHz Selectivity

| Offset [MHz] | PMSE Power [dBm] at which SMR falls below 90% at 1090 MHz |  |
| --- | --- | --- |
|  | Mode S | Mode A/C |
| -9.0 | -69.4 |  |
| -8.0 | -74.0 |  |
| -7.0 | -78.7 | -75.3 |
| -6.0 | -82.7 | -80.0 |
| -5.0 | -85.6 | -86.8 |
| -4.0 | -87.7 |  |
| -2.0 | -91.0 |  |
| 0.0 | -88.6 | -85.9 |
| 4.0 | -84.7 | -85.9 |
| 5.0 | -78.0 | -83.9 |
| 6.0 | -73.4 | -73.7 |
| 7.0 | -68.6 | -73.0 |
| 8.0 | -63.8 |  |
| 9.0 | -61.8 |  |
| 10.0 | -57.2 |  |

- SSR 1030 MHz Selectivity

| Offset [MHz] | PMSE Power [dBm] at which SMR falls below 90% at 1030 MHz |  |
| --- | --- | --- |
|  | Mode S | Mode A/C |
| -10.0 | -54.9 |  |
| -9.5 | -58.7 |  |
| -9.0 | -62.0 |  |
| -8.5 | -64.8 |  |
| -8.0 | -67.0 | -57.9 |
| -7.5 | -68.9 |  |
| -7.0 | -70.3 | -61.4 |
| -6.5 | -70.8 |  |
| -6.0 | -71.8 | -63.9 |
| -5.5 | -72.8 |  |
| -5.0 | -73 | -65.5 |
| -4.5 | -74.7 |  |
| -4.0 | -75.9 | -68.8 |
| -3.5 | -77.2 |  |
| -3.0 | -77.6 | -68.9 |
| -2.5 | -78.2 |  |
| -2.0 | -79.1 | -69.4 |
| -1.5 | -79.7 |  |
| -1.0 | -79.0 | -67.6 |
| -0.5 | -77.5 |  |
| 0.0 | -76.5 | -78.9 |
| 1.0 |  | -78.6 |
| 2.0 |  | -79.2 |
| 2.5 | -76.7 |  |
| 3.0 |  | -77.9 |
| 4.0 |  | -76.3 |

- 2.3	Based on these results a set of spectrum management rules were developed for the deployment of low power PMSE devices in the frequency band 960-1164 MHz that were based on the following assumptions:

- That sufficient testing had been carried out to develop planning criteria that will ensure the protection of DME/TACAN from PMSE

- That the planning criteria will ensure the protection of DME/TACAN from PMSE would be based on the worst case selectivity envelope

- That there has been insufficient testing to develop detailed planning criteria that will ensure the protection of SSR/TCAS/ADS-B from PMSE

- That in the absence of sufficient PMSE vs SSR/TCAS/ADS-B testing a ±15 MHz guard band would be used around 1030 & 1090 MHz

- That a guard band be implemented to protect RNSS operation above 1164 MHz

- That the operational coverage areas would be based on the greater of:

  - the co-ordinated designated operational coverage

  - The ARINC 424 Navigation System Data Base Standard

- Clutter but not building loss is applied from PMSE into DME on the terrestrial to terrestrial path only. Clutter loss is, however, applied only at the PMSE end of this path on the assumption that the DME transponder end is generally free of clutter

- That interference would be allowed in 1% of occasions

- That PMSE equipment would have to be licenced. PMSE licenses are time, location and frequency specific.

- 2.4	In October 2015 Ofcom published a consultation document addressing the possible options for accommodating low power PMSE in the frequency bands 960-1164 & 1525-1529 MHz.  The content of that document with respect to the frequency band 960-1164 MHz was based on the testing undertaken by JCsys and spectrum management rules developed in conjunction with the CAA.

- 2.5	Based on comments received in response to the consultation, the project team commissioned additional work to look at the compatibility of low power PMSE devices with scanning DME.  The results of these studies showed that there is no evidence that scanning DME interrogators are more susceptible to interference than the DME equipment used in the original study. Consequently, it is concluded that the measures taken within the spectrum management rules established between Ofcom and the CAA for planning PMSE vs DME/TACAN are sufficient for scanning DME.

- 2.6	In March 2016 Ofcom published its statement (See Annex 1) which included the agreed spectrum management rules (See Annex 2 of Annex 1) with respect to the consultation mentioned in paragraph 2.4 in which they declared their intention to allow low power PMSE devices to operate, on a licenced basis, in the frequency band 960-1164 MHz subject to compliance with the agreed spectrum management rules.

- 2.7	On a risk based approach the CAA are content that compliance with the spectrum management rules will ensure, to an acceptable level of risk, that low powered PMSE will not cause interference to existing aeronautical systems.  The following issues still need to be addressed and/or agreed:

- Production of a safety case that meets the relevant requirements

- How to address the introduction of new aeronautical systems (e.g. LDACS, UAT) into the frequency band 960-1164 MHz

- Provision of suitable supporting evidence for any proposed reduction in the guard bands around 1030 & 1090 MHz

- Validation or otherwise that CW is a suitable representation of a digital PMSE signal when considering interference to aeronautical systems operating in the frequency band 960 – 1164 MHz

ACTION BY THE MEETING

The meeting is invited to:

note and review the contents of this information paper

- — END —

- Annex 1

- Spectrum Management Rules

Test Report for the Coexistence of PMSE with Aeronautical Services in the Band 960-1164 MHz (http://stakeholders.ofcom.org.uk/binaries/consultations/new-spectrum-audio-PMSE/annexes/annex6.pdf)

Explained further on p.11 and p.43 of Ofcom’s statement on New Spectrum for Audio PMSE: http://stakeholders.ofcom.org.uk/binaries/consultations/new-spectrum-audio-PMSE/statement/statement.pdf

New Spectrum for Audio PMSE (http://stakeholders.ofcom.org.uk/binaries/consultations/new-spectrum-audio-PMSE/summary/new-spectrum-audio-pmse.pdf)