FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

Fourth Working Group meeting

Bangkok, Thailand, 29 March – 7 April 2017

Agenda Item 3:	Updates to the Frequency Spectrum Policy Statements (Chapter 7)

Aviation HF usage and proposed edits to HF descriptions in Chapter 7

(Presented by Andrew Roy, ASRI)

SUMMARY

A proposal for edits to the HF section in Chapter 7 are made, with additional information on current HF considerations to provide context for the edits.

INTRODUCTION

In the FSMP’s Sept 2016 meeting, it was noted that the ICAO Spectrum Management Handbook (Doc 9718) was out of date for the description of HF in Chapter 7.  The existing descriptions of HF did not accurately reflect current usage, and several comments were either not explained or out of date.  An action was taken to provide a relevant rewrite to the text.

DISCUSSION

In reviewing HF usage, several points were made that are relevant to HF usage and planning.  These provide some context to the edits being made:

- HF message traffic has been mostly static over the last few years, and there are no indications of it beginning to decrease.  Although L-band SATCOM has provided a welcome capability to certain aircraft and airlines, many airlines and ANSPs are content with the capability and availability that HF voice (and particularly HF Datalink (HFDL)) provides without spending additional money on SATCOM avionics and support.  It is likely that aircraft currently without SATCOM will not include it in the lifetime of the airframe.

- Several ANSPs and airlines have also noted that the HF networks are the key backbone for fall back from SATCOM outages or avionics failures.  SATCOM outage mitigation directly impacts capacity planning, as suitable contingency bandwidth needs to be available dependant on regional activity.  Therefore, although traffic is not increasing, some ANSPs are continuing to deploy additional ground sites and frequencies to ensure they can meet operational requirements.  Interestingly, this has put a new focus on SELCAL, as ANSPs must immediately reacquire an aircraft after loss of SATCOM.  Using SELCAL on HF voice is the prime means of raising the crew in such scenarios, and the current duplicate SELCAL code issue is directly affecting the performance of ATC message relay in such cases.

- There are several early projects being considered to enhance HF datalink performance with new waveforms and technology.  ASRI has seen one concept being developed of a wider HF bandwidth signal allowing for both HF data and digital voice.  This would provide equivalent bandwidth to SATCOM systems, allowing for an aircraft CMU to continue the same service no matter which transmission medium is being used in oceanic or remote areas worldwide.  Should such projects gain momentum, a review of ITU-R Radio Regulations Appendix 27 may need to be considered.

- In identifying additional regional frequencies for new HF ground stations, it is becoming apparent that the ITU-R MIFR is not current for all aeronautical HF assignments.  There are currently multiple legacy assignments still registered that are limiting potential frequency use within the existing aeronautical HF allocations.  If new HF technology does not drive a review of aeronautical HF usage, the need to meet existing HF capacity requirements may require such activity in the future.

A revised excerpt from Doc 9718 is embedded below for review.  The aim of rewriting the HF chapter was to simplify the text where possible, eliminating unnecessary or inaccurate information that may vary over time.  It became clear during the editing that certain sections required an actual rewrite.  Therefore, whole paragraphs have been deleted and re-written where appropriate for ease of understanding.  Specific points that were included:

- Update how HF is currently being used for voice and HF Datalink (HFDL).

- Clarify need for frequency changes throughout the day as the ionosphere varies.

- Relate HF usage to L-band SATCOM usage.

- Clarify HF spectrum planning requirements.

- Made minimal changes to Appendix 27 references as this section of the radio regulations has changed little in many years.

- Added note that administrations ensure MIFR is appropriately updated for HF assignments.

- Removed statistics that can vary between months.

- Clarified or deleted references to other committees/documents that were out of date.

- Abbreviations have been highlighted to ensure they are appropriately tracked in the glossary and avoid repeated explanations of them in the rest of the document.

CONCLUSION

HF continues to be a core element of aviation communications, and although some expect the system to decline in favour of SATCOM, this is clearly not happening in the short-to-medium term.  HF voice and data are still a core capability for many airlines, and will remain that way unless there is a significant change at an international level.  The edits provided to Chapter 7 of ICAO Doc 9718 should enable a more accurate description of HF services pending any major developments of HF in the future.

ACTION BY THE MEETING

  - The meeting is invited to:

- note the contents of this working paper;

- review the embedded document

- use text to make suitable recommendations to replace the relevant text in Chapter 7 of ICAO Doc 9718

— END —

Doc 9718, Chapter 7 contains a list of each frequency band and a description of its uses, together with the approved ICAO policy statements for the civil aviation spectrum requirements in that band. Also included are relevant provisions (Regulations), Resolutions and Recommendations which are contained in the ITU Radio Regulations