|  | International Civil Aviation Organization REPORT | 16 February 2024. |
| --- | --- | --- |

FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP)

EIGHTEENTH MEETING OF THE WORKING GROUP OF FSMP

(FSMP-WG/18)

Montréal, Canada

06-16 February 2024

REPORT

- Agenda Item 1 - Opening and Working Arrangements

  - The 18th working group meeting of the Frequency Spectrum Management Panel (FSMP-WG/18) was held from 6 February to 16 February 2024. This meeting was held as a partial hybrid meeting (in person but broadcasting for the remote listeners) and the physical parts of the meeting was held in Montréal, Canada.

  - Mr. Andrew Roy, the chairperson of the FSMP and the rapporteur of FSMP-WG/18, opened the meeting and expressed appreciations to participants for their active contributions to the FSMP-WG/18 meeting. Ms. Mie Utsunomiya, CNS Officer at HQ ICAO, acted as the Secretary of this meeting, assisted by Mr. Isaiah Tefutor, CNS Officer at HQ ICAO.

  - Mr. Roy welcomed the group and provided introductory remarks and meeting information. The meeting was held in English. After the opening of the meeting the agenda was approved by the group. The agenda is contained in Appendix A.

  - The list of papers submitted for consideration by FSMP-WG/18 is contained in Appendix B.  It was noted that two information papers had been received after the submission deadline for the meeting.  These had been sent to the membership before the meeting began, but had not been formally included in the meeting agenda until a discussion on the first day.  During this discussion, it was agreed that as the papers were for information only, they would be formally accepted into the meeting.  They were added to the meeting agenda as IP10 and IP11.  The list of participants is in Appendix C.

  - The material in this report is organized by meeting agenda item number and does not necessarily reflect the order of discussions.  Actions captured during discussions are shown in Appendix D, together with the status of prior-meeting(s) actions.

- Agenda Item 2 - Outcome of World Radiocommunication Conference (2023) (WRC-23)

  - An informal presentation was provided by the Secretariat on the outcomes of WRC-23 relevant to aviation.  There was a discussion on several items given the results and implications for aviation.  The meeting appreciated the input from the Secretariat.

  - IP03 brought to the attention of the FSMP a development at WRC-23 with respect to the need for studies between aeronautical systems to be carried out in the International Telecommunication Union (ITU).  It highlighted discussions at the ITU that ICAO should account for in future WRC agenda items.  In the discussions, it was noted that it seemed more of a political action at the ITU, rather than a future technical concern.  However given the nature of WRC discussions, it was agreed that it may happen again, and aviation should be prepared for such an approach.

  - WP18 raised a concern with the new text approved at WRC-23 for Resolution 676 (WRC-23) that stated administrations have the right to deny access to radionavigation-satellite service (RNSS), for security or defence purposes.  It stated that the text was a late addition to the resolution at WRC-23 that had not been anticipated and created concerns that it may be used as justification for additional jamming and spoofing by both defence and national security entities which is not in line with Article 15.1.  In the discussion, there was general agreement about the concerns raised.  Although noting that ITU Constitution Article 48 indicates that members states retain their entire freedom with regards to military installations, recognizing that measures to prevent harmful interference must be observed as far as possible.

- Agenda Item 3 - Development of ICAO position for World Radiocommunication Conference (2027) (WRC-27) – FSMP.002.02

  - Discussion of considerations for WRC-27

3.1.1	WP06 reviewed the items that have been included in the agenda for WRC-27 and provisionally for WRC-31, identifying at a high level those that are direct threats to, and those that are potential threats to, spectrum used by aviation.  It then discussed the implications for ITU meeting attendance by the aviation industry as a whole.  In a discussion on sharing responsibilities to preserve limited resources in some areas for attendance, it was noted that it would be difficult to combine national positions given various interests.  However, the meeting generally agreed with the need for additional coordination amongst the aviation industry.  The meeting also agreed that FSMP should encourage attendance by aviation experts at the relevant ITU and regional activities.  Finally, it was suggested that a future standing agenda item to the FSMP be created to identify and timetable relevant spectrum meetings to further assist these efforts.

  - Material for ICAO Position for WRC-27

    - Four contributions (WPs 07, 09, 23, & 24) proposed initial text for a draft ICAO position to WRC-27. Flimsy01 was developed that combined all proposals and was used by the meeting for the review.  After discussion of multiple topics that included offline consultations, Flimsy01 incorporated these updates in a preliminary text ready for additional contributions and eventual finalization at the next FSMP meeting.  The Chair noted that as well as a general request to provide contributions, multiple highlighted areas were specifically seeking input or resolution.  The current draft of the ICAO position to WRC-27 is included in Appendix E.

- Agenda Item 4 – Development of material for ITU-R studies

  - ITU Engagement

    - WP11 followed up on document FSMP-WG/17 WP17 about the provision of operational and technical parameters for aeronautical system in a manner that ensures ICAO remain in control of the information.  It proposed development of ICAO material that could assist the ITU-R in its studies involving aviation systems.  In the discussion of the paper, it was noted that how the ITU-R would proceed with such data was not necessarily in ICAO’s control but that some industries do use a similar process.  It was strongly supported that ICAO should make every effort to compile the necessary information to then make a decision, with broad agreement that handbook restructuring should drive the collection of data.  Once this was completed, then a decision could be made on how to relate it to the ITU-R.

  - WRC-27 AI 1.7 IMT

    - WP27 proposed a framework for material to be developed for an ICAO Liaison Statement (LS) to the ITU-R, informing studies on WRC-27 Agenda Item 1.7 involving the radio altimeter and WAIC systems.  The proposal was an early draft, recognizing that additional input is needed.  There was extensive discussion about the proposal, including what material may be needed, how it should be presented, and how it would relate to existing ITU-R documentation.  It was agreed to request additional inputs to the document at the next meeting from the FSMP membership (ACTION ITEM 18-01), and that two additional actions to clarify the data also be created:  ACTION ITEM 18-02 requested input from radio altimeter manufacturers on available data and if this might augment or replace existing technical details in the ITU-R documentation.  ACTION ITEM 18-03 was also created to send the operational scenarios proposed to both the ICAO Flt Ops and Aerodrome panels for input and any other considerations back to the next FSMP meeting, July 2024, so they could be forwarded to the ITU-R.

- Agenda Item 5 – Radio Altimeter issues FSMP.006.02

  - Report from correspondence group on radio altimeters (CG-RA)

    - IP09 was provided from the CG-RA and included several national and stakeholder organizations (e.g. IATA) updates that were made both before and during the meeting.  The meeting asked the CG to continue its work collecting updates on relevant activities associate with the radio altimeter, and to communicate with the authors of the IPs to propose amendment to the FSMP-WG/18 IP09. An updated version of IP09 was provided at the end of the meeting to include information gathered at FSMP-WG/18.

    - The meeting also was informed that ICAO Circular 360 which was originally drafted by ICAO MID Regional Office and finalized by the FSMP-WG/16 is now in the final stage of publication and an edited version of the Circular 360 will be published in Q1 2024 on the ICAO-NET.

  - National efforts to implement broadband mobile near 4200-4400 MHz - Mitigation Measures

    - No contributions were received on this topic, though it was noted that the updated IP09 contained multiple country specific updates.

  - National efforts to implement broadband mobile near 4200-4400 MHz - Safety Cases/Compatibility Analyses

    - IP02 presented a report with the initial results of radio altimeter interference path loss (IPL) characteristics of Beechcraft B300 aircraft during the flight. Although RTCA DO-307B/EUROCAE ED-239A specifies IPL evaluation when the aircraft is on the ground, the report obtained IPL characteristics during flight and compare the differences.  Numerous locations inside of the cabin were measured on the assumed operating parameters of the 5G mobile devices and the Wireless Avionics Intra-Communication system.  The results confirmed that the minimum IPL values are always higher during the flight than on the ground.  After some discussions about the performance values and potential extrapolation, the meeting thanked the contributors for the information and noted previous versions of such data had been used in radio altimeter studies.

  - Development of SARPS material for Annex 10, Vol. V

    - WP26 provided some updates on the proposed SARPs framework for radar altimeter RF characteristics based on the RTCA/EUROCAE MOPS work.  It requested feedback form the FSMP on the required data that could be sent to RTCA and EUROCAE.  In the resulting discussion, it was broadly agreed the type of parameters proposed were correct but requested further feedback from FSMP members to finalize the SARPs, which aims to finalize by Sep 2025. An action (ACTION ITEM 18-04) was created to send the comment back to RTCA and EUROCAE to ensure the development of new radio altimeter standards was coordinated between MOPS and SARPS

- Aeronautical Band Planning – FSMP.005.03

  - 108 – 137 MHz band planning

    - WP03 proposed amendments to ICAO Annex 10, Volume V by extending the possibility of use of 8.33 kHz channel spacing beyond 136.475 MHz in the assignment of frequencies.  Noting the currently, Annex 10 specifies that the frequencies 136.500 – 136.975 MHz are not available for assignment to channels of less than 25 kHz width, limiting the use of 8.33 kHz channels. There was a discussion on the implications of such changes for both current 25 kHz voice/data systems and possible VHF SATCOM. It was suggested that additional consideration was needed to fully understand the long-term implications of such a proposal, and therefore contributions on this topic at future meetings were encouraged.

    - WP10 posed several questions on how the WP 5B report "Space-based aeronautical VHF communications in the frequency band 117.975-137 MHz" may be completed now the aeronautical mobile satellite (route) service (AMS(R)S) allocation was approved at WRC-23.  This was pertinent as the WP 5B report provided necessary context in defining the adjacent band interference from 137-138 MHz systems that the new AMS(R)S systems must withstand.  In the discussion it was agreed that a quick completion of the report that did not risk the final values used in the decision made at WRC-23 was the preferred outcome.  However, if significant changes were received or the process was prolonged, then the group would not support that.  It was suggested that concerned individuals from administrations may wish to coordinate offline before WP 5B to ensure a better outcome at the ITU-R.

    - WP15 provided details on the Space-VHF technology, using a representative satellite and a representative constellation to help with the work that is required within the FSMP to update Annex 10 Volume V and Volume II of Doc 9718. In addition, some considerations about operations of the service both from a Voice and Data perspective were discussed.  The meeting appreciated the detailed information, and there were many questions about the technical and operational capabilities.  It was agreed that the paper should be considered by the VHF SATCOM CG (see WP19).

    - WP19 contained a proposed Terms of Reference (TOR) of the Space-Based VHF Correspondence Group that was agreed to at the FSMP-WG/17 meeting.  It included both the defined terms of reference for such work, the expected timeframe of the project, and potential deliverables.  It also incorporated the need of AMS(R)S SARPs and ICAO frequency planning to account for ITU-R RESOLUTION 406 (WRC-23).  After a discussion on proposal, a short drafting activity occurred, and an updated TOR was approved as Flimsy 06 (Appendix F) to establish a CG chaired by Mr. Martín and co-chaired by Mr. Kelly.

  - 960 – 1215 MHz band planning

    - No contributions were received on this topic.

  - 5000 – 5150 MHz band planning

    - IP04 contained an example draft channel plan for Terrestrial C2 Links operating in the 5040-5050 MHz Sub-Band.  Provided only for information purposes, it was based on the RTCA MOPS for UAS operating in the 5030-5091 MHz band (DO-362) and is focused on non-networked operations.  It noted that this was one possible configuration and did not address networked terrestrial or satellite-based operations.  There was a discussion on the implications of such a band plan and how it would be implemented.  The meeting appreciated the information.

    - IP05 provided a presentation that discussed band sharing options between networked and non-networked terrestrial as well as satellite based C2 Link systems in the 5030-5091 MHz band.  After some clarifications to the concepts discussed, the meeting thanked the presenter for the material.

- Agenda Item 7 – Interference from Non-Aeronautical Sources

  - Mobile service communications adjacent to 1518 MHz satellite communications frequency band (see action item 14-04)

    - WP14 and WP21 provided suggestions to progress the action item 14-04 on the development of a note providing information on ensuring compatibility between IMT operated below 1518 MHz and aeronautical MSS services above 1525 MHz.  After an offline drafting group, Flimsy 03 (Appendix G) was produced that combined the inputs and produced an agreed text between those involved.  An action (ACTION ITEM 18-05) was created to seek any comments on the note at the next FSMP meeting, where it is intended to finalize the material.

  - LED lighting systems

    - No contributions were received on this topic.

  - Other reported interference

    - IP07 briefly summarized the current status and work of FM broadcast digital emissions testing against VHF navigational aids in the United States around the 108 MHz allocation boundary.  IP08 also briefed on the same topic from the Canadian perspective, noting that although the compatibility of FM modulated analog broadcast signals in this band with aeronautical systems above 108 MHz is already well specified in Recommendations ITU-R SM.1009-1 and SM.1140, it should be reviewed for the deployment of new terrestrial digital sound broadcasting systems.  In the discussion, there were multiple concerns raised the apparent non-conformity with the Radio Regulations (RR) by the necessary bandwidth not remaining within the allocated frequency band.

- Agenda Item 8 – Revision of ICAO Frequency Spectrum Handbook (Doc 9718)

  - Restructure of handbook

    - Five contributions (WP01, WP17, WP22, WP28, & WP29) proposed material and concepts to be included into the restructured handbook.  The meeting was encouraged to provide further contributions on the material at the FSMP-WG/19 meeting.  Additionally, the CG established at FSMP-WG/05 was reinitiated to further develop the material and continue development of the current Volume I for the next FSMP meeting. Mr. Kelly was nominated as rapporteur of this CG.

  - Update of Volume I

    - Five contributions (WP04, WP13, WP16, WP25, and IP06) proposed updates to the current Volume 1 of Doc 9718.  Additionally, material from WP28 and WP29 was also deemed relevant to the current publication while developing future material from Agenda Item 8.1.  The group held a drafting group, creating Flimsy 02R1 (refer to Appendix H) with a consolidated proposal to be discussed at FSMP-WG/19.  It is intended for this to be approved at that meeting alongside the initial ICAO WRC-27 position.  To further support the work, and also progress any discussion on the ICAO policy statements for certain bands, the CG established at FSMP-WG/05 was reinitiated to develop the material in time for FSMP-WG19. Mr. Kelly was nominated as rapporteur of this CG.

    - In discussions of IP06, there was a debate over regional section variations of potential Detect and Avoid (DAA) usage.  It became clear that additional information was needed from the Remotely Piloted Aircraft Systems (RPAS) panel to understand their spectrum requirements for RPAS, both for implementation in the handbook, but also for implications for Annex 10 Vol. V.  The meeting agreed that there should be more engagement on the DAA system aspects to fully understand the spectrum implications and advise the RPAS panel on the resulting actions.  An action item (ACTION ITEM 18-06) was created for the Secretariat to request regular updates on the DAA system aspects, specifically frequency arrangements, at future FSMP meetings.  This included any feedback from the RPAS panel on the information provided in IP06, including suggestions to the FSMP for potential updates and the basis for any proposed inputs.  A second action (ACTION ITEM 18-07) was created to seek input contributions at the next FSMP meeting on necessary changes needed (if any) to Annex 10, Vol. V to reflect the spectrum updates made as part of the RPAS Panel’s updates to other Annex 10 Volumes.  This would then be sent to the RPAS panel for comments and feedback.

  - Update of Volume II

    - WP02 presented modifications from the Navigation Systems Panel (NSP) Joint Working Groups to the ICAO Spectrum Handbook, Volume II, Chapter 6 on Ground-based augmentation system (GBAS)/VHF data broadcast (VDB) frequency assignment planning. The proposed modifications included clarifications and corrections of the DOC for the GBAS/VDB, and optional use of the airborne correction factor in frequency assignment planning. The draft of the changes was provided in Appendix X.  During the presentation, questions and comments were raised that will be processed and replied to by correspondence by the WP02 NSP authors' coordinators.  It was agreed that an action (ACTION ITEM 18-08) be created for the FSMP membership to provide comments in a written form by e-mail to the NSP sub working group (SWG) Rapporteur and to the NSP Secretary when nominated on the proposed changes before 30th April 2024. That FSMP-WG18 output will then be provided in May 2024 NSP JWGs/12, by the ICAO NSP SWG rapporteur.

    - For July 2024 ICAO FSMP-WG/19, the NSP SWG rapporteur and the NSP Secretary will then submit a paper to provide FSMP about feedback from the NSP, and potentially if requested by the NSP, to ask the FSMP to review and approve the final proposed changes to ICAO Spectrum Handbook, Volume II, Chapter 6 on GBAS/VDB frequency assignment planning.

- Agenda Item 9 – Any other business

  - WP08 provided an update to FSMP-WG/17 WP06 and the Excel file containing useful RF conversions and formulas. FSMP-WG/17 WP06 had received the group support, however a relevant comment was raised concerning the security and the responsibility.  In a short discussion, the meeting agreed the usefulness of the tool for guidance or an additional check of calculations.  The meeting thanked the author for the material.

  - WP12 discussed what is a safety service/system/link and how to define those concepts in a manner that is clear and makes understandable when aviation can use commercial spectrum to non-aeronautical spectrum experts.  The meeting appreciated the start of such a discussion that provided some considerations for ICAO policy.  The proposed safety classification of certain aviation systems resulted in a lot of discussion concerning the understanding of aviation system usage, and it was clear that no agreement could be reached on such a categorization at this time.  It was agreed that this topic required additional consideration and development, and an action was created to seek further contributions (ACTION ITEM 18-09)

  - WP20 analyzed the impact of new interference from L-band Low Earth Orbit Positioning, Navigation and Timing (LEO PNT) constellations on aeronautical GNSS receivers.  The paper discussed the need for a coordination process in each administration for the introduction of a new RNSS system, and if the civil aviation authority is part of this process.  It also sought more information on the technical characteristics of these new LEO PNT systems and feedback on the assumptions considered in the paper.  The meeting agreed the need to verify coordination process in each administration for the introduction of a new RNSS systems, and suggested that the author attempt direct follow up with radio regulators and relevant parties to further clarify.

  - IP01 provided a copy of the presentation made to the ANC in November 2023 that gave a background brief on the what and how of international spectrum management.  It was provided to the meeting and welcomed others to use the material as needed.  The meeting appreciated the input.

  - IP10 provided a copy of a new draft Electronic Communications Committee (ECC) Recommendation and Report on “Receiver Resilience to Transmission on Adjacent Frequency Ranges” that is being sent for approval within Europe.  While it was noted that the material would not be directly applicable to aircraft equipment (though ground based aviation systems would be required to meet it), it was suggested the aviation industry should be aware of this material and consider it in the developments of future aviation systems.  It may also have the potential to be adopted by other international organizations, and then could eventually become applicable to all aviation systems.

  - IP11 was a presentation from Boeing and Airbus summarizing a recent concept paper on Hyper-connectivity Air Traffic Management.  In the extensive discussion covering how such a proposal would impact aviation’s spectrum usage, many concerns were expressed about using non-safety systems for aviation safety functions in such a widespread manner.  The meeting asked the authors to take on board all the comments and bring back to the FSMP-WG/19 additional clarifications and follow up.

  - Lastly, the Secretariat provided details of the early responses to the State Letter on WAIC SARPs, with several comments received that had proposed the applicability date be postponed until the radio altimeter SARPs had been approved.  In discussing this item, the FSMP membership expressed strong concerns about such proposals, noting that the FSMP had carefully reviewed and validated the technical compatibility between WAIC and radio altimeters.  In the meeting, comments were made by those participating in the RTCA/EUROCAE process to develop radio altimeter MOPS that WAIC SARPs are already fully taken into account in the new radio altimeter design and not would affect radio altimeter performance.  Additionally, comments were expressed by the FSMP membership that delaying the WAIC SARPS further would create a risk that the future external RF environment changes again, requiring additional WAIC redesigns and further SARPs development work.

- Date of next meeting and administrative issues

  - FSMP-WG/19 is scheduled for 15 – 26 July, 2024 at the ICAO HQ in Montréal, Canada. For planning purposes FSMP-WG/20 is tentatively planned for February/March 2025 in the Asia Pacific region.  It was agreed that the trial of the broadcast functionality would continue at the next meeting, with a nominated day being allocated to allow only IPs to be briefed remoted by those wishing to use such a feature.

  - In discussing future plans, a tentative timeline of Q3 2025 was agreed for the radio altimeter and VHF SATCOM Annex 10 Vol V amendments to be completed and ready for a full FSMP panel consideration.  It was noted that both items may vary on their progress, and therefore the timeline will be reassessed in future meetings as the work develops.  The restructuring of the handbook had a tentative target date of Sep 2027 for a first draft, though more detail on the work is expected at later meetings.  Additionally, it was agreed that the last date for proposals for WRC-31 agenda items to be considered by the FSMP was Sep 2026, though it was recommended that any such proposals be submitted to earlier meetings to allow for full consideration by the group.

  - The meeting agreed that papers for FSMP-WG/19 are due 23:59 Montréal Time (EST), Monday, 8 July 2024, and any papers requiring restricted access should be identified when being submitted.   In exceptional circumstances, a framework detailing at minimum the paper summary and introduction will be provided by the due date, with the full paper provided no later than one workday before the start of the meeting. Any papers received after this deadline will not be accepted unless agreed by the meeting on the first day. No papers received after the first day of the meeting will be accepted.

APPENDICES

Appendix A – Agenda

Appendix B – List of Working Papers, Information Papers and Flimsies

Appendix C – List of Participants

Appendix D – Action Item List

Appendix E – Draft ICAO Position for WRC-27

Appendix F – Space-Based VHF Correspondence Group Terms of Reference

Appendix G – Draft Information Note (Measures to address potential blocking of MES operating in frequency bands 1 525-1 559 MHz on the ground at airports from terrestrial IMT systems planned in the frequency band below 1518 MHz) Clean version

Appendix H – Draft Doc.9718.Volume I. (Clean version)

APPENDIX A

INTERNATIONAL CIVIL AVIATION ORGANIZATION

EIGHTEENTH working Group Meeting of

the Frequency Spectrum Management Panel (FSMP-WG/18)

(Montréal, Canada, 06-16 February 2024)

Agenda Item 1              Opening and Working Arrangements

- Action Item Review

- Status of tasks identified on Job Cards

Agenda Item 2              Outcome of WRC-23

Agenda Item 3              Development of ICAO position for WRC-27 – FSMP.002.02

Agenda Item 4              Development of Material for ITU-R Studies – FSMP.003.02

Agenda Item 5              Radio Altimeter issues – FSMP.006.02

- Report from correspondence group on radio altimeters (CG-RA)

- National efforts to implement broadband mobile near 4200-4400 MHz

  - Mitigation Measures

  - Safety Cases/Compatibility Analyses

- Development of Radar Altimeter SARPS material for Annex 10, Vol. V

Agenda Item 6              Aeronautical Band Planning – FSMP.005.03

- 108 – 137 MHz

- 960 – 1215 MHz

- 5000 – 5150 MHz

Agenda Item 7		Interference from Non-Aeronautical Sources – FSMP.004.03

- Mobile service communications adjacent to 1518 MHz satellite communications frequency band (see action item 14-04)

- LED lighting systems

- Other reported interference

Agenda Item 8              ICAO Frequency Spectrum Handbook (Doc 9718)

– FSMP.001.02 & FSMP.005.03

- Restructure of handbook

- Update of Volume I

- Update of Volume II

Agenda Item 9              Any Other Business

APPENDIX B

List of Papers

“*” papers available on closed website(s)

List of Working Papers

| Paper # |  | Source |  | Topic | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| 1 | * |  | M. Kelly | Report on Handbook Drafting Group.docx | 8 a) |
| 2 |  |  | C. Macabiau | NSP input on GBAS frequency assignment planning | 8 c) |
| 3 |  |  | C. Fleury | Extending the use of 8.33 channels beyond 136.475 MHz | 6 a) |
| 4 | * |  | K. Eltanany | IATA’s proposal for additional text to Volume I Doc 9718 | 8 b) |
| 5 |  |  | This WP# was not used. | (The WP originally uploaded was requested for the withdrawal from FSMP-WG/18) | Not used |
| 6 |  |  | J. Mettrop | WRC27 and WRC31 Meeting Coverage | 3 |
| 7 |  |  | J. Mettrop | Draft ICAO Position to WRC-27_Jm | 3 |
| 8 |  |  | C. Fleury | Continued from proposal to facilitate RF conversions and compatibility analysis calculations | 9 |
| 9 |  |  | C. Fleury | Draft ICAO Position to WRC-27 France | 3 |
| 10 |  |  | C. Fleury | Draft new report ITU-R M. Space-VHF France | 6 a) |
| 11 |  |  | J. Mettrop | Provision of Information to the ITU | 4 |
| 12 |  |  | J. Mettrop | Safety Service | 9 |
| 13 |  |  | J. Mettrop | Doc.9718 Vol. I update | 8 b) |
| 14 |  |  | C. Fleury | Proposal to update and finalize the MSS note | 7 a) |
| 15 |  |  | C. Benson | Space VHF | 6 a) |
| 16 |  |  | K. Arakawa | Handbook update related to FOD detection system | 8 b) |
| 17 |  |  | J. Mettrop | System characteristics.doc | 8 a) |
| 18 |  |  | J. Mettrop | Denial of Access to GNSS | 2 |
| 19 |  |  | M. Garcia and M. Kelly | Space-Based VHF CG ToR | 6 a) |
| 20 |  |  | G. Novella | Impact of LEO PNT Systems on Radio Navigation Systems Used for Civil Aviation Purpose | 9 |
| 21 |  |  | K. Masrani | L-band satcom protection | 7 a) |
| 22 | * |  | J. Mettrop | Probabilistic Approach to Link Budgets | 8 a) |
| 23 |  |  | C. Tourigny | Draft ICAO Position to WRC-27_US | 3 |
| 24 |  |  | J. Cramer | ICCAIA WRC-27 Working Document for FSMP-WG18 | 3 |
| 25 |  |  | D. C. Ladson | Iridium System Characteristics | 8 b) |
| 26 | * |  | A. Roy | RA SARPs Framework Proposal | 5 c) |
| 27 | * |  | A. Roy | RA and WAIC Parameters for ITU-R | 4 |
| 28 |  |  | A. Ferr | Material for revision of Doc 9718 - Volume A | 8 a) |
| 29 |  |  | A. Ferr, S.B. Ram and D. Lopez | Material for revision of Doc 9817-Volume B | 8 a) |

List of Information Papers, Presentations and Flimsies

| Paper # |  | Source | Topic | Agenda Item |
| --- | --- | --- | --- | --- |
| 1 |  | J. Mettrop | ANC presentation | 9 |
| 2 |  | S. Futatsumori | Radio altimeter interference path loss evaluations of small-sized fixed wing aircraft during flight | 5 b) |
| 3 |  | J. Mettrop | Studies in the ITU.doc | 2 |
| 4 | * | D. Nellis | Draft Channel plan for the 5040-5050 MHz sub-band | 6 c) |
| 5 | * | D. Nellis | C-Band Spectrum Compatibility | 6 c) |
| 6 |  | D. Nellis | Proposed updates to Doc.9718 Vol 1C-Band Spectrum Compatibility | 8 b) |
| 7 | * | A. Roy | FM Digital Broadcasting in US | 7 c) |
| 8 | * | A. Ferr | FM Digital Sound Broadcasting - VHF Aeronautical Band | 7 c) |
| 9 |  | C. Fleury | CG-RA Report Feb 2024 | 5 a) |
| 10 |  | J. Mettrop | Receiver Standards | 9 |
| 11 |  | J. Cramer and L. Azoulai | Hyper Connected ATM Concept overview | 9 |
| FLIMSIES |  |  |  |  |
| Flimsy # |  | Source | Topic | Agenda Item |
| 1 |  |  | Initial Consolidation of Contributions for ICAO WRC-27 Position | 3 |
| 2 |  |  | Draft Doc.9718.VI. 3rd.Ed clean version /track change version | 8 b) |
| 3 |  |  | Update L band MSS note clean version/track change version | 7 a) |
| 4 |  |  | Band L Inmarsat ICAO position | 3 |
| 5 |  |  | FVSG-job card | 6 a) |
| 6 |  |  | Space Based VHF CG TOR | 6 a) |

APPENDIX C

EIGTEENTH WORKING GROUP MEETING OF

THE FREQUENCY SPECTRUM MANAGEMENT PANEL (FSMP-WG/18) (ICAO HQ, Montréal, Canada  – 06 February to 16 February 2024)

|  | States |  |  |
| --- | --- | --- | --- |
|  | Nominated By | Name | E-mail address |  |
| M | Australia | Matthew Kelly | matthew.kelly@airservicesaustralia.com |  |
| A | Australia | Craig Benson | craig.benson@skykraft.com.au |  |
| M | Brazil | Wallace Gutemberg Luz | gutembergwgml@decea.mil.br |  |
| A | Brazil | Vahe Antoine Yaghdjian | vahevay@decea.gov.br |  |
| M | Canada | Seyed Rastaghi | Seyed.Rastaghi@navcanada.ca |  |
| A | Canada | Siamak Bonyadi Ram | Siamak.BonyadiRam@navcanada.ca |  |
| A | Canada | Abed FERR | Abed.Ferr@navcanada.ca |  |
| A | Canada | Damian LOPEZ | Damian.Lopez@navcanada.ca |  |
| A | Canada | Ricardo Hisano | ricardo.hisano@tc.gc.ca |  |
| A | China | Chi Yun Joseph Ho | jcyho@cad.gov.hk |  |
| M | France | Christian Fleury | christian.fleury@aviation-civile.gouv.fr |  |
| A | France | Christophe Macabiau | macabiau@recherche.enac.fr |  |
| A | France | Jérome Andre | Jerome.ANDRE@anfr.fr |  |
| A | France | Guillaume Novella | guillaume.novella@aviation-civile.gouv.fr |  |
| A | Germany | Sergio Bovelli | sergio.bovelli@airbus.com |  |
| M | Japan | Katsuyuki Arakawa | arakawa-k24fe@mlit.go.jp |  |
| A | Japan | Toshio Nomi | nomi.toshio@jransa.or.jp |  |
| A | Japan | Shunichi Futatsumori | futatsumori@mpat.go.jp |  |
| O | Singapore | Henry Foo | Henry_FOO@caas.gov.sg |  |
| O | Singapore | John Chong | john_chong@caas.gov.sg |  |
| O | Spain | José Luis Chinchilla | jlchinchilla@indra.es |  |
| O | Spain | Manuel Garcia Martín | mangarcia@enaire.es |  |
| O | Thailand | Chavalit Ithiapa | chavalit.i@caat.or.th |  |
| O | Thailand | Phichpawis Plengsiriwat | phichpawis.p@caat.or.th |  |
| M | United Kingdom | John Mettrop | john.mettrop@caa.co.uk |  |
| A | United Kingdom | Kamlesh Masrani | Kamlesh.Masrani@inmarsat.com |  |
| A | United Kingdom | Stuart Rankin | stuart.rankin@caa.co.uk |  |
| A | United Kingdom | Stephen Parry | stephen.parry@nats.co.uk |  |
| M | United States | Chris Tourigny | chris.tourigny@faa.gov |  |
| A | United States | Donald Nellis | donald.nellis@faa.gov |  |
| A | United States | Gregory Baker | Gregory.Baker@fcc.gov> |  |
| A | United States | Kathryn Bernazzani | Kathryn.Bernazzani@dot.gov |  |
| A | United States | Miles Bellman | miles.e.bellman@faa.gov |  |
| A | United States | Bruce Lamb | blamb@ntia.gov |  |
| A | United States | Damon Ladson | dladson@hwglaw.com |  |
| A | United States | Eric Lee | elee@ntia.gov |  |

|  | International Organization |  |  |
| --- | --- | --- | --- |
| M | ASRI | Andrew Roy | acr@asri.aero |
| A | ASRI | Kris Hutchison | keh@asri.aero |
| M | EUROCONTROL | Raffi Khatcherian | Raffi.khatcherian@eurocontrol.int |
| A | EUROCONTROL | John Micallef | john.micallef@eurocontrol.int |
| M | IATA | Ruby Sayyed | sayyedr@iata.org |
| A | IATA | Khaled Eltanany | eltananyk@iata.org |
| M | ICCAIA | Joseph Cramer | joseph.cramer@boeing.com |
| A | ICCAIA | Laurent Azoulai | laurent.azoulai@airbus.com |
| A | ICCAIA | Claude Pichavant | claude.pichavant@airbus.com |
| A | ICCAIA | Marcella Ost | Marcella.s.ost@boeing.com |
| M | ITU | Nikolai Vassiliev | nikolai.vassiliev@itu.int |
| O | SITA | Eric Wawrzynkowski | eric.wawrzynkowski@sita.aero |
| S | ICAO | Loftur Jonasson | LJonasson@icao.int |
| S | ICAO | Mie Utsunomiya | mutsunomiya@icao.int |
| S | ICAO | Isaiah Tefutor | itefutor@icao.int |

APPENDIX D

ACTION ITEM LIST

| Number | Description | Actionee | Due Date | Status |
| --- | --- | --- | --- | --- |
| 10-03 | Report on current use in States of the 1300-1350 MHz frequency band for radars and longer-term plans. | All | CLOSED | CLOSED in FSMP-WG/18 Strongly encouraged given possible future WRC-27 agenda item. Relevant papers: WG/16 WP22, WG/17 WP13, and WG/17 IP02 |
| 10-08 | Develop contributions on how to ensure protection of aeronautical systems that are not included in the ITU MIFR. | All | CLOSED | CLOSED in FSMP-WG/18 Possible options need to be investigated on generic MIFR registrations. Has potential relevance to WRC-23 AI 1.7. Relevant papers: WG/17 WP17 and WG/17 IP14 |
| 11-04 | Participate in the radio altimeter correspondence group (CG-RA). | All | Ongoing | Send Email to christian.fleury@aviation-civile.gouv.fr |
| 12-10 | Determine if generic guidance can be developed to help aviation authorities analyse potential 5G impacts on radio altimeters given their specific State implementation. |  | CLOSED | CLOSED in FSMP-WG/18 Addressed in WG/16 WP05 and 26, establishing a CG to coordinate possible material. Relevant papers: WG/17 WP18 |
| 13-01 | Provide examples of national specifications or regulations on adjacent-band signal rejection for receivers. | FSMP participants who are also spectrum regulators in their country | FSMP-WG/19 |  |
| 14-03 | Liaise FSMP-WG/14 WP06 to RPASP WG3 to solicit input to the under-development ITU-R Handbook on detect and avoid systems. | Secretary | CLOSED | CLOSED in FSMP-WG/18 – See Action 18-06 FSMP Liaison was sent, waiting for finalization of DAA Manual. Secretary to confirm current status and liaise to WP5B if finalized |
| 14-04 | Participate in CG-MSS to complete the Note providing information on ensuring compatibility between IMT operated below 1518 MHz and aeronautical MSS services above 1525 MHz. | All | CLOSED | CLOSED in FSMP-WG/18 – See action item 18-05 To participate send an email to Kamlesh Masrani Kamlesh.Masrani@inmarsat.com Relevant papers: WG/16 Flimsy07 WG/17-WP14 |
| 16-01 | Encourage inputs to the new CG on GNSS RFI to help develop the text for a possible ITU-R Resolution at WRC-23. See paper WG/16 WP04. | All | CLOSED | CLOSED in FSMP-WG/18 CG Chair: Guillaume Novella Relevant papers: WG/17 WP08 |
| 16-04 | Contributions be sought on appropriate material for radio altimeter SARPs. See paper WG/16 WP26. | All | CLOSED | CLOSED in FSMP-WG/18 See Action Items 18-04 Relevant papers: WG/17 WP15 |
| 16-05 | Send documents WG/16 WP05 and WP26 on possible new radio altimeter SARPS to RTCA/EUROCAE for their consideration and input. | ICAO Secretariat | CLOSED | CLOSED in FSMP-WG/18 See Action Items 18-04 The document was sent and presented at the relevant RTCA/EUROCAE meeting. CLOSED. |
| 16-06 | Send WG/16 WP05 and WP26 to the Flt Ops Panel for comments and any interim guidance it could provide on the radio altimeter operational topics raised by the next FSMP. | ICAO Secretariat | CLOSED | CLOSED in FSMP-WG/18 See Action Item 18-03 The document was sent, waiting for their comments from Flt Ops. Secretary to provide updates on this action item if any at the future meeting of FSMP |
| 16-10 | Provide an update of the outcome from the NSP reorganizing the work of the Spectrum SWG. See paper WG/16 IP02. | ICAO Secretariat | FSMP-WG/18 | CLOSED Completed in FSMP-WG/18 discussion |
| 17-01 | Propose TOR for the VHF SATELLITE BASED CG and present it at the next WG. (see section 3.2.3.5 of this meeting report). | All | FSMP-WG/18 | CLOSED See FSMP-WG/18 Flimsy06 |
| 17-02 | Refine and submit the updated proposal (referring to FSMP-WG/17 WP04) to be included in Chapter 9 of Volume I Handbook (see section 7.1.3 and 7.1.5 of this meeting report). | IATA | FSMP-WG/18 | CLOSED See FSMP-WG/18 WP04 |
| 17-03 | Referring to FSMP-WG/17 WP05, provide proposal describing the framework for out-of-band/adjacent band coexistence studies, to be included in the Handbook. (see sections 7.1.4 and 7.1.5 of this meeting report). | IATA | FSMP-WG/18 | CLOSED See FSMP-WG/18 WP04 |
| 18-01 | Provide additional inputs to the proposed LS on WRC-27 Agenda Item 1.7. | All | FSMP-WG/19 |  |
| 18-02 | Request for radio altimeter manufacturers provide any data input that may augment or update existing technical details in the ITU-R documentation on radio altimeters. | A. Roy J. Cramer | FSMP-WG/19 |  |
| 18-03 | Send the aircraft operational scenarios proposed in FMSP WG/18 WP27 to both the ICAO Flt Ops and Aerodrome panels for input and any other considerations. | Secretariat | FSMP-WG/19 |  |
| 18-04 | Send meeting feedback to FSMP-WG/18 WP26 on radio altimeter SARPs back to RTCA and EUROCAE, and report back at next meeting. | A. Roy | FSMP-WG/19 |  |
| 18-05 | Provide comments on FSMP-WG/18 Flimsy03 to finalize the MSS L-Band note. | All | FSMP-WG/19 |  |
| 18-06 | Request regular updates on the DAA system aspects from the RPAS panel, specifically frequency arrangements. | Secretariat | FSMP-WG/19 |  |
| 18-07 | Provide input contributions at the next FSMP meeting on necessary changes needed (if any) to Annex 10, Vol. V to reflect the spectrum updates made as part of the RPAS Panel’s updates to other Annex 10 Volumes. | All | FSMP-WG/19 |  |
| 18-08 | Provide comments to the NSP on the proposed changes on GBAS/VDB frequency assignment planning in Doc 9718 Vol II from FSMP-WG/19 WP02. | All | FSMP-WG/19 |  |
| 18-09 | Provide contributions to discuss and develop a comprehensive definition of safety systems and services as proposed in FSMP-WG/18 WP12. | All | FSMP-WG/19 |  |
| 18-10 | Taking into account comments provided by FSMP-WG/18 on IP11, provide update of concept paper on Hyper connectivity Air Traffic Management. | Mr. Cramer and Mr. Azoulai | FSMP-WG/19 |  |

APPENDIX E

Draft ICAO Position for WRC-27

APPENDIX F

Space-Based VHF Correspondence Group Terms of Reference

APPENDIX G

Draft Information Note

(Measures to address potential blocking of MES operating in frequency bands 1 525-1 559 MHz on the ground at airports from terrestrial IMT systems planned in the frequency band below 1518 MHz) Clean version

APPENDIX H

Draft Doc.9718.Volume I (Clean version)

-END-