	[image: image4.png]



	[image: image4.png]
International Civil Aviation Organization

INFORMATION PAPER
	FSMP-WG/18- IP/10
06/02/2024




ATMRPP-WG/WHL/4-WP/


.././06
FSMP-WG/18- IP/10
- 2 -

- 3 -
FSMP-WG/18- IP/10

FREQUENCY SPECTRUM MANGEMENT PANEL (FSMP)
EIGHTEENTH MEETING

Montreal, Canada, 6-16 February 2024
Agenda Item 9:
Any Other Business
Receiver Characteristics
(Presented by John Mettrop)

	SUMMARY

	This information paper provides a copy of  a new ECC Recommendation and Report on “Receiver Resilience to Transmission on Adjacent Frequency Ranges” that is being sent for approval. 


1. DISCUSION
1.1 Within the ITU it is only recently as spectrum has become an apparent scarce resource, that receiver performance has become an issue. The current Radio Regulations hardly have any provisions about the performance of receivers with only the following articles being directly relevant:
3.9

The bandwidths of emissions also shall be such as to ensure the most efficient utilization of the spectrum; in general this requires that bandwidths be kept at the lowest values which the state of the technique and the nature of the service permit. Appendix 1 is provided as a guide for the determination of the necessary bandwidth.

3.11

Wherever necessary for efficient spectrum use, the receivers used by any service should comply as far as possible with the frequency tolerances of the transmitters of that service, due regard being paid to the Doppler effect where appropriate.

3.12

Receiving stations should use equipment with technical characteristics appropriate for the class of emission concerned; in particular, selectivity should be appropriate having regard to No. 3.9 on the bandwidths of emissions.

3.13

The performance characteristics of receivers should be adequate to ensure that they do not suffer from interference due to transmitters situated at a reasonable distance and which operate in accordance with these Regulations.

1.2 However, there are indications that Study Group 1 intend to review all of the ITU material related to receivers and transmitters with a view to tightening requirements.

1.3 Within Europe and the CEPT a draft new report and recommendation on “Receiver Resilience to Transmission on Adjacent Frequency Ranges” by SE21 which will be considered by the next SE meeting (Copies attached). Once these documents have been approved then it is likely that ETSI will look to develop a new Harmonized Standard that will be applicable to all equipment covered by the Radio Equipment Directive. Noting that includes all aeronautical ground equipment and airborne equipment working in frequency bands that are not are intended to operate only on frequencies allocated by the Radio Regulations of the International Telecommunications Union for protected aeronautical use.  

1.4 The documents produced by SE21 give a clear indication of the path regulation of receivers is likely to go and aviation when developing new or revisions to existing equipment should take this into account

— END —

Attachment

[image: image1.emf]Rx_Rep.docx



[image: image2.emf]Rx_Rec.docx

[image: image3.png]Editor‘s Note:




(3 pages)

FSMP-WG18-IP10_Receiver Standards.doc

[image: ecc_logo]

[image: cept logo]





 DRAFT

DRAFT



Draft ECC REPORT <No> - Page 1

Draft ECC REPORT <No> - Page 1

Technical analysis to support the ECC Recommendation on receiver resilience to transmission on adjacent frequency ranges

[bookmark: Text8]approved DD Month YYYY (Arial 9)[bookmark: Report_Number]ECC Report <No>



[bookmark: Text3][last updated: DD Month YYYY] (Arial 9)



[bookmark: _Toc380056496][bookmark: _Toc380059747][bookmark: _Toc380059784][bookmark: _Toc396153635][bookmark: _Toc396383862][bookmark: _Toc396917295][bookmark: _Toc396917344][bookmark: _Toc396917406][bookmark: _Toc396917459][bookmark: _Toc396917626][bookmark: _Toc396917641][bookmark: _Toc396917746][bookmark: _Ref129085003][bookmark: _Toc129955989][bookmark: _Toc155273990]Executive summary

[bookmark: _Hlk115801412][bookmark: _Hlk117516969]This report describes the method for receiver resilience “MRR” to calculate the receiver resilience levels, namely the adjacent channel PR and blocking levels, to be included in the ECC Recommendation on receiver resilience to transmission on adjacent frequency ranges [1].

Section 1 presents the definition of the most important terms used in this report. Section 2 provides the basic theory of receiver resilience and presents the derivation of the basic equation of MRR. Section 3 presents in detail the use of MRR to calculate/specify the receiver resilience levels to be included in CEPT / ECC Recommendation on receivers. The last section of the report, Section 4, presents the conclusions of the report. Extensive information on the work carried out within the development of MRR is provided in ANNEX 1: to ANNEX 6:. MRR is a technology neutral generic method. It uses four parameters N, M, ILR and FOS to calculate the receiver resilience levels by a single equation derived from fundamental equations describing the mechanism of receiver resilience to a frequency offset interfering signal.

The validity of MRR was assessed by calculating receiver resilience levels defined in eight different ETSI Harmonised Standards for numerous receiver configurations. The results obtained showed good agreement with the adjacent channel PR and blocking levels defined in the respective harmonised standards.

Based on the analysis carried out in this report, the proposed receiver resilience levels in the ECC Recommendation on receivers are determined as follows:

In the Out-Of-Band (OOB) domain, the proposed resilience levels have been determined from the intra/inter-system interfering signals and interference scenarios defined in ETSI Harmonised Standards. 

In the reciprocal spurious blocking domain (RSBD), the proposed resilience levels have been determined based on: 

a reference interfering signal (OFDM signal) having SE (spurious emissions) levels in accordance with the most common SE levels defined in ERC Recommendation 74-01 [2]; and

the equivalent blocking levels for a CW interfering signal to allow the conformance tests to be done with a CW signal as an alternative; 

It should be noted that MRR is intended to calculate the resilience levels of receivers in the presence of a single frequency offset interfering signal as in all harmonised standards. It is not intended to evaluate the impact of undesired signal variations, inherent in receivers, on receiver performance (e.g., to predict intermodulation response rejection or to model a badly-behaved non-linear receiver), or to predict the behaviour of a specific receiver or for system design/development. Furthermore, in some cases the initial outcome of MRR may not lead directly to receiver levels which improve spectrum efficiency.

There are some types of receivers, e.g. receivers used by passive radio services, where the applicability of the MRR would require further investigation.






TABLE OF CONTENTS









0	Executive summary	2

1	Introduction	9

2	Definitions	10

3	Basic theory on receiver resilience	11

3.1	Frequency offset interfering signal power received by the victim receiver	12

3.2	Relation between FOS, RIR and ILR	14

3.3	Derivation of levels of receiver resilience to transmission on adjacent frequency ranges	15

4	Method for Receiver Resilience (MRR) used to dereive blocking and selectivity levels	16

4.1	Use of the method	16

4.2	Consideration of different bandwidths of victim and interferer	16

4.3	Consideration of peak-to-average power ratio (PAPR)	16

4.4	Basics and application of MRR	16

4.4.1	Basics of the method	16

4.4.2	Application of the method	17

5	Conclusions	23

ANNEX 1: Derivation of the Method for Receiver Resilience (MRR)	25

ANNEX 2: Examples of application of MRR	27

ANNEX 3: General guidance on parameters M, N, ILR and FOS	38

ANNEX 4: Potential Methods for testing	48

ANNEX 5: Reference interfering signal and a single interference scenario to be used in the calculation of receivers resilience levels using MRR	50

ANNEX 6: Point-to-point FS emission masks for ILR/ACLR evaluation	66

ANNEX 7: Background information on RAS receivers	68

ANNEX 8: List of References	77








LIST OF ABBREVIATIONS 



		Abbreviation

		Explanation



		CEPT

		European Conference of Postal and Telecommunications Administrations



		ECC

		Electronic Communications Committee



		ACLR

		Adjacent Channel Leakage Ratio



		ACS

		Adjacent Channel Selectivity



		A/D

		Analog/Digital



		ADC

		Analog Digital Converter



		BB

		Base Band



		BS

		Base Station



		BER

		Bit Error Rate



		BW

		Bandwidth



		C/I

		Carrier-to-Interference ratio



		CW

		Continuous Wave



		D/A

		Digital/Analog



		DAB

		Digital Audio Broadcasting



		DFRS

		Digital fixed Radio Systems



		DRM

		Digital Radio Mondiale



		DTT

		Digital Terrestrial Television



		DVB-T

		Digital Video Broadcasting – Terrestrial



		ECSD

		Enhanced Circuit-Switched Data



		EIRP

		Effective Isotropic Radiated Power



		ETSI

		European Telecommunications Standards Institute



		E-UTRA

		Evolved Universal Terrestrial Radio Access



		FOS

		Frequency Offset Selectivity



		FPGA

		Field Programmable Gate Array



		FRB

		Fast Radio Burst



		FS

		Fixed Service



		GPU

		Graphics Processing Unit



		GSM

		Global System for Mobile



		HS

		Harmonised Standards



		HTS

		High Temperature Superconductor



		IF

		Intermediate Frequency



		ILR

		Interference Leakage Ratio



		IMT

		International Mobile Telecommunications



		IoT

		Internet of Things



		LAN

		Local Area Network



		LNA

		Low Noise Amplifier



		LTE

		(IMT) Long Term Evolution (network)



		M

		Receiver desensitisation



		NB

		Narrow-Band



		N

		Noise Floor



		NF

		Noise Figure



		NFD

		Net Filter Discrimination



		OMT

		Orthogonal Mode Transducer



		OFDM

		Orthoganal Frequency Division Multiplexing



		OOB

		Out-Of-Band



		OOBD

		Out-Of-Band Domain



		OOBE

		Out-Of-Band Emissions



		PAF

		Phased Array Feed



		PAPR

		Peak-to-Average Power Ratio



		PtP

		Point-to-Point



		PR

		Protection Ratio



		QAM

		Quadrature Amplitude Modulation



		QoS

		Quality of Service



		RAS

		Radio Astronomy Service



		RF

		Radio Frequency



		RED

		Radio Equipment Directive



		RI

		Reference Interferer



		RIR

		Receiver Interference Ratio



		RLAN

		Radio Local Area Network



		RMS

		Root-Mean-Square



		RSBD

		Reciprocal Spurious Blocking Domain



		RQZ

		Radio Quiet Zone



		MRR

		Method for Receiver Resilience



		RX

		Receiver



		SD

		Spurious Domain



		SE

		Spurious Emissions



		SEAMCAT

		Spectrum Engineering Advanced Monte Carlo Analysis Tool



		SINR

		Signal to Interference and Noise Ratio



		S/N

		Signal/Noise



		SNR

		Signal to Noise Ratio



		SRD

		Short Range Devices



		SRR

		Short Range Radar



		TX

		Transmitter



		UWB

		Ultra-Wide Band



		VGOS

		VLBI Global Observing System



		WBDT

		Wide-Band Data Transmission



		VLBI

		Very Long Baseline Interferometry



		WBSEL

		Wide-Band Selectivity



		WiMAX

		Worldwide Interoperability for Microwave Access





[bookmark: _Toc380056497][bookmark: _Toc380059748][bookmark: _Toc380059785][bookmark: _Toc396153636][bookmark: _Toc396383863][bookmark: _Toc396917296][bookmark: _Toc396917345][bookmark: _Toc396917407][bookmark: _Toc396917460][bookmark: _Toc396917627][bookmark: _Toc396917642][bookmark: _Toc396917747][bookmark: _Toc129783561][bookmark: _Toc155273991][bookmark: _Toc129783821][bookmark: _Toc129783902][bookmark: _Toc129955111][bookmark: _Toc129955990]Introduction

[bookmark: _Hlk107475195][bookmark: _Toc129783822][bookmark: _Toc129783903][bookmark: _Toc129955112][bookmark: _Toc129955991]This technical report aims to develop a methodology, which will be part of the ECC Recommendation on receiver resilience to transmission on adjacent frequency ranges, to be used for specifying recommended levels of receiver resilience to transmission on adjacent frequency ranges. Two different levels of receiver resilience will potentially be specified in the future recommendation; these are receiver adjacent channel PR (PRadj-ch) and blocking level (Iblk), both with a related frequency offset selectivity (FOS).

[bookmark: _Toc129783823][bookmark: _Toc129783904][bookmark: _Toc129955113][bookmark: _Toc129955992]Section 3 provides the basic theory of receiver resilience and Section 4 presents the method for receiver resilience (MRR) developed to calculate/specify the receiver resilience levels to be included in the ECC Recommendation on receivers [1].

[bookmark: _Ref131501245][bookmark: _Toc129955993][bookmark: _Toc155273992][bookmark: _Toc380056498][bookmark: _Toc380059749][bookmark: _Toc380059786][bookmark: _Toc396153637][bookmark: _Toc396155266][bookmark: _Toc396383864][bookmark: _Toc396917297][bookmark: _Toc396917346][bookmark: _Toc396917408][bookmark: _Toc396917461][bookmark: _Toc396917628][bookmark: _Toc396917643][bookmark: _Toc396917748]Definitions 

[bookmark: _Hlk147208156]The definitions below, with the exception of “Receiver desensitisation” and " Reciprocal spurious blocking domain", (M)”, are taken from ECC Report 310 [1] with some small modifications and are used in this report for receivers working in their linear range.



		Term

		Definition



		Blocking

		A measure of the receiver capability to receive a wanted signal without exceeding a given degradation due to the presence of an unwanted signal at any frequency other than those of the spurious responses or of the adjacent channels and it is defined as the maximum interfering signal level expressed in dBm reducing the specified receiver sensitivity by a certain number of dBs (desensitisation).



		Frequency Offset Selectivity (FOS)

		A measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted adjacent signal at a given frequency offset from the centre frequency of the assigned channel. In this context, it is defined as the ratio of the receiver filter attenuation on the offset frequency to the receiver filter attenuation on the assigned channel frequency (normally a positive number in dB).

FOS is of general use for any mixed wanted and unwanted signal situation.



		Interference Leakage Ratio (ILR)

		The ratio of the filtered mean power centred on the assigned channel frequency to the similarly filtered mean power centred on a given frequency offset.



		Receiver Interference Ratio (RIR)

		[bookmark: _Hlk147132415]The ratio of the in-channel interference power on a given frequency offset to the interference power received by the victim receiver.





		Receiver desensitisation (M)

		Reduction in the signal to noise ratio of the receiver or a reduction in the effective sensitivity in the presence of an interfering signal, given in dB. It corresponds to the ‘noise rise’ due to the interfering signal.



		Receiver noise floor (N)

		The total noise power at the receiver including the effect of thermal noise and the receiver noise figure.



		Reciprocal spurious blocking domain (RSBD)

		Implies that the victim receiver channel is in the spurious domain of the interfering transmitter and reciprocally the interfering transmitter channel is in the spurious domain of the victim receiver.





[bookmark: _Ref107398781][bookmark: _Ref107398793][bookmark: _Toc115888586][bookmark: _Toc129955994][bookmark: _Toc155273993][bookmark: _Toc380056507][bookmark: _Toc380059757][bookmark: _Toc380059795][bookmark: _Toc396153645][bookmark: _Toc396383873][bookmark: _Toc396917306][bookmark: _Toc396917417][bookmark: _Toc396917637][bookmark: _Toc396917652][bookmark: _Toc396917757]Basic theory on receiver resilience

It is important to understand the mechanism of receiver resilience to a frequency offset interfering signal before attempting to develop a methodology to be used for specifying recommended levels of receiver resilience to transmission on adjacent frequency ranges, which are blocking (Iblk) or adjacent channel protection ratio (PRadj-ch).

The resilience of a receiver to a frequency offset interfering signal depends on the receiver's overall performance based on filtering, linearity impairment effects, demodulation and decoding techniques used. Nevertheless, it can be simply defined by the receiver blocking (Iblk), frequency offset selectivity (FOS) or receiver interference ratio (RIR) levels.

Note that in the theoretical development presented in the following sections the receiver intermodulation response rejection is not considered.

Receiver intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal, without exceeding a given degradation due to the presence of at least two interfering signals at frequencies f1 and f2, with a specific frequency relationship to the wanted signal frequency, which give rise to 2nd and 3rd order intermodulation products that may fall into the victim receiver channel.

However, the receiver intermodulation response rejection test is not included in all ETSI Harmonised Standards, because intermodulation in the RF tuner will result in a degradation in the adjacent channel selectivity, which is extensively tested in all ETSI Harmonised Standards. Moreover, real-life experience indicates that interference due to intermodulation is not a widespread problem between different system/services in Europe.

For example, HS dealing with DTT, DAB and fixed service do not include a specific intermodulation response rejection test. The impact of the intermodulation products on the receiver performances are covered by adjacent channel selectivity tests. The method developed in this report aims to derive blocking or selectivity levels of a receiver in the presence of a single interfering signal as it is done in all harmonised standards.

There are some types of receivers, e.g. receivers used by passive radio services, and the applicability of the MRR for these receivers would require further investigation. Relevant information about the receiver resilience of RAS can be found in Annex 7.

[bookmark: _Toc115888587][bookmark: _Ref131501847][bookmark: _Ref131501854][bookmark: _Ref129084087][bookmark: _Ref136602520][bookmark: _Toc155273994]Frequency offset interfering signal power received by the victim receiver



[image: ]

[bookmark: _Ref127774060]Figure 1: Frequency offset interfering signal power received by the victim receiver

The victim receiver receives both the interfering signal in-channel emissions (Iin-ch) attenuated by FOS and the interfering signal out-of-channel emissions (Ioo-ch) falling into its channel, which are defined by ILR. The interfering signal power received by the victim receiver is shown in Figure 1 and can be expressed in the linear domain as:

		



		[bookmark: _Ref129084061][bookmark: _Ref127805638](1)





In the logarithmic domain:

		

		(2)





Where:

: frequency offset interfering signal power received by the victim receiver after RF/IF/BB filtering;

: frequency offset interfering signal in-channel power, measured in its channel bandwidth at the receiver input before RF/IF/BB filtering;

: frequency offset interfering signal out-of-channel power, received by the victim receiver after RF/IF/BB filtering;

: frequency offset interfering signal in-channel power received by the victim receiver after RF/IF/BB filtering.

Note that when defining the resilience levels of a receiver, Iin-ch is often referred to as Iadj-ch or Iblk depending on the frequency offset between the useful and interfering signals.

Note also that, as shown in Figure 1, the ILR and FOS are, formally, calculated as the ratios of the total in-channel interfering power () to the result of the following integrations:

ILR in the linear domain:

		

		(3)







ILR in the logarithmic domain:

		

		[bookmark: _Ref129084027][bookmark: _Ref131501671](4)





Where:

: centre frequency of the interfering transmitter;

: centre frequency of the victim receiver;

 and : interferer and victim bandwidth respectively.

FOS, which is the frequency offset selectivity of the receiver (equivalently the overall receiver filter attenuation at the related frequency offset), can be expressed in the linear domain as:

		

		(5)





Therefore:

		

		(6)





FOS in the logarithmic domain:

		

		[bookmark: _Ref129084023][bookmark: _Ref131501655](7)





Where:

 and : interferer and victim bandwidth respectively;

: interferer spectral density in the linear domain;

: victim selectivity in the linear domain, in number, relative to the 0 value at its centre frequency .

For a sufficiently large offset frequency, where the interferer relative spectral density and the victim receiver selectivity can be considered as flat and equal to a constant value, the value of FOS is independent from the interference signal bandwidth () as shown in Equation (7) while the value of ILR varies as a function of the ratio  as shown in Equation (4).

The receiver desensitisation “M” is the reduction in the signal to noise ratio of the receiver or a reduction in the effective sensitivity in the presence of an interfering signal. The receiver desensitisation corresponds to the ‘equivalent noise rise’ due to the interfering signal, which can be expressed in the linear domain as:

		

		[bookmark: _Ref129243446](8)





Where:

N: receiver noise floor measured in the receiver bandwidth;

: interfering signal power received by the victim receiver after RF/IF/BB filtering.

Note from Equation (8) that M is always greater than 1. Then, the interfering signal power received by the victim receiver is:

		

		[bookmark: _Ref129084044][bookmark: _Ref131501706](9)





In the logarithmic domain:

		





		(10)





Equation (9) shows the relation between ,  and . For more detailed information on , ,  see ANNEX 3:.

[bookmark: _Toc115888588][bookmark: _Ref131501860][bookmark: _Ref129084092][bookmark: _Toc155273995]Relation between FOS, RIR and ILR

From its definition, the RIR can be expressed in the linear domain as:

		

		(11)





By replacing  with the expression given in Equation (1):

		



		(12)





Or, equivalently,

		

		(13)





By rearranging the above equation:

		

		[bookmark: _Ref129083986][bookmark: _Ref131501787](14)





In the logarithmic domain:

		



		(15)





Equation (14) shows the relation between FOS, RIR and ILR.

[bookmark: _Toc115888589][bookmark: _Toc155273996]Derivation of levels of receiver resilience to transmission on adjacent frequency ranges

The levels of receiver resilience to transmission on adjacent frequency ranges for a given receiver desensitisation “M” can be defined either by , FOS or RIR. These three parameters are linked to each other as demonstrated in the previous sections,  and FOS are more commonly used in harmonised standards to specify receiver resilience.

Equations (1), (9) and (14) developed in Sections 3.1 and 3.2 are the fundamental equations describing the mechanism of receivers resilience to a frequency offset interfering signal. They can be used to develop a method to derive levels of receiver resilience to transmission on adjacent frequency ranges.

Equating equations (1) and (9) (linear terms) leads to the basic equation of MRR, which is used to derive levels of receiver resilience to transmission on adjacent frequency ranges as described in ANNEX 1:, Section A1.1:

		

		[bookmark: _Ref129084184][bookmark: _Ref131501933](16)





Or in the logarithmic domain:

		

		(17)





[bookmark: _Ref107398251][bookmark: _Toc115888590][bookmark: _Toc129955995][bookmark: _Ref136617295][bookmark: _Toc155273997]Method for Receiver Resilience (MRR) used to dereive blocking and selectivity levels

[bookmark: _Toc115888591][bookmark: _Toc155273998]Use of the method

MRR aims to calculate/specify the receiver resilience levels to be included in the ECC Recommendation on receivers. The basic equation of the method Equation (16) predicts the frequency offset interfering signal level (Iin-ch) at the receiver input and consequently the PRadj-ch and blocking level (Iblk) of the receiver based on four input parameters N, M, ILR and FOS.

The basic equation of MRR is not intended to evaluate the impact of undesired signal variations, inherent in receivers, on their performance (or to model a badly-behaved non-linear receiver), nor to predict the behaviour of a specific receiver.

[bookmark: _Toc115888592][bookmark: _Toc155273999]Consideration of different bandwidths of victim and interferer

The parameters ILR and FOS of the basic equation of MRR implicitly take into account the interfering and victim signal bandwidths as explained in ANNEX 3:, Section A3.2.3.

[bookmark: _Toc115888593][bookmark: _Toc155274000]Consideration of peak-to-average power ratio (PAPR)

In modern communication systems, the PAPR of the majority of transmitted signals is optimised (minimised) to improve the efficiency of transmitters. This enables to prevent high undesirable power dissipation at the transmitter (cost reduction) and to improve the reliability of the transmitted signal (high QoS). The minimisation of the PAPR reduces the impact of interfering signal on the receiver resilience levels.

However, the intended operation of some of radio systems requires significantly high peak to mean ratio (e.g. most radio-determination/radio-location and UWB applications).

In most of the ETSI HS, the blocking level of receivers (Iblk) is defined as mean power of the interfering signal. Usually, Iblk is measured in RMS (root mean square) measurement mode in a bandwidth equal to the interfering signal channel bandwidth at the input of the victim receiver in the presence of the interfering signal under normal operating conditions. In the case of a time varying interfering signal, the Iblk is the maximum RMS power of the active portions of the interfering signal measured over a sufficiently long. interval of time. The Iblk predicted by the basic equation of MRR is the interfering signal in-block mean power.

In the presence of interfering signals with a relatively small average power associated with a possible high peak-to-average ratio as pulse modulated signals, both peak and average power may need to be considered. Applicability of the basic equation of MRR to such interfering scenarios should be checked. ETSI EN 303 340 [4] is one of the few harmonised standards that uses a pulsed type interfering signal, for a specific interference configuration, to define the blocking level of digital terrestrial television (DTT) receivers. In this specific case the blocking level is defined from the mean rms value of the interference signal measured during its active period (which should be of the same order as the peak power), the PAPR being about 18 dB. The validity of the defined blocking level was successfully checked by using MRR. 

[bookmark: _Toc107486746][bookmark: _Toc115888594][bookmark: _Toc155274001]Basics and application of MRR

[bookmark: _Toc155274002]Basics of the method

MRR derives the frequency offset interfering signal level (Iin-ch) at the receiver input and consequently the blocking level (Iblk) and PRadj-ch of the receiver, for given values of M, N, ILR and FOS, from the equation below (see ANNEX 1:). Iin-ch is referred as Iblk when calculating the blocking level and as Iadj-ch when calculating the adjacent channel protection ratio (PR), where:



Iin-ch: frequency offset interfering signal power measured in the interferer channel bandwidth at the receiver input before RF/IF/BB filtering

Iblk: blocking interfering signal power measured in the interferer channel bandwidth at the receiver input before RF/IF/BB filtering;

Iadj-ch: adjacent channel interfering signal power measured in the interferer channel bandwidth at the receiver input before RF/IF/BB filtering.

The bandwidth of the wanted received signal and the bandwidth of the interfering signal need to be determined in advance. More information on bandwidth definition can be found in recommends 1 of ERC Recommendation 74-01 [2]. The frequency offset is the offset between the centre frequency of the wanted received signal bandwidth and the centre frequency of the interfering signal bandwidth.

[bookmark: _Hlk147129932]For the frequency offsets beyond the second adjacent channel, the blocking level :

		

		[bookmark: _Ref128056495](18)





[bookmark: _Hlk147129969]Or, for first and second adjacent channels frequency offsets, the carrier to interference ratio () as follows:

		



		(19)





Where:

N: Noise floor of the victim receiver;

M: Victim receiver maximum acceptable desensitisation at a given frequency offset;

FOS: Frequency offset selectivity of the victim receiver;

ILR: Leakage power ratio of the interfering signal at offset frequency;

C: useful signal level received by the victim receiver;

: blocking interfering signal power measured in its channel bandwidth at the receiver input before RF/IF/BB filtering;

: adjacent channel interfering signal power measured in its channel bandwidth at the receiver input before RF/IF/BB filtering;

: receiver sensitivity level, specified for the victim receiver.

[bookmark: _Ref129092108][bookmark: _Toc155274003]Application of the method

[bookmark: _Hlk117514781]The steps below should be followed to derive the receiver blocking level (Iblk) or the adjacent channel protection ratio (PRadj-ch=C/Iadj-ch):

Step 1	Calculate the noise floor (N) of the receiver:

Every radio receiver is subject to a noise floor that can be described using the following equation and expressed in dBm:

		

		(20)





Where:

N: receiver noise floor (dBm);

k: Boltzmann constant in Joules per Kelvin (1.381×10-23);

T: temperature in degrees Kelvin (for common terrestrial radio receivers, 290 K can be used);

B: receiver bandwidth in Hertz;

kTB: receiver thermal noise in Watts;

NF: receiver Noise Figure (dB).

In cases where the noise figure is not known, the noise floor may also be derived from the receiver sensitivity provided that the interference signal bandwidth is equal to that of the receiver signal bandwidth: 

		



		(21)





Step 2	Calculate the receiver desensitisation (M) (go to Step 3 if M is known):

M is often found in ETSI Harmonised Standards, either explicitly mentioned, or calculated from the required “Receiver sensitivity” (in absence of interference) and the “Minimum wanted received signal level” required in the blocking test through the equation:

		

		(22)





If it is not possible to find/calculate M from the ETSI Harmonised Standard as above, the following values: M=3 dB and/or M = 15 dB should be assumed. The M = 3 dB value may be assumed for systems deployed in a “noise driven performance” operational environment, while the M = 15 dB may be assumed for systems deployed in an “interference driven performance” operational environment (see ANNEX 3:, Section A3.2.2).

Note that the value of M should be chosen based on the expected operational range of the receiver.

Step 3	Determine the ILR value to be used in Step 4 (see ANNEX 3:, section A3.2.4 for more detailed information):

a) [bookmark: _Hlk117515352]Identify the interfering signal and interference scenario:

The relevant interfering signal and interference scenario can be identified based on:

the existing or planned deployment of victim and interfering systems and the compatibility studies presented in various CEPT/ECC, ETSI and ITU-R technical reports (see Annex A3.3)

or, if the information in the previous paragraph is not available, information may be sourced from either of the following:

from the intra/inter-system interfering signals and interference scenarios defined in ETSI Harmonised Standards; or

from the reference interfering signal and interference scenario defined in ANNEX 5: 



Determine the ILR value

Once the relevant interfering signal and interference scenario are identified the following approach can be used to determine the appropriate ILR value:

If the victim receiver channel is in the out-of-band (OOB) domain of the interfering signal (see ANNEX 3:, Section A3.2.4):

Use either of the following:

the ILR (ACLR) value defined in existing EC/ECC decisions, CEPT/ITU-R recommendations, CEPT/ECC reports and ETSI Harmonised Standards /technical specifications/reports;

the ILR value determined by measurements;



If the victim receiver channel is in the reciprocal spurious blocking domain (RSBD) (see ANNEX 3:, Section A3.2.4):

Use either of the following:

the ILR value derived from the in-channel power of the interfering signal and the spurious emission levels defined in external documents including: the existing EC/ECC decisions, and ETSI Harmonised Standards/technical specifications/reports, ECC/ITU-R Recommendations (e.g. ERC/REC 74-01 [2], Rec. ITU-R SM.329-12 [7]);

the ILR value derived from the reference interfering signal defined in ANNEX 5:.

Step 4	 Define the frequency offset selectivity (FOS) of the receiver based on the following options:

Depending on the scenario, FOS can either be derived directly from the ETSI Harmonised Standard, or according to:



		

		[bookmark: _Ref131509239](23)





where the value of X depends on the maximum acceptable increase of the interfering signal power () at the receiver input due to , e.g.:

X = 0 dB would correspond to		

X = 10 dB would correspond to	          

Where:



: frequency offset interfering signal out-of-channel power received by the victim receiver;

: frequency offset interfering signal in-channel power received by the victim receiver.

FOS is defined based on the scenario as follows:



If the victim receiver channel is in the OOB domain of the interfering signal:



In the case of an interfering signal and an interference scenario identified based on an ETSI Harmonised Standard (i.e from Step 3):

[bookmark: _Hlk147144576]if the interfering signal is an unmodulated continuous wave (CW) signal, the value of FOS can be determined based on the adjacent channel interfering signal limit defined in ETSI Harmonised Standards as follows, noting that this is a rearrangement of equation (18) and the value may have already been available:

				           (24)

if the interfering signal is similar to the wanted signal, the value of FOS is determined according to equation (23) with the value of X defined according to the relevant ETSI harmonised standard;

In this interference configuration the impact of the interfering signal OOBE falling into the receiver channel, added to the interfering signal in-channel emission received by the receiver after filtering, is important. As the emissions falling into the receiver channel cannot be reduced by filtering, it is sensible to reduce as much as possible the interfering signal in-channel emission at the receiver input by filtering to reduce the impact of the interfering signal on the receiver performance. Therefore, setting X = 10 such that FOS=ILR + 10 dB would be an appropriate choice when deriving the adjacent-channel protection ratio of a receiver.



For all other cases (i.e. the reciprocal spurious blocking domain or interfering signal/scenario not based on an ETSI harmonised standard), the value of FOS is determined by equation (23) with the value of X set according to the maximum acceptable increase of the interfering signal power.	Comment by France: Comment 4 from annex 1 of the LS from ETSI

In most cases with the spurious emissions (SE) level of the interfering signal being low, X=0 dB such that FOS=ILR can be used in the calculation in order to avoid unnecessary constraint on the receiver. X=10 such that FOS=ILR+10 dB would be an appropriate choice if higher selectivity is required.

If a CW interfering signal is used, instead of the RI signal, to calculate the blocking level of a victim receiver, the receiver FOS value should be equal to the FOS value specified for the blocking level calculation with the RI signal, since the ILR value of the CW signal is assumed to be very high or infinite (≥130 dB).

Step 5	Derive the receiver blocking level (Iblk) or adjacent channel protection ratio (PRadj-ch):

In the RSBD:	Comment by France: Comment 4 from Annex 1 the Ls from ETSI



		

		[bookmark: _Ref131511639](25)





Or if the ILR of the interfering signal can be considered infinite (very high):

		

		(26)





In the OOB domain:

Calculate PRadj-ch (dB) as follows, with Iadj_ch evaluated from equation (25) by replacing Iblk with Iadj_ch:

  by replacing  with :

		



		(27)





[bookmark: _Ref131322240]The method is illustrated in the following flowchart.



 

Figure 2: Flowchart describing the method for receiver resilience

[bookmark: _Toc129955996][bookmark: _Toc155274004]Conclusions

[bookmark: _Toc169147730][bookmark: _Toc380059616][bookmark: _Toc380059758]The method for receiver resilience (MRR) is a technology neutral generic method. It uses four parameters N, M, ILR and FOS to calculate the receiver resilience levels by a single equation derived from fundamental equations describing the mechanism of receiver resilience to a frequency offset interfering signal.

The validity of MRR was assessed by calculating receiver resilience levels defined in eight different ETSI Harmonised Standards for numerous receiver configurations. The results obtained showed good agreement with the adjacent channel PR and blocking levels defined in the respective harmonised standards.

Five distinct steps should be followed to derive the receiver resilience levels when using MRR. 

The most important step is Step 3, which aims to identify the interfering signal and interference scenario, and to determine the ILR value to be used in the calculation. This can be achieved, as proposed in this report, by two different approaches each with advantages and disadvantages:

1. Based on the existing or planned deployment of victim and interfering systems and the compatibility studies presented in various CEPT/ECC, ETSI and ITU-R technical reports or ETSI harmonised standards.

Advantages:

Possibility to choose the interfering signal and interference scenario for each system/service if they are specified in the existing technical reports or harmonised standards.

Possibility to compare the calculated resilience levels with those assessed in the existing technical reports or defined in the existing harmonised standards.

Disadvantages:

Possible difficulty to choose the most relevant interfering signal and interference scenario for each system/service if they are not clearly specified in the existing technical reports or harmonised standards. Note that this difficulty may not apply to all systems/services and can be overcome by dealing with the issue within the competent CEPT or ETSI technical group.

Difficulty to choose the most relevant interfering signal and interference scenario for a given system/service if there are no technical reports or harmonised standards available.

Difficulty to choose the ILR value if it is not clearly defined in the existing technical reports or harmonised standards, or if there are no technical reports or harmonised standards available.

From a reference interfering signal and interference scenario (one example is specified in ANNEX 5: of this report): 

Advantages:

No need to choose an interfering signal and interference scenario for each system/service, since a single well specified reference interfering signal and interference scenario are used for all systems/services independent of the radio environment in the field.

No need to choose the ILR value, since it is calculated from the well-defined spectrum mask of the reference interfering signal.

Disadvantages:

The specified reference interfering signal and interference scenario cannot be changed and should be used for all systems/services;

The specified reference interfering signal and interference scenario may not be fully relevant to some systems/services, since the real radio environment in the field is not considered in this approach.

Finally, the proposed receiver resilience levels in the ECC Recommendation on receivers are determined as follows:

In the Out-Of-Band (OOB) domain, the proposed resilience levels have been determined from the intra/inter-system interfering signals and interference scenarios defined in ETSI Harmonised Standards.

In the reciprocal spurious blocking domain (RSBD), the proposed resilience levels have been determined based on:	Comment by France: Comment 4 from Annex 1 of the LS from ETSI

i) a reference interfering signal (OFDM signal) having SE (spurious emissions) levels in accordance with the most common SE levels defined in ERC/REC 74-01 [2]; and

the equivalent blocking levels for a CW interfering signal to allow the conformance tests to be done with a CW signal as an alternative.

The report also presents the main information on how MRR was developed and can be used to calculate/specify the receiver resilience levels in the ECC Recommendation on receivers.

[bookmark: _Ref107392213][bookmark: _Toc115888596][bookmark: _Toc129955997][bookmark: _Toc155274005]Derivation of the Method for Receiver Resilience (MRR)

[bookmark: _Ref131501896][bookmark: _Ref129084144]Derivation of the basic equation of MRR

The basic equation of MRR shown below allows calculation of the receiver blocking level as a function of N, M, ILR and FOS:

		

		(28)





This equation has been derived through the following steps. Note that the derivation is carried out in the linear domain and only the final result is transformed from the linear domain to the logarithmic domain:

The victim receiver receives both the interfering signal in-channel emissions (Iin-ch) attenuated by FOS and the interfering signal out-of-channel emissions (Ioo-ch) falling into its channel, which are defined by ILR. 

		



		[bookmark: _Ref129956125][bookmark: _Ref156294417](29)





Where:

: frequency offset interfering signal power received by the victim receiver;

: frequency offset interfering signal power measured in its channel bandwidth at the receiver input before RF/IF/BB filtering;

: frequency offset interfering signal out-of-channel power received by the victim receiver;

[bookmark: _Hlk147145287]: frequency offset interfering signal in-channel power received by the victim receiver. Note that when defining the resilience levels of a receiver, Iin-ch is often referred to as Iadj-ch or Iblk depending on the frequency offset between the useful and interfering signals.

The received interfering signal power causing a receiver desensitisation of M can be calculated as follows, where N is the noise floor of the receiver:

		

		[bookmark: _Ref129086228][bookmark: _Ref127785561](30)





By equating Equations (29) and (30) :

		

		[bookmark: _Ref127785563](31)





The above equation can be rewritten as:

		

		[bookmark: _Ref131513025](32)





and then:

		

		(33)





Transforming from the linear domain to the logarithmic domain results in:

		

		(34)





As the values of N, M, ILR and FOS are always defined in the logarithmic domain the above equation can be rewritten as:

		

		(35)





which is the basic equation of MRR.

Note that the receiver frequency offset selectivity can also be expressed as a function of N, M, Iin-ch and ILR from Equation (32):

		

		(36)





and:

		

		(37)





then:

		

		(38)





Transforming from the linear domain to the logarithmic domain results in:

		

		(39)





and finally:

		

		(40)





[bookmark: _Toc129783829][bookmark: _Toc129783910][bookmark: _Toc129955119][bookmark: _Toc129955998]Note that when defining the resilience levels of a receiver, Iin-ch is often referred to as Iadj-ch or Iblk depending on the frequency offset between the useful and interfering signals.

[bookmark: _Toc129783830][bookmark: _Toc129783911][bookmark: _Toc129955120][bookmark: _Toc129955999][bookmark: _Toc107486750][bookmark: _Toc129956000][bookmark: _Toc115888597][bookmark: _Toc155274006]Examples of application of MRR

Application of MRR where receiver blocking level is independent of the ILR for the requirement

In a real-life interference scenario, the desensitisation of the receiver will depend on both ILR and FOS as shown in ANNEX 1:, leading to the MRR for deriving receiver blocking described in section 4. In a compatibility scenario the impact of both FOS and ILR is important. However, it may be desirable to define a receiver requirement based only on FOS for the cases where the specific ILR of the interfering signal used in a harmonised standard is difficult to determine. A sufficiently high ILR that would not impact the requirement should then be assumed. Achieving adequate ACLR/ILR on test signals is important to reduce the impact on I/C measurements. The ideal configuration uses separate signal generators for the wanted and interferer signals which also allow a band pass filter to be added to the interference path to further improve the ACLR/ILR.

While some harmonised standards only imply such requirements on the interfering signal through the way receiver requirements are defined, other standards have explicit requirements on interfering signals. One example is EN 303 340 [4], which stresses the importance of improving ACLR in informative Annex D.3 and sets normative requirements for minimum ACLR of the interfering signal in informative Annex F.

In certain scenarios, the choice of a FOS value at least equal or even considerably higher than ILR, may be reasonable in order that the FOS is not the limiting in the compatibility scenario. If FOS >> ILR is assumed, Step 5 of MRR boils down to:

		

		(41)





Such a choice is clearly not desirable, since the blocking level then depends very little on the desired receiver resilience (FOS), but almost completely on the ILR of the interfering signal. Such a requirement is not very useful.

Note that the same reasoning (but reversed) can be made for transmitters. Defining a transmitter requirement using a receiver with a non-adequate FOS would impair the definition of the requirement and make the result depend mostly on the receiver impairments. Clearly the limits in ERC Recommendation 74-01 [2] are not defined that way, but rather assume a sufficiently good spectrum analyser to detect the unwanted emissions independent of receiver impairments.

The solution would be to separate the assumptions of the compatibility scenario from the requirement scenario. In the compatibility scenario, FOS and ILR are analysed to make an informed choice of FOS for the receiver according to Step 4 of Method MRR, for example:

1. FOS based on “balance” between Tx and Rx requirements: FOS (dB) = ILR (dB);

FOS defined with some margin above the impairment from unwanted emissions, in cases where it is desirable to not be limited by the receiver: FOS (dB) > ILR (dB).

FOS defined with some margin below the impairment from unwanted emissions, in cases where it is desirable to not be limited by the transmitter, e.g. when the allowed complexity is much higher for the transmitter than for the receiver:  FOS (dB) < ILR (dB).



Once FOS is selected, the blocking requirement is defined using a different scenario, ensuring that ILR >> FOS. It is reasonable to assume that ILR is infinite and has no impact, leading to a blocking level in Step 5 of MRR independent of the ILR of the interfering (test) signal:

		

		(42)





Where:

N: Noise floor of the victim receiver (dBm);

M: Victim receiver desensitisation at a given frequency offset (dB);

FOS: Frequency offset selectivity of the victim receiver (dB);

ILR: Leakage power ratio of the interfering signal at offset frequency (dB).

Generic application of MRR for specifying recommended levels of receivers resilience to transmission on adjacent frequency ranges in ETSI Harmonised Standards

MRR is proposed to be used for specifying recommended levels of receiver resilience to transmission on adjacent frequency ranges. The proposed method is technology neutral and uses N, M, ILR and FOS to derive the receivers blocking level by a single equation derived from the basic equations describing the receiver resilience mechanism (see Section 3).

This Annex presents the results of in-depth analysis carried out:

to understand how receivers adjacent channel selectivity and blocking requirements are defined in different ETSI harmonised standards;

to check if the basic equation of MRR can be used to derive the blocking requirements defined in harmonised standards;

to check if the basic equation of MRR can be used to verify the consistency between different requirements defined in harmonised standards;

to find out how to choose Rx desensitization value (M) when calculating Iblk.

MRR consists of deriving the receiver blocking level (Iblk) or the carrier to interference blocking level ratio (), for a given acceptable receiver desensitization “M”, from the equation below:

		

		(43)





Or, in terms of  ratio:

		



		(44)





Where:

N: Noise floor of the victim receiver (dBm); 

M: Victim receiver maximum acceptable desensitisation at a given frequency offset (dB);

FOS: Frequency offset selectivity of the victim receiver (dB);

ILR: Leakage power ratio of the interfering signal at offset frequency (dB);

Csens: receiver sensitivity level, specified for the victim receiver.

Note that even if the basic equation is proposed to calculate Iblk, the calculated Iblk is simply Iadj in the case of adjacent channel interference and Ico-ch in the case of co-channel interference, as far as the victim receiver operates in its linear range. This is demonstrated in the analysis presented in this document.

MRR has been used in the analysis presented in this document. No circular calculations have been made in the analysis. This means that the blocking requirements defined in harmonised standard were not used to calculate ILR or FOS needed to derive the receiver blocking level by using MRR.

Receiver blocking requirements defined in different ETSI Harmonised Standards and applicability of MRR to determine the receiver blocking levels

[bookmark: _Hlk83654985]Four ETSI Harmonised Standards have been analysed to understand how the receiver resilience requirements (FOS and Iblk) are defined in these standards. The analysed harmonised standards cover Broadcasting, Mobile, Fixed services and GSM respectively:

1. ETSI EN 303 340 V1.2.1: Digital Terrestrial TV Broadcast Receivers; Harmonised Standard for access to radio spectrum [4];

ETSI EN 301 908-14 V13.1.1: IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 14: Evolved Universal Terrestrial Radio Access (E-UTRA) Base Stations (BS) [8];

ETSI EN 302 217-2 V3.3.1: Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 2: Digital systems operating in frequency bands from 1 GHz to 86 GHz; Harmonised Standard for access to radio spectrum [9];

ETSI EN 301 502 V12.5.1: Global System for Mobile communications (GSM); Base Station (BS) equipment; Harmonised Standard covering the essential requirements of article 3.2 of the Directive 2014/53/EU [10].

Harmonised Standard ETSI EN 303 340 “Digital Terrestrial TV Broadcast Receivers”

Note: the ACLR (or ILR) mentioned in this section refers to the ACLR (or ILR) of the interfering test signal.

General comments

FOS and ILR defined in ECC Report 310 [1] have not been taken into account in ETSI EN 303 340 [4] yet. Instead, ACS (adjacent channel selectivity) and ACLR (adjacent channel leakage ratio) are used independently from the frequency offset between the useful and interfering signals in ETSI EN 303 340.

Moreover, the term ACS used in this harmonised standard does not have the same meaning as the term ACS defined in ECC Report 310 and in compatibility studies carried out within CEPT. The term ACS used in ETSI EN 303 340 is equivalent to the measured I/C ratio of the receiver under test.

[bookmark: _Hlk70517684]The receivers ACS and blocking level (Iblk) are defined in the presence of an interfering signal with a minimum ACLR value (ACLRmin) with the assumption ACS = ACLRmin, which is normative (see ETSI EN 303 340 Annex F). This assumption implies that the OOBE of the interfering signal received by the receiver under test is not negligible and consequently will impact the compliance measurement results. However, note that ACLRmin means that the value of ACLR used in the measurement can be higher than the value of ACLRmin. This is explicit in ETSI EN 303 340 informative Annex D/D3, where it is proposed to improve the interfering signal ACLR to minimise its impact on measurement results.

The above findings are summarised in Table 1:.

[bookmark: _Ref131270960][bookmark: _Ref129085590][bookmark: _Ref127782759]Table 1: Derived ACLR values from ETSI EN 303 340 V1.2.1: Digital Terrestrial TV Broadcast Receivers; Harmonised Standard for access to radio spectrum [4]

		Victim receiver

		Frequency offset

		Requirement

		Type of interfering signal

		ACLR requirement for the interfering signal (Note 1)

		Assumption used to define blocking requirements (Note 1) (Note 2)

		ACLR proposed for conformance testing (Note 1) (Note 3)



		DVB-T/T
(8 MHz)

		8 MHz

		Receiver ACS (Iadj/C)

		DVB-T (8 MHz)

		43 dB (ACLRmin)

		ACS = ACLRmin

		ACLR ≥ ACLRmin



		DVB-T/T2
(8 MHz)

		8 MHz

		Receiver ACS (Iadj/C)

		DVB-T (8 MHz)

		47 dB (ACLRmin)

		ACS = ACLRmin

		ACLR ≥ ACLRmin



		DVB-T 
(8 MHz)

		10 MHz

		Receiver ACS (Iadj/C)

		10 MHz LTE 700 MHz BS light load (near idle)

		53 dB (ACLRmin)

		ACS = ACLRmin

		ACLR ≥ ACLRmin



		DVB-T 
(8 MHz)

		73 MHz

		Receiver ACS (Iadj/C)

		10 MHz LTE 700 MHz BS light load (near idle)

		61 dB (ACLRmin)

		ACS = ACLRmin

		ACLR ≥ ACLRmin



		DVB-T2 (8 MHz)

		10 MHz

		Receiver ACS (Iadj/C)

		10 MHz LTE 700 MHz BS light load (near idle)

		58dB (ACLRmin)

		ACS = ACLRmin

		ACLR ≥ ACLRmin



		DVB-T2 (8 MHz)

		73 MHz

		Receiver ACS (Iadj/C)

		10 MHz LTE 700 MHz BS light load (near idle)

		65 dB (ACLRmin)

		ACS = ACLRmin

		ACLR ≥ ACLRmin



		DVB-T 
(8 MHz)

		73 MHz

		Receiver blocking level (Iblk)

		10 MHz LTE 700 MHz BS fully loaded

		64 dB (ACLRmin)

		ACS = ACLRmin

		ACLR ≥ ACLRmin



		DVB-T2 (8 MHz)

		73 MHz

		Receiver blocking level (Iblk)

		10 MHz LTE 700 MHz BS fully loaded

		66 dB (ACLRmin)

		ACS = ACLRmin

		ACLR ≥ ACLRmin



		Note 1: The ACLR mentioned in this table refers to the ACLR of the interfering test signal

Note 2: The required minimum ACLR level depends on the magnitude of I/C values to be measured. Minimum ACLR values to just pass the tests in the present document are provided in table F.1. These values assume a 3 dB ACLR degradation contributing to on the measured performance (see ETSI EN 303 340 V1.2.1 Annex F (Normative).

Note 3: In Annex D/D3 (informative) it is proposed to improve the interfering signal ACLR to minimise its impact on measurement results.





Also note the measured DTT receiver sensitivities, and not those defined in ETSI EN 303 340 have been used - in all the calculations presented in the following sections. This is justified by the fact that the DTT receiver sensitivities defined in ETSI EN 303 340 are far above the measured (actual) DTT receiver sensitivities (see ECC Report 310).

Applicability of MRR to determine the receiver blocking levels defined in ETSI EN 303 340

The applicability of MRR to determine the receiver selectivity (Iadj-ch/C) and blocking levels (Iblk) defined in ETSI EN 303 340 [4] has been checked. The calculations have been made for two cases:

FOS = ILR = ACLRmin;

FOS = ACLRmin and ILR = 100 dB (ILR>>FOS).

When verifying the applicability of MRR to determine the receiver blocking levels defined in ETSI EN 303 340, no calculations have been made when the values of FOS (or ACS) and ILR (or ACLR) are not defined/provided and they cannot be calculated independently from the Iblk defined in the harmonised standard.

Results of the calculations

The results of the calculations , showed that for DVB-T/T2 receivers the C/ Iadj-ch and Iblk derived by using MRR are quite close to those defined in ETSI EN 303 340 [4], the difference being less than 2 dB, except 4 values (over 16) where the value of FOS used in the calculations was different than ILR. This can be explained by the fact that in ETSI EN 303 340 C/Iadj-ch and Iblk are defined with the assumption of ACS (FOS)=ACLRmin (ILR).

The value of M to be used in the conformity tests is not directly defined in ETSI EN 303 340. Nevertheless it can easily be calculated as follows:

		



		(45)





The calculated values of M are about 13 and 14 dB for receivers blocking tests and vary from 25 to 34 dB for receivers selectivity (C/ Iadj-ch) tests.

Harmonised Standard ETSI EN 301 908-14 [8] “IMT cellular networks”

Note: the ACLR (or ILR) mentioned in this section refers to the ACLR (or ILR) of the interfering test signal.

General comments

Note that while the receiver selectivity requirement in the harmonised standard is called ACS, it is expressed with an interferer level and not with an ACS value as used in compatibility studies within CEPT (see ECC Report 310 [1]). The term ACS used in ETSI EN 301 908-14 [8] is equivalent to the measured interfering signal level (Iadj).

The receivers ACS and blocking level (Iblk) are defined in the presence of a 5 MHz E-UTRA interfering signal for frequency offsets lower than 20 MHz and a CW interfering signal for frequency offsets higher than 20 MHz from the uplink operating band. There is no explicit requirement for interfering signal ACLR in ETSI EN 301 908-14. Nevertheless, the ACLRmin of the interfering signal to be used for conformance testing should be equal to ACS+10.2 dB (see ETSI TS 136 141 [12]). This condition implies that the OOBE of the interfering signal received by the receiver under test are very low and consequently will barely impact the compliance measurement results (0.4 dB additional rise in interference,).

In the case of ACS and narrow-band blocking, the ACLR values to be used in compliance measurements are defined in ETSI TS 136 14. ACLR is not applicable to CW interfering signal.

The above findings are summarised in Table 2:.

[bookmark: _Ref131270964][bookmark: _Ref129085558][bookmark: _Ref127781250]Table 2: ACLR values derived from ETSI EN 301 908-14 V13.1.1: IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 14: Evolved Universal Terrestrial Radio Access (E-UTRA) Base Stations (BS)

		Victim receiver (10 MHz)

		Frequency offset

		Require-ment

		Type of interfering signal

		ACLR requirement for the interfering signal 
(Note 1)

		Assumption on ACS/ACLR used to define blocking requirements (Note 1) 
(Note 2)
(Note 3)

		ACLR requirement for conformance testing
(Note 1) 
(Note 2)
(Note 3)



		Wide Area IMT BS

		±2.5075 MHz

		Receiver ACS (Iadj)

		5 MHz E-UTRA signal

		No requirement

		ACLRmin =56 dB
ACS= ACLRmin -10.2 dB=45.8 dB

		ACLRmin=56 dB
ACLRmin=ACS+10.2 dB



		Wide Area IMT BS

		±(347.5 + m*180),
m = 0, 1, 2, 3, 4, 9, 14, 19, 24

		Receiver narrowband blocking level (Iblk)

		5 MHz E-UTRA signal, 1 RB (Note 4))

		No requirement

		ACLRmin =59 dB
ACS= ACLRmin-10.2 dB=48.8 dB

		ACLRmin=59 dB
ACLRmin=ACS+10.2 dB



		Wide Area IMT BS

		(FUL_low-20) to (FUL_high+ 20) (Note 5)

		Receiver blocking level (Iblk)

		5 MHz E-UTRA signal

		No requirement

		ACLRmin =ACS+10.2 dB

		ACLRmin =ACS+10.2 dB



		Wide Area IMT BS

		1 to (FUL_low-20)
(FUL_low+20) to (FUL_high+ 12750) 
(Note 5)

		Receiver blocking level (Iblk)

		CW signal

		No requirement

		ACLR not applicable

		ACLR not applicable



		Note 1:The ACLR mentioned in this table refers to the ACLR of the interfering test signal.

Note 2: The contribution from the Test equipment ACLR is calculated to give a 0.4dB additional rise in interference. This corresponds to a Test equipment ACLR which is 10.2 dB better than the BS ACS (see ETSI TS 136 141 Table 4.1.2-2, 7.5 Adjacent Channel Selectivity (ACS) and narrow-band blocking; Note 2: d and e)

Note 3: The contribution from the Test equipment ACLR is calculated to give a 0.4dB additional rise in interference (see ETSI TS 136 141 Table 4.1.2-2, 7.6.5.1 Blocking (General requirements))

Note 4: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. Frequency offsets are such that the interfering signal is outside the channel

Note 5: FUL_low and FUL_high are the lowest and highest frequencies of the uplink operating band







Applicability of MRR to determine the receiver blocking levels defined in ETSI EN 301 908-14

The applicability of MRR to determine the receiver blocking levels (Iblk) defined in ETSI EN 301 908-14 [8] has been checked. The calculations were carried out for two cases:

FOS = ILR = ACLRmin;

FOS = ACLRmin and ILR = 100 dB (ILR>>FOS).

The ACS of the victim receiver being defined by the interfering signal level Iadj, this level has also been calculated by using the basic equation of MRR.

When verifying the applicability of MRR to determine the receiver blocking levels defined in ETSI EN 301 908-14, no calculations have been made in when the values of FOS (or ACS) and ILR (or ACLR) are not defined/provided and they cannot be calculated independently from the Iadj or Iblk defined in the harmonised standard. This is the case for the receiver blocking level tests.

In this section and the following section calculations have only been performed for 1.4 MHz, 5 MHz and 10 MHz Wide Area IMT BS receivers.

Results of the calculations

The results of the calculations, showed that for 1.4, 5 and 10 MHz Wide Area IMT BS receiver the Iadj-ch and narrowband Iblk derived by using MRR are very close to those defined in ETSI EN 301 908. Their difference is less than 1 dB. Iblk cannot be calculated since there is no information on the ACS and ACLR values used to define this parameter in ETSI EN 301 908 [8] and TS 136 141 [12].

The value of M to be used in the conformity tests is defined in ETSI EN 301 908 [8]. It varies from 6 to 27 dB depending on the channel BW and type of the IMT BS under test.

Harmonised Standard ETSI EN 302 217-2 [9] “Fixed Radio Systems”

Note: the ACLR (or ILR) mentioned in this section refers to the ACLR (or ILR) of the interfering test signal.

General comments

Neither ACS or FOS nor ACLR or ILR are used in ETSI EN 302 217-2 [9].

In this harmonised standard, the receiver selectivity is specified in terms of receiver sensitivity degradation (i.e. equal to M) in presence of a wanted signal like interfering signal of predefined protection ratio (PR=C/Iref) in the adjacent channels and in the presence of a generic unmodulated (CW interference) signal anywhere in a large portion of the spurious domain (blocking and spurious response requirement). Then, a kind of wide-band selectivity (WBSEL) is derived (i.e. based on the NFD method for FOS assessing referred in Section A4.1 of this document) by comparing the adjacent channel C/Iadj-ch ratios, or the C/Iblk at given BER threshold degradation (M) to the co-channel C/Ico-ch ratio producing the same BER degradation.

In the presence of a wanted signal like-modulated interfering signal up to 2nd adjacent channel:

		

		(46)





and, in the presence of a CW interfering signals beyond 2nd adjacent channel.

		



		(47)





Note that when defining the resilience levels of a receiver, Iin-ch is often referred as Iadj-ch or Iblk depending on the frequency offset between the useful and interfering signals.

It is considered that this kind of wide-band selectivity (WBSEL) response is comprehensive of all effects (linear and not linear) that define the overall response of the receiver to interference; therefore, it is intended as the real selectivity of the digital receiver.

It is worth noting that WBSEL is equivalent to RIR (receiver interference ratio) or ACIR (adjacent channel interference ratio) within the adjacent channel frequency ranges (see ECC Report 310 [1]). Note also that for ILR>>FOS, WBSEL is equal to FOS. Consequently, it can be safely assumed that , in the presence of CW interfering signals beyond 2nd adjacent channel.

As WBSEL is directly linked to the PR of digital fixed radio systems (DFRS) receivers, for the sake of simplicity, MRR has been be used to derive C/Ico-ch, C/Iadj-ch and C/Iblk defined in ETSI EN 302 217-2.

The above findings are summarised in Table 3:.

[bookmark: _Ref129085562][bookmark: _Ref127781436]Table 3: ILR values derived from ETSI EN 302 217-2 V3.3.1: Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 2: Digital systems operating in frequency bands from 1 GHz to 86 GHz; Harmonised Standard for access to radio spectrum
 [9]

		Victim receiver

		Frequency offset

		Requirement

		Type of interfering signal

		Assumption on ILR requirements (Note 1)

		Assumption on FOS/ILR used to define blocking requirements (Note 1)

		ILR requirements for conformance testing 
(Note 1)



		Digital Fixed Radio Systems (DFRS)

		0 to 2nd adjacent channel

		Selectivity (C/Ico-ch-C/Iadj-ch)

		Wanted signal like-modulated interfering signal

		Wanted signal like-ILR (Note 2)

		FOS=ILR (1st adjacent)
FOS ≥ ILR+ 10 dB (2nd adjacent)

		No requirements



		Digital Fixed Radio Systems (DFRS)

		beyond 2nd adjacent channel

		Selectivity (C/Ico-ch-C/ICW) (Note 3)

		CW carrier

		No requirements

		ILR >> FOS

		No requirements



		Note 1: The ILR mentioned in this section refers to the ILR of the interfering test signal

Note 2: Derived from the spectrum mask requirement, tested up to the 2nd adjacent channel included, in ETSI 302 217-2 V3.3.1 (see also ANNEX 3: of this document)

Note 3: WBSELCW range evaluated as C/IC - C/ICW; requirement in clause 4.3.3.3; value is valid from ±3rd CS (as centre frequency of the channel fully within the CW requirement range) and up to the frequency range where the CW test is defined by clause 7 of ETSI EN 301 390 [13]. It should also be understood that WBSELCW value is applicable on real interference environment only if the interfering signal emission exhibits a corresponding reduction of its OOB and spurious emissions within the victim DFRS RX bandwidth (see ETSI EN 302 217-2 V3.3.1, Figure P.1, NOTE 4).







Applicability of the basic equation of MRR for determining the receiver protection ratios defined in ETSI EN 302 217-2

The applicability of MRR for determining the PR (C/Ico-ch, C/Iadj-ch and C/Iblk) of receiver defined in ETSI EN 302 217-2 [9] has been checked. The calculations were carried out for two cases:

in the presence of a co- channel wanted signal like-modulated interfering signal. In this case Equation (1) was simplified as FOS and ILR have no impact on the calculation of Ico-ch:

		

		(48)





in the presence of a 1st adjacent channel wanted signal like-modulated interfering signal. In this case there is no information on receivers FOS in ETSI EN 302 217-2. Nevertheless, with the interfering signal as a wanted signal like-modulated signal, it is sensibly assumed that FOS=ILR (or equivalently ACS=ACLR).

When verifying the applicability of MRR to determine the receiver blocking levels defined in ETSI EN 302 217-2, no calculations have been made when the values of FOS (or ACS) and ILR (or ACLR) are not defined/provided and they cannot be calculated independently from the Iblk defined in the harmonised standard. This is the case for the receiver blocking level tests with a wanted signal like-modulated interfering signal at the 2nd adjacent channel and in the presence of a CW interfering signal beyond the 2nd adjacent channel.

In this section, calculations have only been performed for 7 MHz and 14 MHz channels DFRS Class 2 receivers operating respectively in the bands 3.5 GHz and 26 GHz have been considered. On the other hand, the spectral emission requirements (spectrum mask) depend on the channel size and are equal for frequency bands from 3.5 to 57 GHz; in addition, DFRS modulation-demodulation techniques, optimised for best S/N, apply the Nyquist principle of equal split between Tx and Rx of the overall square-cosine pulse shaping filtering (rendering parametric as well the RX filtering). Therefore, the results can be intended valid for any channels size and frequency bands.

Results of the calculations

The results of the calculations , show that in the 3.5 and 26 GHz bands the 7 and 14 MHz DFRS receiver C/Ico­ch, and C/Iadj-ch derived by using MRR are close to those defined in ETSI EN 301 390 [13]. Their difference is less than 3 dB.

Two single M values, 1 and 3 dB, are used in the conformity tests defined in ETSI EN 302 217-2 [9].

Harmonised Standard ETSI EN 301 502 [10] “Global System for Mobile communications (GSM)”

Note: the ACLR (or ILR) mentioned in this section refers to the ACLR (or ILR) of the interfering test signal.

 General comments

Neither ACS or FOS nor ACLR or ILR are used in ETSI EN 301 502 [10]. 

In this harmonised standard, the receiver selectivity is specified in terms of receiver sensitivity degradation in presence of a wanted signal like interfering signal of predefined protection ratio (PR=C/Iref) in the adjacent channels (1st and 2nd). The blocking level (Iblk) is defined beyond the 2nd adjacent channel and in a large portion of the spurious domain (blocking and spurious response requirement) in the presence of a generic unmodulated (CW interference) signal. Consequently, MRR has been used to derive C/Ico-ch, C/Iadj-ch and Iblk defined in ETSI EN 301 502.

The above findings are summarised in the following table.

Table 4: ILR values derived from ETSI EN 301 502 V12.5.1: Global System for Mobile communications (GSM); Base Station (BS) equipment; Harmonised Standard covering the essential requirements of article 3.2 of the Directive 2014/53/EU [10]

		



		Victim receiver

		Frequency offset

		Requirement

		Type of interfering signal

		Assumption on ILR requirements (Note 1)

		Assumption on FOS/ILR used to define blocking requirements (Note 1)

		ILR requirements for conformance testing
(Note 1)



		GSM 400/GSM 900/DCS 1800 BTS 

		0 to 2nd adjacent channel

		Selectivity (Note 2) (C/Iadj-ch); M=20 dB

		Wanted signal like-modulated interfering signal

		Wanted signal like-ILR

		FOS=ILR for 1st adjacent channel

		No requirements



		GSM 400/GSM 900/DCS 1800 BTS

		Beyond 2nd adjacent channel

		Blocking (Iblk); M=3 dB

		CW signal

		No information provided

		No information provided

		No requirements



		Note 1: the ILR mentioned in this table refers to the ILR of the interfering test signal

Note 2: Adjacent channel interference rejections for circuit switched channels except Enhanced Circuit-Switched Data (ECSD)





[bookmark: _Hlk83738992]It is also important to note here that no information has been found on GSM BS noise figure (NF) as well as SNR in ETSI HS or TS. A GSM BS NF of 8 dB is used in ECC Report 146 [14]. This value has also been used in this document to calculate the noise floor of GSM receivers.

 Applicability of the basic equation of MRR for determining the receiver protection ratios defined in ETSI EN 301 502

The applicability of MRR for determining the PR (C/Ico-ch, C/ Iadj-ch and C/ Iblk) of receiver defined in ETSI EN 301 502 [10] has been checked. The calculations were carried out for two cases:

in the presence of a co- channel wanted signal like-modulated interfering signal. In this case Equation 1 was simplified as FOS and ILR have no impact on the calculation of Ico-ch:

		

		(49)





					3

in the presence of a 1st adj channel wanted signal like-modulated interfering signal as well as in the presence of a 2nd adj channel wanted signal like-modulated interfering signal. In these cases, there is no information on receivers FOS in ETSI EN 301 502. Nevertheless, GSM BS 1st and 2nd adjacent channels FOS values provided in ECC Report 146 [14] has been used in the calculation. The interfering signal being wanted signal like-modulated signal, it is sensibly assumed that FOS=ILR in the case of 1st adjacent channel interference.

When verifying the applicability of MRR to determine the receiver blocking levels defined in ETSI EN 301 502, no calculations have been made when the values of FOS (or ACS) and ILR (or ACLR) are not defined/provided and they cannot be calculated independently from the Iblk defined in the harmonised standard. This was the case for the receiver blocking level tests in the presence of a CW interfering signal beyond the 2nd adjacent channel.

In this section, calculations have only been performed for GSM 400/GSM 900/ER-GSM 900/DCS 1 800 Macro-BTS (GMSK modulated carrier) type receivers have been considered.

Results of the calculations

The results of the calculations show that for 200 kHz GSM BST receiver the C/Ico-ch and C/Iadj-ch derived by using MRR are very close to those defined in ETSI EN 301 502 [10]. Their difference is less than 1 dB, except the two C/Iadj-ch calculated with assumption of ACS=ACLR. In these latter cases the difference is 3 dB. This difference is probably due to the assumption made on the GSM ACS in ECC Report 146. It seems that in ECC Report 146 [14] the GSM ACS values were derived as follows:

		

		(50)





which is RIR (dB) and not ACS (dB) unless ILR>>FOS.

RIR is equal to FOS if and only if ILR>>FOS. Consequently, it can be safely assumed that  in the presence of CW interfering signals beyond the 2nd adjacent channel.

The value of M to be used in the conformity tests is defined in ETSI EN 301 502 [10]. They are 3 and 20 dB respectively for blocking and selectivity tests.

Conclusions

The results of the in-depth analysis carried out on four ETSI Harmonised Standards show that, when accurate information on FOS and ILR is provided in ETSI harmonised standards, the receiver blocking level (Iblk) or the carrier to interference blocking level ratio C/ Iblk derived by MRR are very close to those defined in HS. This confirms that MRR is an accurate and robust method for deriving blocking levels of receivers for the future recommendation on receivers. Moreover, the method can also be used to check the consistency between different requirements defined in HS;

The main obstacle to using MRR to check the relevance of the receiver blocking requirements defined in HS is the lack of information on the ACS and ACLR values used to define those requirements. It would be sensible that this important information is provided in HS.

[bookmark: _Toc129956001][bookmark: _Ref107397296][bookmark: _Ref107397472][bookmark: _Ref107397549][bookmark: _Ref107397564][bookmark: _Toc115888598][bookmark: _Toc155274007]General guidance on parameters M, N, ILR and FOS

Introduction

Administrations have a duty to ensure efficient use of spectrum. There needs to be sufficient and complete information to assess the ability of services to coexist both on the same and adjacent frequencies and bands. This is an important consideration when services are seeking spectrum to use and applies to both new and existing services.

This need was less prevalent in previous years where spectrum was more sparsely used. Administrations now face the challenge of stretching spectrum utilisation with greater numbers of users. Administrations require accurate information on the relevant parameters for the receive parts of services in addition to the transmission radio emission characteristics.

All the following needs to be considered when defining the representative values for receiver parameters. All these considerations relate to the overall design of a radio receiver including for example, sensitivity, selectivity, desensitization, blocking and noise performance. To achieve efficient spectrum sharing, services should only use sufficient signal power levels which provide a satisfactory quality of required operation. This required quality of operation will vary: some services absolutely must work immediately during an emergency while others may require a less reliable service where a message is delivered even after a long delay.

A common way to overcome interference is to increase the transmitter power levels for the wanted service to increase the C/I ratio. However, this will increase the likelihood of interference to other users driving higher signal level requirements into other systems. This is clearly not energy-efficient neither cost-effective in the long term. 

Some services operate with a very low field strength. This implies that services in adjacent frequencies should operate at very low levels of unwanted emissions to avoid interference. Furthermore, very sensitive receivers need to be able to avoid early overload effects if its receiver filtering does not remove all the adjacent signals. 

The table below is an extract from ECC Recommendation (02)01 [15] and gives an overview of the negative effects on spectrum use and compatibility when the performance of receivers is poor in one or several of its characteristics:

Table 5: Impact of Reference receiver performance parameters on spectrum utilisation and efficiency of radio equipment 

		Reference receiver performance parameters 

		Impact on spectrum utilisation and efficiency of radio equipment with poor receiver performance parameters



		Sensitivity

		Increase of number of transmitters (base stations)

Increase of transmitter power

Increased spectrum demand if number of transmitters and transmitter powers cannot be changed

Increased difficulty to elaborate channel plans which leads to more interference to other services and system capacity loss and therefore an inefficient spectrum use



		Blocking, desensitisation, spurious response, protection ratio, co-channel rejection, receiving mask, selectivity and adjacent band rejection

		Decrease of number of transmitters of the interfering service and

Decrease of transmitter power of the interfering service which leads to system capacity loss for the interfering service and consequently more spectrum for the other service and increase of the interfering probability to the wanted radio service.





Radio services very often have the capacity to dynamically adapt the transmission modulation and operating characteristics based on the radio channel environment. Where a service has an adaptive capability, its technical specification needs to provide sufficient detail about the equipment state during each individual measurement of the dynamic adaptation (including modulation, throughput, etc.). Lack of clarity in the technical details for each measurement will result in difficulties for carrying out compatibility studies. The characterising documentation should provide this additional information alongside the values measured in each case.

Similarly, the lowest signal level at which the radio service can or is designed to work at usually conveys a low capability (for example, low data rate streams or low audio quality telephony codecs). Higher capability, e.g. throughput, is usually only achieved at higher quality received signal levels. Clarity around received signal levels and system performance is needed in the documented values in order to facilitate administrations sharing and compatibility studies requirements. They should not be masked inside a large margin or, for example by omitting the margin at a particular measurement to describe a failure point. These values need to have clear explicit additional notes indicating the performance capability or operating state for each measurement and if they incorporate a safety margin or are to the point of full failure.

Administrations also need to consider the expectation of equipment users. Measurements lacking clarity on which modulation mode is in use and that mode’s capability, can also make sharing and compatibility study calculations very difficult without additional contextual information. This, for example, may then require additional bench measurement campaigns to supplement the available documentation. Clarity in how a service/application equipment works as a system is required at least for the RF air interface capability where dynamic mode changes may occur in both transmitters and receivers.

In summary, both the environment and the whole set of equipment performance parameters need to be considered in the assessment, with existing examples of services and services as well as new ones. The receiver noise figure, sensitivity, dynamic range and its signal filtering all play a part, and not just the filtering alone. Similarly, the filtered final emission characteristics of the transmissions also directly interact in the assessment.

Overview of relevant receiver parameters to consider in the methodology

This section includes details about how the four different receiver parameters under discussion (M, N, ILR and FOS) are interlinked in the context of receiver resilience against harmful interference.  

As defined in Section 3.1 of this Report, the interfering signal power received by the victim receiver is, in the logarithmic domain:

		



		[bookmark: _Ref129246599](51)







Where:

N is the receiver noise floor, defined in Section 2.1 of ECC Report 310 [1] as the total noise power at the receiver including the effect of thermal noise and the receiver noise figure and

M is the receiver desensitisation, as defined in Section 2 of ECC Report 310, the reduction in the signal to noise ratio of the receiver or a reduction in the effective sensitivity in the presence of an interfering signal, given in dB. It corresponds to the ‘noise rise’ due to the interfering signal.

When the interfering signal is adjacent to the victim, the signal power received by the victim is also defined as:

		

		[bookmark: _Ref129246608](52)





Where, as defined in Section 2.1 of ECC Report 310:

Frequency Offset Selectivity (FOS (dB)) is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an interfering adjacent signal at a given frequency offset from the centre frequency of the assigned channel. It is most often defined as the ratio of the receiver filter attenuation on the offset frequency to the receiver filter attenuation on the assigned channel frequency (normally a positive number in dB). FOS is of general use for any mixed wanted and interfering signal situation;

Interference Leakage Ratio (ILR (dB)) is the ratio of the (nominally rectangular) filtered mean power centred on the assigned channel frequency to the similarly filtered mean power centred on a given frequency offset.

Equating (51) and (52), the four parameters are related to the interference signal as:

		



		(53)







Receiver noise floor (N)

Every radio receiver is subject to a noise floor that can be described using the following equation and expressed in dB[footnoteRef:2]: [2:  Also defined in ECC Report 252 (SEAMCAT handbook) as the level of noise introduced by the receiver system below which the signal that is being captured cannot be isolated from the noise (see ECC Report 252, Section 1.2.3) [16])] 


		

		(54)





Where: 

N is the receiver noise floor;

k is the Boltzmann constant in Joules per Kelvin (1.381×10-23);

T is the temperature in degrees Kelvin (for common terrestrial radio receivers, 290 K can be used);

B is the receiver bandwidth in Hertz;

kTB is the receiver thermal noise in dBW;

NF is the receiver Noise Figure in dB.

Compatibility studies require a figure for, or be able to derive, the composite value of “N”. Without this essential figure, the overall baseline of the systems operating characteristics has no reference. “N” is an aggregate noise value that includes: 

The receiver thermal noise, which is always present from thermal considerations by the classic k, T and the receiver bandwidth (B) and; 

the receiver’s own internal system noise contribution or noise figure (NF).

The NF is mainly defined by the first active device (e.g. an LNA or a simple diode for direct signal conversion/demodulation) of the front end, added to the attenuation of any passive circuitry (e.g. a filter) between the antenna port and that active device itself. The design target of the NF depends on the expected performance of the radio system in terms of the operational range (e.g. maximum distance or cell size diameter between Tx and Rx in the field); therefore, NF target should be balanced by the corresponding Tx EIRP.

Usually the NF is minimised in order to reduce the costlier Tx power; however, in some cases, e.g. in SRDs, the operational range might be significantly low, justifying a high (and cheaper) NF target. Clearly, the higher is the NF and the less sensitive is the Rx to high level of “in-channel” interference (i.e. the same level of interference produces a smaller desensitisation than on a receiver with better NF). When off-channel” interference is considered, the NF plays a lesser role, provided that low NF receivers are designed with better selectivity.

If not known through the receiver characteristics given by the manufacturer, the value of NF may have to be derived from not one, but indirectly from several measurements when testing a particular product. Without the knowledge or a good estimation of NF, there is no baseline reference for any studies.

[bookmark: _Ref107393135][bookmark: _Ref131502901]Receiver desensitisation (M)

General considerations

Elaborating from ANNEX 1: (Equation (30)) the desensitisation M (dB) can be defined as:

		

		(55)





Where:

 is the total interfering signal power (in W) (see Figure 1);

N is the receiver noise floor (in W) measured in the receiver bandwidth. 

To ensure proper operation, the operational range of the radio system is designed to include a margin equal or lower than M which allows it to tolerate a certain level of interference (I) in the listened channel. This can be caused by co-channel and/or non-co-channel interference sources. When running a radio network or a radio link, the objective is to maintain the signal to interference and noise ratio SINR as close as possible to the signal to noise ratio (SNR), in any case within the factor M difference representing the operational margin to interference.

Like N, M is an aggregate value that includes the common protection ratio to ensure a stability of the service reception. This protection ratio in addition to the receiver dynamic performance need to be clearly defined. 

M corresponds to the ‘equivalent noise rise’ due to the interfering signal (in-band and off-band) at the input stages of the receiver. Less predictable Receiver desensitisation also occurs for non-linear effects when a strong off-channel signal overloads a receiver front end and thus reduces the sensitivity to weaker on-channel wanted signals. This effect is caused for example by reciprocal mixing, due to phase noise, and A/D or D/A performance ranges.

Use of large values of additional margin may overprotect a service unnecessarily; low values of additional margin may under-protect a service and make it prone to interference. The proper choice of balanced “N” and “M” values depends on the service requirements. This must include other external effects (for example propagation fading and building signal path loss variations etc) that may require consideration as well as the expectation of the user of the equipment.

In many cases of radio services where stringent QoS (Quality of Service) is required (e.g. in fixed service links), M should not be considered as a “variable”, but as the “maximum acceptable degradation” of the victim receiver due to interference. Its value depends on (or it is related to) the QoS of the specific victim service receiver and, in some cases it is fixed by the ITU-R or other international regulation (e.g. for FS links by ITU-R Recommendation F.1094 [5] and its practical application guidelines in Recommendation F.758 [6]).

Practical approach

The desensitisation values (M) used for receivers blocking conformity tests in the following ETSI Harmonised Standards are presented in Table 6:

ETSI EN 303 340 V1.2.1: Digital Terrestrial TV Broadcast Receivers; Harmonised Standard for access to radio spectrum [4];

ETSI EN 301 908-14 V13.1.1: IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 14: Evolved Universal Terrestrial Radio Access (E-UTRA) Base Stations (BS) [8];

ETSI EN 302 217-2 V3.3.1: Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 2: Digital systems operating in frequency bands from 1 GHz to 86 GHz; Harmonised Standard for access to radio spectrum [9];

ETSI EN 301 502 V12.5.1: Global System for Mobile communications (GSM); Base Station (BS) equipment; Harmonised Standard covering the essential requirements of article 3.2 of the Directive 2014/53/EU [10].

[bookmark: _Ref127783440]Table 6: Desensitisation values (M) used for blocking conformity tests in the analysed ETSI Harmonised Standards

		Desensitisation values (M) used for blocking conformity tests in the analysed ETSI Harmonised Standards



		Harmonised Standard

		M (dB)



		DTT

		13 and 14



		IMT cellular networks:

- Blocking and NB blocking

- NB blocking for NB-IoT wanted signal

		

6 and 14(1)



6(2), 8, 10, 11 and 12



		Fixed Radio Systems

		1



		GSM

		3



		1. Used only for Home BS which has a sensitivity 8 dB higher than that of other BS categories

2. Used in the majority of the NB-IoT wanted signal blocking tests





As shown in Table 6, the value of M to be used in blocking requirement conformity tests may vary from one HS to another. It may also vary depending on the interference configurations considered, as is the case in the HS dealing with IMT cellular networks. Consequently, the choice of Rx desensitisation value (M) when calculating receiver blocking levels by MRR may seem to be difficult at first, and it is worthwhile to explore this matter in more detail. This is done in the following section.

Variation of blocking level as a function of M

In this section Equation (30) has been used to evaluate the variation of receivers blocking level (Iblk) as a function of the value of M.

[image: Chart, line chart

Description automatically generated]

[bookmark: _Ref127774126]Figure 3: Variation of blocking level as a function of M

Figure 3 shows three different blocking curves, calculated from Equation (30), representing the variation of Iblk as a function of M for three different sets of N, ACS and ACLR values.

These blocking curves show that:

while the value of Iblk depends on N, M, ACS and ACLR, the gradient (or slope) of a blocking curve is identical for all possible sets of N, ACS and ACLR values;

for the values of M greater than 9 dB the gradient of the curve is equal to 1. Consequently, above this value of M the blocking curves can be approximated to unity slope lines;

for the values of M smaller than 9 dB the blocking curve deviates from the unity slope line due to the impact of the receiver noise floor (noise power). Actually, a low M value means a low useful signal level thus an increased impact of noise on the receiver performances, while a high M value means a high useful signal level that reduces the impact of noise on the receiver performances ().

In conclusion, independently from the technology used, the blocking curve of receivers varies, as a function of M, linearly with unity slope for M > 9 dB and non-linearly with a slope depending on M for M < 9 dB. For a given M, the gradient (or slope) of the blocking curve is identical for all possible sets of N, ACS and ACLR values.

Choice of the Rx desensitisation value M

[image: Chart, line chart

Description automatically generated]

[bookmark: _Ref127774148]Figure 4 Choice of the Rx desensitisation value M

As shown in Figure 4 and explained in the previous section, for the values of M greater than 9 dB receivers blocking level varies linearly as a function of M, while for the values of M smaller than 9 dB its variation is not linear due to the impact of the receiver noise floor (noise power). Considering this behaviour, which is identical for all receivers, it would be sensible to define the blocking requirements of a receiver for two different values of M, one below and one above 9 dB.

For example, M=3 dB (historical desensitisation value), which takes into account the impact of the receiver noise floor on Iblk and M=15 dB, which takes into account high useful signal levels where the receiver noise floor does not have any impact on Iblk, might be a reasonable choice. This choice is shown in Figure 4 (red points on the blocking curve). When the blocking level of a receiver is defined with these two values of M, its blocking curve can easily be predicted thanks to the unity slope line.

Consequently, if M is not given or cannot be calculated from  it is proposed to define the blocking requirements of receivers for M = 3 and/or 15 dB in the ECC Recommendation on receivers.

[bookmark: _Ref107397477]Frequency offset selectivity and interference leakage ratio

Consideration of different bandwidths of victim and interferer

The parameters FOS and ILR of the basic equation of MRR implicitly take into account the interfering and victim signal bandwidths as described in this section.

However, when the FOS and ILR are derived from ETSI standards, it might be possible that their definition in the standard is relative to the same bandwidth of the system subject of the standard, either the victim or the interferer. Therefore, care should be taken when such ETSI data are used for compatibility study.

FOS is of general use for any mixed wanted and interfering signal situation and can be calculated as follows:

		

		(56)







Where:

FOS: Frequency Offset Selectivity;

RIR: Receiver Interference Ratio at offset frequency;

ILR: Leakage Power Ratio of the interfering signal at offset frequency.

RIR and ILR can be expressed as follows:

		



		(57)





Where:

I: Interfering signal power measured in its bandwidth;

Ir: Total interfering signal power received by the victim receiver;

IOOB: Interfering signal out-of-block or out-of-band power falling into the victim receiver bandwidth.

The above mathematical equations clearly show that ILR and therefore FOS can be calculated for all type of useful and interfering signal bandwidths. However, if ILR is measured in a bandwidth that is different than the victim receiver bandwidth under consideration, and there is no possibility to carry out new measurements, it is necessary to use an appropriate correction factor to define the ILR in the victim receiver bandwidth.

If ILR is measured in the bandwidth of victim receiver A and will be used to calculate Iblk of victim receiver B at the same frequency offset from the interferer and there is no abrupt variation of the interferer emission mask around this frequency offset, then the ILR in the victim receiver B bandwidth can be expressed as follows:

		

		(58)





Note also that a FOS calculated for a given interferer bandwidth may not be applicable to an interferer that have a different bandwidth, which is also true for the FOS (or ACS) values defined in ETSI HS.

It is also important to note that FOS implies attenuation ratios and not power ratios as in the case of ILR. Consequently, it is not possible to apply any bandwidth ratio correction factor to FOS. For example, if the frequency offset between victim and interferer is very large, then the receiver FOS may be flat. Under such conditions the interfering signal bandwidth will have no effect on the value of FOS. This can also be the case when two interferers with different bandwidth have the same offset frequency from the victim and there is no abrupt variation of the receiver FOS.

In conclusion, the bandwidths of victim and interfering systems in EC/ECC decisions, ECC recommendation, ETSI Harmonised Standards should be carefully used and possibly corrected on case by case.

[bookmark: _Ref128054503]Out-of-band and spurious domains and derivation of ILR value

Out-of-band (OOB) domain (of an emission) is the frequency range, immediately outside the necessary bandwidth but excluding the spurious domain, in which OOB emissions (OOBE) generally predominate. The spurious domain (of an emission) is the frequency range beyond the OOB domain in which spurious emissions generally predominate.

The boundary between the OOB and spurious domains is the separation between the centre frequency of an emission and the beginning of its spurious domain. According to Recommendations ITU-R SM.329-12 [7] and Appendix 3 of the Radio Regulations, the boundary between the OOB and spurious domains is generally the centre frequency of the emission by 250% of the necessary bandwidth of the emission as shown in Figure 5  extracted from ECC Recommendation (02)05 [17].

Note that for single carrier systems, including systems using OFDM/OFDMA, channel bandwidth or channel separation can be used as a substitute for necessary bandwidth provided that they are found in ITU-R Recommendations or in relevant regional and national regulations. Multicarrier systems are not relevant to the determination of receiver resilience levels.

[image: A picture containing text, screenshot, line, plot

Description automatically generated]

[bookmark: _Ref129091796]Figure 5: Illustration of the OOB and Spurious Domains (not part of Recommendation ITU-R SM.329 [7])

Note: The crossover point between OOB and spurious emissions is not defined and Figure 5 shows only an example.

However, the OOB/spurious domain boundary needs to be modified for narrow-band and wideband (including multicarrier) systems, and certain other situations. This modification is very straightforward as described in Table 2 of Rec. ITU-R SM. 1539-1 [11].

As in OOB domain OOB emissions generally predominate, while in spurious domain spurious emissions generally predominate, it is proposed to choose the ILR value to be used in MRR according to the OOB/spurious domain boundary of the interfering signal. Since:

If the victim receiver channel is in the interfering signal OOB domain, the interfering signal ILR would be similar to its ACLR values defined on the 1st and 2nd adjacent channels. Consequently, the interfering signal ILR can be found in the existing EC/ECC Decisions, ECC/ITU-R recommendations, CEPT/ECC Reports and ETSI harmonised standards/technical specifications/reports or can be easily determined by measurements;

If the victim receiver channel is in the interfering signal spurious domain, the interfering signal ILR can be derived from the spurious emission levels defined in the existing EC/ECC decisions, ECC/ITU-R Recommendations (e.g. ERC Recommendation 74-01 [2], Rec. ITU-R SM.329-12 [7]) and ETSI harmonised standards.

[bookmark: _Ref128054841]Guidance to determine relevant interfering signal and interference scenario

The use of reference interfering signal and a single interference scenario as proposed in ANNEX 5: will overcome the difficulty of identifying the relevant interfering signal and interference scenario for each service/system. 

Otherwise, the most relevant interfering signal and interference scenario can be defined between the competent CEPT and ETSI technical groups. 

Moreover, if needed, a guidance on how to determine relevant interfering signal and interference scenario for a given system can be included in the Recommendation on receivers.

[bookmark: _Toc115888599][bookmark: _Toc129956002][bookmark: _Toc155274008]Potential Methods for testing

[bookmark: _Ref85106539]	NFD oriented test of FOS

RIR is the generic extension to any frequency offset of the ACIR defined for adjacent channel offset when a channel arrangement is identified; ECC Report 310 (Equation (7)) [1] also considers the following equivalence:

		

		(59)





Where the C/I ratios are considered at the same desensitisation M; in addition, the report recognises that: “ACIR is practically equivalent to the Net Filter Discrimination (NFD) which may be extended to any wanted and interfering frequency separation;” (e.g. to the frequency offset () where the FOS is defined)

Therefore, for the purpose of this test method, when FOS evaluation can be made through C/I requirements, the following formula can also be considered:

		

		(60)





and, when ILR << FOS:

		

		[bookmark: _Ref129784611](61)





Therefore, whenever a radio equipment standardises also a  behaviour and assuming that the required FOS is derived as in Step 3 of Section 4.4.2, the tests can be made on the basis of NFD defined as in the above equation (64). 

This implies a (usually with a CW interfering signal) at the required  with the same M desensitisation used for the  test.

It is important to note that NFD can be considered as a sub-set of MRR. The major shortcoming of NFD, compared to MRR, is its lack of ability to directly take into account N and M, and therefore the results of NFD are accurate and identical to those obtained by MRR only for high M values (M ≥ 9 dB).

MRR and NFD can be compared simply by comparing the results of Equations (62) and (63) when calculating the adjacent channel PR (PRadj-ch) of a receiver based on the receiver parameters presented in Table 7:

MRR:

			

		[bookmark: _Ref129784503](62)





NFD:

			

		[bookmark: _Ref129784546](63)





[bookmark: _Ref129784557]Table 7: Receiver parameters used to calculate the adjacent channel protection ratio PRadj-ch of a receiver, for comparison between MRR and NFD

		Rx N
(dBm)

		Rx sensitivity
(dBm)

		C/N
(dB)

		PRco-ch
(dB)

		ILR
(dB)

		FOS
(dB)

		M
(dB)



		99.43

		90.43

		9

		9

		50

		60

		1 to 30







[image: Une image contenant graphique

Description générée automatiquement]

[bookmark: _Ref129784584]Figure 6: Comparison of MRR and NFD

The results of the calculations are shown in Figure 6, which make it obvious that the results of NFD are accurate only for high M values (M ≥ 9 dB), since the basic equation of this method does not contain any information about N and M. Consequently, for M < 9 dB, the adjacent channel PR, including blocking level, and their variation as a function of N and M cannot be evaluated using NFD unless for a given N value the co-channel PR of the receiver is calculated/measured for different M values and the calculated/measured values are used step by step in Equation (61) to calculate the adjacent channel PR as a function of M. Consequently, NFD can be considered as a simplified form (sub-set) of MRR.

[bookmark: _Toc115888600][bookmark: _Ref127779632][bookmark: _Ref127780870][bookmark: _Ref131501193][bookmark: _Ref131503044][bookmark: _Ref131512851][bookmark: _Ref129084514][bookmark: _Ref129244067][bookmark: _Ref129253720][bookmark: _Toc129956003][bookmark: _Toc155274009]Reference interfering signal and a single interference scenario to be used in the calculation of receivers resilience levels using MRR

Introduction

When using MRR to define the adjacent channel protection ratio (PR) and the blocking level of receivers, the relevant interfering signal and interference scenario are determined from existing or planned deployment of victim and interfering systems and the compatibility studies presented in various CEPT/ECC, ETSI and ITU-R technical reports, or, if the information in the previous reports is not available, from the intra/inter-system interfering signals and interference scenarios defined in ETSI Harmonised Standards or from the reference interfering signal and interference scenario defined in this annex.

The blocking levels are calculated using MRR and based on the reference interfering signal (RI) described in this section. Consequently, the RI signal can be used to check the conformity of receivers with the related blocking levels. Additionally, the equivalent blocking levels for a CW interfering signal are derived to allow the conformance tests to be done also with a CW signal.

Approach used to define a reference interfering signal based on an ofdm signal generated using an rf signal generator

In nearly all Harmonised Standards (HS), a wanted signal like modulated adjacent channel interfering signal is used to define receiver resilience levels for intra-system compatibility.

On the other hand, for large frequency offsets beyond the operating band of a given system/service, in most cases an unmodulated continuous wave (CW) interfering signal is used to define receiver resilience levels for inter-system compatibility based on the assumption that beyond a large frequency offset the victim receiver FOS is constant, thus independent of the interfering signal bandwidth. This approach avoids the difficulty of identifying the most relevant interfering signal on a case-by-case basis, which is not an easy task due to the increasingly densely used radio spectrum.

For the reasons above, only very few HS use a well-defined interfering signal to define receiver resilience levels for possible inter-system/service compatibility, which justifies the need to define a reference interfering signal for all systems/services when calculating receiver resilience levels using MRR.

[bookmark: _Ref127779696]Proposed reference interfering signal

The proposed reference interfering signal has been derived based on an OFDM signal generated using an RF signal generator and has the following characteristics:

Signal type: OFDM 3GPP FDD-LTE DL. , whichOFDM is the most common modulation type (e.g., IMT (LTE), DVB-T, DAB, DRM, Wireless LAN, WiMAX, etc.); 

Due to the very A wide range of OFDM signal generators are available on the market, and such a signal can easily be generated in a laboratory for test purposes, as part of a larger test configuration;

Channel Bandwidth: 5 MHz. This is a good compromise between potential narrowband and wideband OFDM interfering signals.

Note: other bandwidths maybe be needed for very low (< 30 MHz) and very high frequencies (>10 GHz)

Transmission bandwidth configuration: highest transmission bandwidth allowed for uplink or downlink in a given channel bandwidth, expressed in MHz. For downlink: BW_Config = 15 kHz + NRB × 180 kHz in the downlink (See also Figure 3.1-1 in ETSI EN 301 908-14 V15.1.1 (2021-09)).

OOBD/SD boundary: at ±12.5 MHz offset from the centre frequency of the interfering signal (± 250% of the interfering signal bandwidth). 

OOBE level joins spurious emission (SE) level at the OOBD/SD boundary;

ILR/out-Of-band (OOB) domain:

1st adjacent channel ILR (ACLR) = 48 dB/(5 MHz) 

Value aproximated for IMT BS (44 dB) in ETSI EN 301 908-14 [8] + 3 dB margin (see Figure 8);

2nd adjacent channel ILR = 67 dB/(5 MHz) and 68 dB/(5 MHz), for frequency ranges of 9 kHz ≤ 1 GHz and 1 GHz < f < ≈ 6 GHz respectively.

Value obtained by the method described in Section A5.2.2;

1st adjacent channel ILR to OOBD/SD boundary = 70 dB/(5 MHz) and 74 dB/(5 MHz), for frequency ranges of 9 kHz ≤ 1 GHz and 1 GHz < f < ≈ 6 GHz respectively.

Value obtained by the method described in Section A5.2.2 based on the spurious emissions (SE) limits defined in ERC Recommendation 74-01 (Annex 2, Table 6, line 2.1.1) [2] – 3 dB margin.

Spurious Emissions (SE) domain: 

-39 dBm/(100 kHz) for 9 kHz ≤ 1 GHz and 

-33 dBm/MHz for 1 GHz < f < ≈ 6 GHz

as defined in Rec. ERC Recommendation 74-01 – 3 dB margin, (ERC Recommendation 74-01, Annex 2, Table 6, 2.1.1). 

These limits are the most common limits to radio transmitters including IMT base stations. Note that the maximum SE bandwidth is limited to 2 times of the bandwidth of the reference interfering signal (2*BWRI) to prevent from calculating non-realistic spurious emission levels for a 5 MHz interference signal over a wide range frequency.

[bookmark: _Ref131514224][bookmark: _Ref129092207]Step-by-step Dderivation of the spectrum mask of the reference interfering signal

The spectrum mask of the reference interfering signal described in A5.2.1 has been derived according to the following steps:

Step 1: A 5 MHz OFDM signal having a 1st adjacent channel ILR (ACLR) of 47 dB/5 MHz was generated using an OFDM signal generator. The value of 47 dB/5 MHz is the IMT BS 1st adjacent ILR (ACLR) defined in ETSI EN 301 908-14 [8] increased by 3 dB. The 2nd adjacent channel ILR of this signal is 64 dB/(5 MHz) as shown in Figure 7.

Nevertheless, for measurement purpose, if it is difficult to generate an OFDM signal having exactly the above ILR values, it would be acceptable to generate an OFDM signal having slightly different ILR values (max 2 dB/5 MHz). In all the cases, the ILR value measured for a given receiver channel bandwidth should be equal to the ILR value calculated from the reference interfering signal spectrum mask ±1 dB, defined in Table 11.

[image: Chart

Description automatically generated]

[bookmark: _Ref127775210]Figure 7: 5 MHz OFDM signal (QPSK/4-QAM modulation, PAPR = 11 dB, Spectrum analyser noise floor = -117.5 dBm/kHz)

A numerical integration method based on the trapezoidal rule has been used to calculate the ILR of the spectrum mask of the reference interfering signal as follows:

		

		[bookmark: _Ref127786302](64)





Where:

: frequency offset interfering signal in-channel power at the receiver input;

: victim receiver bandwidth;

: centre frequency of the victim receiver channel;

: frequency offset interfering signal spectral density in the linear domain.



		

		(65)





Where:

		

		(66)





The validity of the integration method was checked as presented in Table 8.

[bookmark: _Ref127784099]Table 8: Comparison of measurements and numerical integration results (Measurement BW=5 MHz, Integration BW=4.97 MHz)

		Offset type

		Integration BW (MHz)

		ILR obtained by measurement (dB)

		ILR obtained by numerical integration (dB)

		Difference (dB)



		Co-channel

		4.975 MHz

		0

		0

		0



		1st adjacent channel

		4.975 MHz

		46.73

		47.27

		0.54



		2nd adjacent channel

		4.975 MHz

		63.86

		63.80

		-0.06



		>2nd adjacent channel

		4.975 MHz

		70

		70.28

		0.28







Step 2 (for 9 kHz < f < 1 GHz): A reference interfering (RI) signal has been derived from the measured 5 MHz OFDM signal spectrum of which 4.5 MHz is the transmission bandwidth configuration (60 W / 4.5 MHz, 47.78 dBm / 4.5 MHz, 11.25 dBm / 1 kHz, normalising the whole mask to the in-band power spectral density of 0 dBm / kHz. For the spurious emissions, -39 dBm / 100 kHz = -59 dBm / kHz and then normalised by applying the 11.25 dB to give ‑70.25 dBm / kHz) using linear interpolation to join the OOBE level to SE level of -36 dBm/(100 kHz) decreased by 3 dB at the OOBD/SD boundary. The derived spectrum mask has four a set of break points as shown in Figure 8. The ILR of the RI signal has been calculated by numerical integration of its power (W) in a bandwidth of 5 MHz, as described above, at three different frequency offsets as shown in Table 9.

[image: Chart, line chart

Description automatically generated]

[bookmark: _Ref127783744]Figure 8: Approximation of the 5 MHz OFDM signal to a reference interfering signal for 30 <  MHz <  f < 1 GHz



[bookmark: _Ref127784119]Table 9: ILR of the reference interfering signal for 30 MHz < f < 1 GHz (60 W/5 MHz conducted Tx power normalised to 0 dBm/kHz; Fnorm=11.25 dB)

		Channel BW (MHz)

		Power per channel BW (W)

		Power per channel BW (dBm)

		Offset type

		ILR of the RI signal (dB)

		ILR of the measured signal (dB)



		4.97

		4.54

		36.57

		Co-channel

		0

		0



		4.97

		6.61E-05

		-11.80

		1st adjacent channel

		48

		47



		4.97

		9.25E-07

		-30.34

		2nd adjacent channel

		67

		64



		4.97

		4.69E-07

		-33.29 (Note 1)

		1st adjacent channel to OOBD/SD boundary in the SD

		70

		70



		Note 1 : -33.29 dBm/5MHz+11.25 dB =-39dBm/100kHz, which is the spurious limit of -36dBm/(100 kHz) minus a 3 dB margin





Step 3 (for 1 GHz < f<  6 GHz): A reference interfering signal has been derived from the measured 5 MHz OFDM signal spectrum used in Step 2 (see that step for how the normalisation was calculated) using linear interpolation to join the OOBE level to SE level of -30 dBm/MHz decreased by 3 dB at the OOBD/SD boundary. The derived spectrum mask has three break points as shown in Figure 9. The ILR of the RI signal has been calculated by numerical integration of its power (W) in its bandwidth of 5 MHz at three different frequency offsets as shown in Table 10.

[image: ]

[bookmark: _Ref127784029]Figure 9: Approximation of the 5 MHz OFDM signal to a reference interfering signal for 1 GHz < f <  6 GHz



[bookmark: _Ref127784138]Table 10: ILR of the reference interfering signal for 1 GHz < f <  6 GHz (60 W/5 MHz conducted Tx power normalised to 0 dBm/kHz; Fnorm=11.25 dB):

		Channel BW (MHz)

		Power per channel BW (W)

		Power per channel BW (dBm)

		Offset type

		ILR of the RI signal (dB)

		ILR of the measured signal (dB)



		4.97

		4.54

		36.57

		Co-channel

		0

		0



		4.97

		6.61E-05

		-11.80

		1st adjacent channel

		48

		47



		4.97

		7.22E-07

		-31.41

		2nd adjacent channel

		68

		64



		4.97

		1.87E-07

		-37.29 (Note 1)

		1st adjacent channel to OOBD/SD boundary in the SD

		74

		70



		Note 1: -37.29 dBm/(5 MHz)+11.25 dB =-33dBm/MHz, which is the spurious limit of -30dBm/MHz minus a 3 dB margin





Approach used to define a single interference Scenario

[bookmark: _Ref131514565][bookmark: _Ref129092423]Rationale

The rationale behind the idea to have a single interference scenario is:

To treat all systems equally;

To define the recommended receiver resilience levels only for the most relevant frequency offsets, thus to keep the size of the ECC Recommendation on receivers reasonable;

Detailed description of the proposed interference scenario

According to the rational presented in Section A5.3.1, it is proposed to define the receiver resilience level for different frequency offsets between the interfering transmitter and the victim receiver:

In the OOB domain, the first adjacent channel to the interfering signal, for the purpose of receiver adjacent channel PR calculation/measurement;

In the RSBD of the interfering signal, provided that the interfering signal is beyond the second adjacent channel of the victim receiver, for the purpose of receiver blocking level calculation/measurement.

The frequency offsets between the reference interfering signal (RI) centre frequency and the victim receiver (Rx) centre frequency are calculated as follows:

In the OOB domain:

Frequency offset (MHz) = BWRI/2 (MHz) +BWRx/2 (MHz)

In the RSBD:	Comment by France: Comment 4 from Annex 1 of the LS from ETSI

If BWTX > BWRX 

	Frequency offset (MHz) ≥ 2.5BWRI (MHz) + BWRx/2 (MHz)

Else

	Frequency offset (MHz) ≥ 2.5BWRx (MHz) + BWRI/2 (MHz)

For channelised systems, the above calculations define that the victim receiver channel is in the spurious domain of the interfering transmitter and reciprocally the interfering transmitter channel is in the spurious domain of the wanted transmitter of the victim receiver, whatever their bandwidth (BW) is. 

For single carrier and multi-carrier receivers the bottom channel at the lower edge and the top channel at the upper edge of the receiver operating band is to be defined and tested. For receivers supporting multiple channel bandwidths, the smallest and the largest channel bandwidths should be defined and tested. 

For non-channelised systems the occupied bandwidth is considered as a single channel (see Figure 10).

Victim receiver operating band

Frequency offset

Frequency

Frequency offset

fc

fc

fc

fc

Victim receiver channel

Victim receiver channel

Reference interfering signal

Reference interfering signal



[bookmark: _Ref131521023]Figure 10: Channel of the receiver to be defined and tested

This interference scenario including reference interferer (RI) and the two offset frequencies is depicted in Figure 11 for the example of a victim receiver having a bandwidth of 2 MHz (BWRI > BWRx).

[image: Chart, line chart

Description automatically generated]

[bookmark: _Ref127783697][bookmark: _Ref127783692]Figure 11: Proposed single interference scenario to be used for all systems/services when calculating receivers resilience levels using MRR

The resulting offset frequencies from the centre of the RI to the centre frequency of the victim receiver are for the 2 MHz receiver:

3.5 MHz: 2.5 MHz (BWRI/2)) + 1 MHz (BWRx/2);

13.5 MHz: 12.5 MHz (2.5x BWRI)) + 1 MHz (BWRx/2).

The ILR values for the two offset frequencies are then to be derived by numerical integration from the spectrum masks defined in Table 11: depending on the victim receiver bandwidth according to Equation (4). Note that the maximum SE bandwidth (BW) of the RI signal is assumed to be 10 MHz (2BWTx). Consequently, for the victim receivers having a BW greater than 10 MHz the SE level of the RI should be reduced by the factor of 10log10(BWRx (MHz)/(10 MHz)) when calculating/measuring ILR.

[bookmark: _Ref129255300][bookmark: _Ref127782621]Table 11: Reference interfering signal spectrum mask

		Offset (MHz)

		RI signal spectrum mask for frequencies of 
9 kHz   f   1 GHz (dBm/kHz)

		RI signal spectrum mask for frequencies of 1 GHz < f < ≈ 6 GHz (dBm/kHz)



		-17.5

		-70.25

		-74.25



		-12.5

		-70.25

		-74.25



		-10

		-70.25

		-72



		-7.5

		-62

		-62



		-2.5

		-42

		-42



		-2.25

		0

		0



		+2.25

		0

		0



		+2.5

		-42

		-42



		+7.5

		-62

		-62



		+10

		-70.25

		-72



		+12.5

		-70.25

		-74.25



		+17.5

		-70.25

		-74.25



		[bookmark: _Hlk129249505]Note: For measurement purposes, the reference interfering signal can be generated using any OFDM RF signal generator available on the market. The recommended modulation schemes are QPSK/4-QAM (max data rate  8 Mbps, sub-carrier spacing  15 kHz), with preferably a PAPR =11 ± 3 dB at the RF generator output. Higher constellations and data rates can also be used provided that the generated signal meets the interfering signal spectrum mask defined in this table as well as the PAPR value. Nevertheless, for measurement purpose, if it is difficult to generate an OFDM signal having exactly the above ILR values, it would be acceptable to generate an OFDM signal having slightly higher ILR values (max 2 dB/5 MHz). In all the cases, the ILR value measured for a given receiver channel bandwidth should be equal to the ILR value calculated from the reference interfering signal spectrum mask defined in the table. The ILR value of the generated reference interfering signal can be adjusted to the required ILR by adjusting the generator output power. If the required ILR value is not reached an external filter can be used to improve the value.





Some calculation examples are provided in the following section.

Derivation of the reference interfering signal ILR to be used in the calculation of receivers resilience levels using MRR

The ILR values of the reference interfering signal to be used to calculate the receiver resilience levels for some systems/applications have been derived by using numerical integration. These systems are DAB, DTT, IMT, Fixed service, GSM, 2.4 GHz WBDT systems, 5 GHz RLAN and 30 MHz-1GHz SRD. The results obtained are presented for information in Section A5.3.5.

Choice of the FOS value

Based on the definition of the reference interfering signal, it is proposed to choose the value of FOS to be used in the calculation, when using MRR to calculate the receiver resilience levels, as follows:

FOS = ILR + 10 dB, when the victim receiver channel is adjacent to the interfering transmitter channel (see Figure 11).

Reason: This configuration is critical since the interfering transmitter channel is very close to the victim receiver channel resulting in a high OOB emission level in the receiver input, which cannot be reduced by filtering. Consequently, a good adjacent channel filtering would be necessary to minimise the interfering signal in-block power at the receiver input.

FOS = ILR dB, when the victim receiver channel is adjacent to the OOBD/SD boundary of the interfering signal in its SD (see Figure 11). If a CW interfering signal is used, instead of the RI signal, to calculate the blocking level of a victim receiver, the receiver FOS value should be equal to the FOS value specified for the blocking level calculation with the RI signal, since the ILR value of the CW signal is assumed to be very high or infinite (≥130 dB).

Reason: This configuration is less critical compared to the first configuration, since the interfering transmitter channel is far away from to the victim receiver channel resulting in a lower OOBE emission level in the receiver input. Consequently, it is sensible to put less constraint on the receiver FOS.

[bookmark: _Ref129092750]Derived reference interfering signal ILR to be used in the calculation of receiver resilience levels

Table 12: ILR values for systems/applications as a function of the victim channel bandwidth and frequency offset: 30 MHz < f < 1 GHz

		System

		Victim channel BW (MHz)

		Power (W)

		Power (dBm)

		Frequency offset

		ILR (dB)



		RI

		4.97

		4.54

		36.57

		Co-channel

		0



		DAB (1.54)

		1.54

		5.05E-05

		-12.97

		1st adjacent channel

		50



		DAB (1.54)

		1.54

		1.45E-07

		-38.37

		1st adjacent channel to OOBD/SD boundary in the SD

		75



		DTT (8 MHz)

		7.60

		5.70E-05

		-12.44

		1st adjacent channel

		49



		DTT (8 MHz)

		7.60

		7.17E-07

		-31.44

		1st adjacent channel to OOBD/SD boundary in the SD

		68



		IMT (1.4 MHz)

		1.09

		4.21E-05

		-13.76

		1st adjacent channel

		50



		IMT (1.4 MHz)

		1.09

		1.02E-07

		-39.90

		1st adjacent channel to OOBD/SD boundary in the SD

		76



		IMT (3 MHz)

		2.70

		6.12E-05

		-12.13

		1st adjacent channel

		49



		IMT (3 MHz)

		2.70

		2.54E-07

		-35.94

		1st adjacent channel to OOBD/SD boundary in the SD

		73



		IMT (5 MHz)

		4.52

		2.91E-05

		-15.36

		1st adjacent channel

		52



		IMT (5 MHz)

		4.52

		4.26E-07

		-33.70

		1st adjacent channel to OOBD/SD boundary in the SD

		70



		IMT (10 MHz)

		9.00

		4.43E-05

		-13.54

		1st adjacent channel

		50



		IMT (10 MHz)

		9.00

		8.49E-07

		-30.71

		1st adjacent channel to OOBD/SD boundary in the SD

		67



		IMT (15 MHz)

		13.51

		3.26E-05

		-14.86

		1st adjacent channel

		51



		IMT (15 MHz)

		10.01

		9.45E-07

		-30.25

		1st adjacent channel to OOBD/SD boundary in the SD

		67



		IMT (20 MHz)

		18.03

		2.92E-05

		-15.34

		1st adjacent channel

		52



		IMT (20 MHz)

		10.01

		9.45E-07

		-30.25

		1st adjacent channel to OOBD/SD boundary in the SD

		67



		GSM (0.2 MHz)

		0.18

		9.88E-06

		-20.05

		1st adjacent channel

		57



		GSM (0.2 MHz)

		0.18

		1.65E-08

		-47.82

		1st adjacent channel to OOBD/SD boundary in the SD

		84



		SRD (0.1 MHz)

		0.11

		6.12E-06

		-22.13

		1st adjacent channel

		59



		SRD (0.1 MHz)

		0.11

		9.91E-09

		-50.04

		1st adjacent channel to OOBD/SD boundary in the SD

		87



		SRD (0.5 MHz)

		0.49

		2.41E-05

		-16.17

		1st adjacent channel

		53



		SRD (0.5 MHz)

		0.49

		4.63E-08

		-43.35

		1st adjacent channel to OOBD/SD boundary in the SD

		80



		SRD (1.7 MHz)

		1.68

		5.25E-05

		-12.80

		1st adjacent channel

		49



		SRD (1.7 MHz)

		1.68

		1.59E-07

		-38.00

		1st adjacent channel to OOBD/SD boundary in the SD

		75



		SRD (0.05 MHz)

		0.04

		2.63E-06

		-26.77

		1st adjacent channel

		62



		SRD (0.05 MHz)

		0.04

		4.13E-09

		-54.81

		1st adjacent channel to OOBD/SD boundary in the SD

		90



		SRD (0.025 MHz)

		0.04

		1.32E-06

		-28.81

		1st adjacent channel

		65



		SRD (0.025 MHz)

		0.04

		2.06E-09

		-56.85

		1st adjacent channel to OOBD/SD boundary in the SD

		93







Table 13: ILR values for systems/applications as a function of the victim channel bandwidth and frequency offset: 1 GHz < f <≈ 6GHz

		System

		Victim channel BW (MHz) 

		Power (W)

		Power (dBm)

		Frequency offset

		ILR (dB)



		RI

		4.97

		4.54

		36.57

		Co-channel

		0



		DAB (1.54 MHz)

		1.54

		6.61E-05

		-11.80

		1st adjacent channel

		50



		DAB (1.54 MHz)

		1.54

		1.87E-07

		-37.29

		1st adjacent channel to OOBD/SD boundary in the SD

		79



		DTT (8 MHz)

		7.60

		5.05E-05

		-12.97

		1st adjacent channel

		49



		 DTT (8 MHz)

		7.60

		5.79E-08

		-42.37

		1st adjacent channel to OOBD/SD boundary in the SD

		72



		IMT (1.4 MHz)

		1.09

		5.69E-05

		-12.45

		1st adjacent channel

		50



		IMT (1.4 MHz)

		1.09

		2.85E-07

		-35.44

		1st adjacent channel to OOBD/SD boundary in the SD

		80



		IMT (3 MHz)

		2.70

		4.21E-05

		-13.76

		1st adjacent channel

		49



		IMT (3 MHz)

		2.70

		4.08E-08

		-43.90

		1st adjacent channel to OOBD/SD boundary in the SD

		77



		IMT (5 MHz)

		4.52

		6.12E-05

		-12.13

		1st adjacent channel

		52



		IMT (5 MHz)

		4.52

		1.01E-07

		-39.94

		1st adjacent channel to OOBD/SD boundary in the SD

		74



		IMT (10 MHz)

		9.00

		2.91E-05

		-15.36

		1st adjacent channel

		50



		IMT (10 MHz)

		9.00

		1.70E-07

		-37.70

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		IMT (15 MHz)

		13.51

		4.41E-05

		-13.56

		1st adjacent channel

		51



		 IMT (15 MHz)

		10.01

		3.38E-07

		-34.71

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		IMT (20 MHz)

		18.03

		3.22E-05

		-14.92

		1st adjacent channel

		52



		 IMT (20 MHz)

		10.01

		3.76E-07

		-34.25

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		FS (1.4 MHz)

		1.75

		2.85E-05

		-15.45

		1st adjacent channel

		49



		FS (1.4 MHz)

		1.75

		3.76E-07

		-34.25

		1st adjacent channel to OOBD/SD boundary in the SD

		78



		FS (1.4 MHz)

		7.00

		5.34E-05

		-12.73

		1st adjacent channel

		48



		FS (1.4 MHz)

		7.00

		6.58E-08

		-41.82

		1st adjacent channel to OOBD/SD boundary in the SD

		72



		FS (1.4 MHz)

		14.00

		6.67E-05

		-11.76

		1st adjacent channel

		48



		FS (1.4 MHz)

		10.01

		2.63E-07

		-35.80

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		SRD (0.1 MHz)

		0.11

		6.70E-05

		-11.74

		1st adjacent channel

		59



		SRD (0.1 MHz)

		0.11

		3.76E-07

		-34.25

		1st adjacent channel to OOBD/SD boundary in the SD

		91



		WLAN (20 MHz)

		19.99

		6.12E-06

		-22.13

		1st adjacent channel

		50



		WLAN (20 MHz)

		10.01

		3.95E-09

		-54.04

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		DCS (0.2 MHz)

		0.18

		4.45E-05

		-13.52

		1st adjacent channel

		57



		 DCS (0.2 MHz)

		0.18

		3.78E-07

		-34.23

		1st adjacent channel to OOBD/SD boundary in the SD

		88





[bookmark: _Ref129783203][bookmark: _Toc129956004][bookmark: _Toc155274010]Point-to-point FS emission masks for ILR/ACLR evaluation

Introduction

The proposed RI mask for bands < 1 GHz and for 1 GHz < bands < 6 GHz was derived as OFDM 5 MHz bandwidth (widely used in broadcasting and mobile radios, which are the most used in those bands).

In higher bands, one of the most used "licensed/authorised" service is the fixed service, of which the most popular worldwide application is point-to-point (PtP) used in many fixed core and mobile backhauling applications.

The allocated bands with major usage (or usable span), with a few exceptions, range from 5.925 GHz to 43.5 GHz, 48.5 GHz to 57 GHz and 71 GHz to 86 GHz. Higher bands up to 174.7 GHz are already considered usable in ECC Recommendation (18)01 [18] and ECC Recommendation (18)02 [19].

The major differences from the OFDM case proposed for < 6 GHz are:

1. The modulation used is X-QAM (with X from 4 to 2048 and higher) with relatively tight roll-off; this implies that the spectrum emission is not rectangular.	
However, while the regulations for broadcast and mobile allocated bands do not provide guard-bands (hence the need for quasi rectangular spectrum), the channel arrangements for FS PtP allocated bands always provide internal guard-bands.

The PtP fixed service operates using high directivity antennas, in line of sight between fixed transmitter and receiver station locations, therefore along the path no interfered radio system (infrastructure or persons) may be present; 

Consequently, while the broadcasting and mobile BS EIRP is generated by low gain antennas (then the actual emitted power at antenna port is similarly high); the PtP EIRP is the result of high directivity antennas and the actual transmit power, at antenna port, is far less (at least 40 dB) than the EIRP.

In addition, the RF power devices capability, in terms of maximum power, rapidly decreases while the operating band increases. 

The approximately needed (at antenna port) 60 Watts considered in the RI for broadcasting and mobile BSs, are reflected in needed 1 Watt (decreasing with frequency to about 0.25 W) generated by FS transmitters at antenna port.

Therefore, the comparison between broadcasting/mobile BS transmit power and FS PtP power (at the antenna port) is here given only for discussing the suitability of using other relative RI emission masks.

The available system bandwidth increases with the operating band; which fits with the constantly increasing demand for backhauling traffic; while there might be relatively narrow bandwidth legacy links (e.g. 7 or 14 MHz channels), the vast majority of new links have channel bandwidth of:

ii) 28, 40 or 56 MHz (in bands from 5.925 GHz to 15.35 GHz);

iii) 56, 112 and possibly 224 MHz (in bands from 17 GHz to 43.5 GHz);

500, 1000 and possibly up to about 2 000 MHz (in bands from 71 GHz to 115 GHz).

The transmitted relative spectral attenuation decreases accordingly with the actual transmit power; in addition, the filtering capability is relative to the pass-band; therefore, at a fixed distance from the channel edge, the capability decreases for the wider bandwidth systems.

[bookmark: _Ref132136250][bookmark: _Toc155274011][bookmark: _Toc380059620][bookmark: _Toc380059762][bookmark: _Toc396383876][bookmark: _Toc396917309][bookmark: _Toc396917420][bookmark: _Toc396917640][bookmark: _Toc396917655][bookmark: _Toc396917760]Background information on RAS receivers

Receiving signals above the linear regime of a radio astronomy telescope frontend can result in a variety of effects, such as saturation of the LNAs or other devices (such as ADCs, optical fibre links), generation of intermodulation products, or even worse, reaching the amplifiers' break point. While the use of filters may look as a potential solution to these issues, it comes associated with many challenges, which are reviewed in this Annex.

This Annex aims to provide more detailed technical information on RAS receivers. While these receivers are not within the scope of the ECC Recommendation on receiver resilience levels[footnoteRef:3], it is important to capture their unique characteristics and requirements to highlight the need for a different approach for non-market scientific equipment, including RAS receivers. It should be also noted that there are some fundamental differences between modern digital-based communication systems and nature-observing ones (e.g. lack of BER characteristic). [3:  ECC Recommnedation (24)XXX  ] 


Modern radio astronomy is heavily dependent on both microwave front-end receiver and digital signal processing technology. To reach the required levels of sensitivity, a combination of huge collecting areas (radio telescopes), extremely low-noise amplifiers, cryogenically cooled passive microwave components, and digital acquisition systems is required. Besides state-of-the-art equipment, long integration times (often hours) and wide frequency bands are used by radio astronomers to reduce the thermal noise and to detect the faintest celestial signals. As the scientific instrumentation requires major investments, usually from governments, it requires appropriate protection from active radio transmitters. Astronomers do their part towards this protection by building new telescopes in very remote sites, often taking advantage of natural terrain shielding (see Recommendation ITU-R RA.611 [20]). Several administrations have also established radio-quiet zones (RQZ) or coordination zones around the RAS stations in their countries (see Report ITU-R RA.2259 [21]).

In order to justify the high cost of operation, resulting from the heavy investments, astronomy stations need to stay internationally competitive and they need to maximise the scientific output of their instruments. Therefore, observing time is granted by independent refereeing committees, which will assign time slots only to those scientists who submitted the highest-ranking observing proposals. Furthermore, most radio observatories are heavily overbooked, i.e., the amount of requested time is much higher than the available time, making the loss of observing time a critical factor for scientific development through radio astronomy observations. 

One way to improve the overall observing efficiency and sensitivity, is the installation of broad-band receivers, which can measure astronomical signals in several allocated RAS bands at the same time. The merit of observing large bandwidths is not restricted to gaining extra sensitivity. Many important astrophysical sources can only be properly investigated if a significant fraction of their spectra is acquired. The relative strength of the electromagnetic signal as a function of wavelength or frequency allows us to determine the radiation mechanisms within the celestial sources. Likewise, spectral line ratios can be used to measure physical properties of the interstellar gas, such as temperature or pressure. Other motivations to perform multi-broadband observations are to measure the dispersive effect of the interstellar medium or calibrating high frequency observations by using low frequency targets. By measuring the delay of a pulsed (brief) signal at different frequencies, astronomers can estimate the density and distribution of the interstellar medium. Time-domain astronomy, which looks out for individual rare events such as fast-radio bursts (FRB), cannot even observe the different bands at different times, because such events are singular.

An example of a modern receiver is the Ultra-Broadband Receiver at the 100 m telescope at Effelsberg (Germany) offering an observing band between 1 and 5 GHz, simultaneously, with analogue-digital converters (ADC) installed at the front-end. A second example of applied wide-band technology is the VGOS system, which uses four or more frequency bands in the range from 2.5 GHz to 14 GHz at the same time for highest precision and to calibrate out atmospheric delays.

Unfortunately, the extreme sensitivity of RAS receivers is also a burden. Even if more effort is made to increase the dynamic range, it is technically impossible to build a receiver capable of observing the faintest signals from the universe (reaching the sub-𝜇Jy to mJy regime, with 1 Jy = 10-26 W/m2/Hz = –260 dB(W/m2/Hz)) and at the same time be robust against strong anthropogenic signal transmissions, which can easily cause levels 100 dB above the noise floor at the RAS receiver input, if active services and the RAS are not well-coordinated. Usually, the RAS and active services do not share allocations in the same bands, especially in the case of airborne and spaceborne transmitters. In addition to the few shared bands, out-of-band and spurious emissions from active services in adjacent and nearby bands may generate harmful interference in protected radio astronomy bands.

However, even if the active transmitter frequencies are well separated in frequency from the RAS observing band and out-of-band and spurious emissions are well coordinated, serious issues can still emerge. Receiving signals above the linear regime of a radio astronomy telescope front-end can result in a variety of effects, such as saturation of the low noise amplifiers (LNAs) or other active devices (such as second stage amplifiers, ADCs, optical fibre links), generation of intermodulation products, or even worse, reaching the amplifiers' break point, leading to (see Report ITU-R RA.2188 [22]). It is also worth noting that today a fair fraction of LNAs used in radio astronomy receivers, especially below about 100 GHz, are often off-the-shelf components. These devices usually come with a relatively large input bandwidth already.

this worst-case situation occurs, the replacement of the damaged device is required to repair the receiver. This incurs a cost, which is reflected in the purchase of new components, their installation and the added cost of having the receiver out of operation for the duration of this process. But even in less drastic scenarios, mitigation techniques may need to be implemented to suppress the strongest signals, which is still costly, and as the spectrum environment is very dynamic and is subject to changes, this solution implies a continuous effort for the observatories.

While the use of filters may appear as a potential solution to improve compatibility between strong transmitters and a RAS station, it is not always technically feasible and it can have significant impact on sensitivity. First, analogue filters have finite suppression factors, so their effectiveness depends on the technology used and the frequency separation between active services and RAS bands. Second, frontend (notch-) filters installed in front of the first LNA add significant noise to the system, as their signal gets amplified by tens of dB in the signal chain. If at least the analogue part of the receiver is kept in the linear domain then, in principle, digital filters can be used further downstream to still use the unaffected frequencies. 

In conclusion, it is important to preserve unique operational conditions of scientific equipment, and especially RAS operations, to enable future scientific breakthroughs. RAS emission sources are natural sources of cosmic origin, which are of analogue nature and are not only out of control, but also out of humankind's reach. Therefore, radio astronomy receiving systems require a certain level of general radio quietness in the environment of an observatory in order to protect them.

RAS RECEIVER SENSITIVITY

For telecommunication applications using a receiving system linked to an emitter, the artificial signal of interest is usually above the receiver system noise and so the signal-to-noise ratio (SNR) is greater than 0 dB. This is not the case for high sensitivity detection applications such as radio astronomy for which the natural signal of interest (for example a radio source as a star or a galaxy) can be 30 to 60 dB below the instantaneous thermal noise. One way to detect such a tiny stationary signal within uncorrelated noise is to decrease the noise fluctuation level using long integration times.

Sensitivity and the radiometer equation

The goal of a radiometer is to convert the energy of the incident radio wave into an electrical signal, feasible to be analysed and processed in a computational system. In general, a radiometer is a high-sensitivity microwave receiver that detects and measures noise-like signals. The setup of a classical radiometer is composed of an antenna and its front-end amplifier (a mixer or a bolometer in millimetric frequency), a radio frequency conditioning system (filtering, frequency conversion) and a data acquisition system. A radio telescope can be viewed as a very high performance radiometer. In astronomy, the sources of radiated emissions can be celestial bodies, such as planets, stars (including the Sun) or meteors, but also the interstellar or intergalactic medium. One can distinguish the celestial sources by spectral properties, e.g. continuum vs. spectral-line emission, but also by temporal properties (constant vs. intermittent or even pulse-like). Often the received signal has similar properties to the noise generated by the receiver itself or to terrestrial natural background signals coupled to the receiver, such as ground or atmospheric radiation. In rare cases receivers also detect diffuse anthropogenic signals that can be hard to distinguish from celestial signals of interest.

Assuming an antenna pointing to a celestial source, the received power at the output port of the radiometer is P (W) expressed as:

		

		(67)





where  is the Boltzmann constant (1.38 × 10-23 J/K),  is the receiver bandwidth in Hertz,  is the receiver gain.  is the receiver noise temperature, which represents the equivalent internal noise generated by the receiver itself brought to the input. Moreover, the antenna temperature () is the convolution between the sky brightness temperature and the antenna pattern (spatial distribution of the antenna response). In simple terms, the brightness temperature, , of any source is the physical temperature of a black body which would produce the same power flux density at the RAS receiver as the source. The sky brightness includes all emitting factors seen by the radio telescope, such as the background signals from the universe (cosmic background noise or galactic centre or other strong celestial emitters), natural ground contribution, atmospheric emission etc. The sum of the two noise contributions,  and , is usually denoted as the system temperature, .

The sensitivity of a radio receiver refers to its ability to detect weak signals and is usually defined as the minimum detectable temperature difference () in the antenna temperature that can be measured by the receiver in a given bandwidth  and integration time . The sensitivity, is given by the radiometer equation:

		

		(68)





where  is the bandwidth of the observation and  is the integration time. The square root in the denominator of this equation is important. If the noise level of a measurement shall be reduced by a factor of two, the integration time needs to be quadrupled. Thus, even small degradations in the sensitivity have drastic consequences on the necessary integration times to reach a certain noise level, increasing the operation cost of a telescope accordingly.

It is common for radio astronomers to observe signals that have mean power levels several times less than background noise. Therefore, long integration exposure and wide integration band (for continuum observations) are commonly applied to increase the sensitivity. 

In a radio astronomy receiver, uncorrelated noise, which is mainly caused by thermal noise, is the dominant type of noise added to the signal of the observed source. This noise is generated by the random motion of electrons in a conductor and is present in all electronic devices, contributing to receiver noise temperature. Additionally, thermal noise can also come from natural sources such as the atmosphere, ground emissions picked-up by the telescope antenna, and background signals from the universe, which may not be of primary interest to the observer. Because the random nature of the noise, integration reduces the level of fluctuations by averaging the signal over a longer period of time. The integration time used in radio astronomy can vary from seconds to several hours or even more. Longer integration times enable the detection of weaker and more distant sources. However, for some scientific applications, such as to detect fast transient phenomena, the integration time must be kept short to reach the desired time resolution. In this case only an increased bandwidth can be used to reduce the noise as described in the radiometer equation. 

The calculation method and typical technical parameters that define the sensitivity of radio astronomy observations over a large range of frequencies can be found in Recommendation ITU-R RA.769 [23]. To quote some example values, power flux density levels that must not be exceeded to protect RAS observations range from about –260 dB(W/m2/Hz) at lower frequencies (e.g., at 150 MHz) to approximately –220 dB(W/m2/Hz) above 200 GHz. 

Noise figure

The noise figure is a standard way to express the noise temperature of a receiver. It is a metric for the amount of additional noise added by an electronic device to the input signal. It is typically expressed in decibels (dB) and is calculated as the difference between the input SNR and the output SNR of the receiver:

		

		(69)





In a multi-stage system with multiple filtering stages, the overall noise figure can be calculated using Friis’ formula:

		

		(70)





Where  is the noise figure of the first stage,  is the noise figure of the second stage, etc. , , etc. are the gains of the first, second, and subsequent stages.

Therefore, the noise figure of a multi-stage system cannot be lower than the noise figure of the first stage and also that subsequent stage’s noise figures are decreased by cumulative gains of previous stages. Therefore, it is essential to minimise the noise figure of the first stage and use subsequent stages only when necessary, in order to achieve the desired gain.

For high sensitivity receiving systems, as those used in radio astronomy, the receiving chain is usually composed by the so-called feed-system consisting of passive components to couple the incoming radio wave reflected by the mirrors of the radio telescopes into a guided transmission line. Such components typically consist of the following elements: feed-horn, polariser, ortho-mode transducer and noise marker coupler. Since all these components come before the first amplifier, their contribution to the overall receiver noise is significant. Therefore, they are cooled to cryogenic temperatures to minimise their ohmic attenuation together with the first low noise amplifier (LNA). Positioning a microwave filter to increase immunity to interference would add further losses in this sensitive section of the chain and therefore it should be considered as a last resource mitigation measure. Filters are typically placed after the first stage of amplification, lowering their noise contribution and also because the last stages of the receiver are more prone to get saturated due to the additional gain provided by the previous stages.

As a rule of thumb, modern LNAs achieve noise temperatures of 0.25–0.5 K times f0 (GHz), e.g., a 4–8 GHz LNA would have between 2 to 4 K when cooled to cryogenic temperatures of 15 K, and thus provide a very sensitive receiver. This implies that its dynamical margin cannot cope with very strong active-service transmissions (e.g. radars or telecommunication towers) in the immediate vicinity. The typical compression level at the input of an LNA is around –50 to –40 dBm noting that this value depends on the material of the transistors of the LNA, the polarisation of the different stages, the frequency, etc. Figure 12 shows the gain and output power vs. input power for a typical LNA used in radio astronomy.

[image: ]

[bookmark: _Ref131270710]Figure 12: Characterization of a broadband LNA (2–14 GHz) for a VGOS receiver. Top: gain vs. input power. Bottom: output power versus input power, the marker indicates the P1dB level (–38 dBm at the input).

NON-LINEAR EFFECTS

An important figure of merit in radio astronomical receivers is the linearity. If the active stages do not operate in linear regime some distortions will occur affecting the astronomical observation. Signals at new frequencies would appear at the output of the receiver and might be interpreted as actual radiation entering in the antenna. Nowadays, this is of primary concern in radio astronomy due to the presence of strong artificial emitters.

The amplifier's burning point and compression points are crucial levels of input power for the LNA. The burning point is the maximum output power level that an amplifier can sustain without damaging itself due to excessive heat generation. This is also called the amplifier's maximum power handling capability.

Report ITU-R RA.2188 [22] provides guidelines for the maximum power flux-density and equivalent isotropically radiated power (e.i.r.p.) levels that could potentially cause damage to radio astronomy receivers in the frequency range of 1 to 275 GHz. Tables 1 and 2 in RA.2188 list the maximum allowable power flux-density and e.i.r.p. levels in units of dB(W/m²) and dB(W/Hz) for two different technologies in the active stages.

Compression point and intermodulation points

The saturation points are the points at which the amplifier enters the non-linear regime. Two types of saturation points should be considered at minimum, the compression point and the intermodulation point, considering amplitude and frequency, respectively, of the non-linear effects.

1dBc compression point

At the compression point, the output power level stops to increase, even if the input signal level is increased further. An amplifier is said to be “saturated” at this point and can no longer provide any additional gain. Further increases in the input signal level will result in distortion of the output signal. This results in a distortion of the received signal and a loss of information because the receiver's output signal becomes clipped. 

The 1 dBc compression point represents the power level at which the output power of the LNA is reduced by 1 dB with respect to a linear scale due to compression. One refers to the input and output power levels associated with the 1 dB compression points as IP1 and OP1, respectively. Higher 1 dB compression points indicate better linearity and less distortion of the signal.

Intermodulation points

When operating in the non-linear regime, the amplifier can also produce harmonics or intermodulation products that emerge above the noise floor. The latter effect is typically the most problematic, due to the proximity of the fundamental tones to be amplified. From the different tones, the third order intermodulation product is generally the most problematic, by falling inside the frequency band of reception. It should be noted that, considering the low level of noise in RAS receivers, intermodulation products can be detected well before the power level reaches the 1 dB compression point.

A common figure of merit for intermodulation products is the IP3, which describes third-order effects. Its value is derived as the intercept point of the fundamental (in log scale has 1 dB slope) and the third order product curve (3 dB slope). In essence, it shows the maximum amplitude of a signal that can be applied to the device before intermodulation distortion occurs.

MITIGATION MEASURES

Filtering and insertion loss

One possibility to improve the resilience of a receiver is to use filters. First active stages in radio astronomical receivers are designed primarily for optimum noise figure and secondarily for high gain, because the radio astronomical power level entering the receiver is very low. As a consequence, initial active stages lack for high robustness performance. On the other hand, subsequent amplifiers in the receiving chain usually have relatively high 1 dB compression points to handle the amplification of the previous stages. Depending on the specific architecture of the receiver, the weakest stage to get compressed earlier in the chain can be either at the initial or at end of the chain.

Filters are commonly used to increase a receiver's resilience to interference by attenuating the entire band that contains interference. However, this solution is not always applicable in radio astronomy. In particular, it is not feasible to place narrow passband filters just around RAS bands. This is especially true if the RAS band is on the first or second adjacent channel of a strong transmitter.

In electronic filter design, the insertion loss is a figure of merit that is frequently employed. This metric is defined as the ratio between the signal level without the filter () and the signal level with the filter installed (), and it is calculated in decibels. The insertion loss can be calculated as follows:

		

		(71)





Insertion loss can have a significant impact on the noise figure of a system, which can influence the overall sensitivity and performance of the system. When a component with insertion loss is added to the system, it can reduce the signal power, leading to a decrease in the SNR at the input of the component.

Ultra-low insertion loss filters

As a tentative solution, high-temperature superconductor (HTS) filters have been used at some radio telescopes. The idea is that such a filter can even be placed before the cryo-cooled LNA without affecting the sensitivity too much – HTS filters have very low insertion loss. The HTS technology is maturing, but is still costly and not flexible enough compared to the quick changes that occur in some parts of the radio spectrum. HTS filters are static components, whose response cannot be adjusted/tuned based to the given scenario. Furthermore, component tests cannot be performed at room temperature, which makes designing the filters more challenging than classical analogue filters. During the last few years a number of HTS microwave filters have been designed and fabricated for three receivers installed on the 64 m Sardinia Radio Telescope (SRT). All these filters are based on an Yttrium Barium Copper Oxide (YBCO) film laid on a Magnesium Oxide (MgO) substrate with a thickness of 0.5 mm and a dielectric constant of 9.65. The front-end receivers for which they have been developed are: a P-band receiver (305–410 MHz), a “High” C-band receiver (5.7–7.7 GHz), and a “Low” C-band receiver (4.2–5.6 GHz). Although there have been strikingly successful and hopeful deployments of HTS microwave filters at a few observatories, they cannot be seen at the moment as the universal solution for the receiver problems associated with very powerful out-of-band signals. High gain wide-band LNAs often have frequency intervals where they are not unconditionally stable, and together with low loss filters being LC circuits attenuating via impedance mismatch, this can lead to spurious oscillations, often outside the pass band of the amplifier and difficult to detect, but which can severely degrade performance. Many specific technical factors determine the feasibility of using appropriate input filters. Retrofitting such filters to existing radio astronomical receivers is often impossible and the question of their use for new receivers has to be evaluated on a case-to-case basis for each band and for each observatory.

Microwave filters

All optics systems (including the primary reflector and feed horn) necessarily act as filters to an extent, so the feasibility of filtering out harmful transmissions in the optical domain rather than the coaxial domain is also considered. The principle of optical filtering relies on waveguide components attenuating radiation below the cut-off frequency, which is inversely proportional to the dimensions of the waveguide. That is, a length of waveguide may be used as a high-pass filter, providing strong attenuation of any signals below the cut-off frequency. The caveat is that access to all frequencies below the cut-off frequency is lost, and not just the frequencies occupied by an exemplary troublesome transmission that needs to be filtered out. Waveguides cannot be employed in the application of bandstop filtering, because there is no upper cut-off frequency.

A key challenge in traditional single-pixel receiver design is the need to improve the impedance match between free space and the transition to the coaxial domain. This requires careful design of the optics system, and while the benefit of waveguide filtering is the minimisation of loss for propagated modes, the degradation of the impedance match with free space will introduce other aspects of undesirable performance reduction, such as bandpass ripple.

In conclusion, a RAS receiver is best optimised when the minimum of required waveguide components is used, such as the feed horn and OMT, and extra waveguide components introduced in an attempt to counter powerful terrestrial transmissions should be avoided.

NEW DEVELOPMENTS IN RECEIVER DESIGN

Until a couple of decades back, most existing receivers at European radio telescopes were single- or dual-pixel receivers, owing to the physical tension imposed by the cooling process. These receivers typically consisted of a single chain for each of the two polarisations needed to sample the ensemble of randomly polarised photons that are focused onto the phase centre of the receiver's feed horn. However, in order to increase the operational capacity of radio telescopes, radio receiver development has tended towards placing an array of receivers on the telescope’s focal plane. There are two basic configurations of such an array, multi-beam receivers and phased array feeds, which use a planar aperture of dozens of antenna elements (similar to active antenna systems in telecommunications, but here there is a large reflector attached – the radio telescope).

A multi-beam receiver takes the form of an array of independent traditional single-pixel receivers. Each pixel of the array is positioned on the focal plane of the telescope in such a way as to point not just along the boresight axis, but also at positions away from the boresight axis. Despite the loss of forward gain associated with placing a feed horn’s phase centre off-axis, the advantages in increasing the instantaneous field-of-view far outweigh the disadvantages in required engineering effort, mostly by enhancing the survey speed especially of large single dish radio telescopes. Examples of very successful multi-beam receivers include the Jodrell Bank Multi-beam that operates in the protected L-band, and the Effelsberg 7-Beam, which is used extensively as a survey instrument, for both spectral lines and pulsar research. 

Phased array feed (PAF) receivers (sometimes also called focal-plane arrays, or colloquially radio cameras) are a development of the multi-beam concept, and take the form of a planar array of detectors that populate the focal plane. The principle of the PAF is to maximally sample the complex voltage field on the focal plane, such that the axis of peak forward gain may be determined in post-processing by appropriately weighting the complex voltages received by every antenna element of the PAF, e.g, in FPGAs. This allows for an arbitrary number of overlapping beams independent of any optical limitations that a feed horn may impose reducing off-axis effects significantly.

Multi-beam receivers and phased array feed receivers are, by definition, optimised to maximise the number of beams in order to maximise science capability, and are typically limited by size and weight considerations. As such, their cryostats are densely populated with RF components, which further drives the development of each antenna’s chain of RF components towards simplicity; a typical PAF will consist of an antenna, an LNA, a bandpass filter, and either a digitiser or further electronics to interface with an RF-over-Fibre system. As such, PAFs are particularly susceptible to being driven nonlinear.

In the context of the potential future requirement to retrofit one or more band-stop filters in order to reject specific powerful transmissions, such a retrofit may be impossible without a substantial redesign. While an alternative approach is to limit the fraction of sky in which such a receiver may be operated, this places heavy limits on the science capability of the instrument. The potential future omnipresence of such transmissions may simply make operation of the receiver unfeasible. 

In summary, placing a requirement on the RAS to make their receivers robust to present and future strong active transmitters will eventually direct the RAS away from developing towards maximising the science capability of their instruments – their raison d’etre – instead towards optimisation for maximum interoperability with commercial users of the spectrum, with necessarily heavily-limited science capability.

PRACTICAL EXPERIENCE

Jodrell Bank Observatory (JBO, UK) has been badly affected by powerful ground-based transmissions. The L-band receivers on the 76 m Lovell, and 25 m Mark II Telescopes each have an unfiltered observing bandwidth ~1 GHz, however long-standing transmissions necessitate increasingly strong bandpass filtering to their current designs with ~500 MHz bandwidth centred on ~1500 MHz.

IMT allocations in the 1452–1472 MHz band for LTE downlinks became active in 2018. The transmissions are sufficient to force the receiver system into the nonlinear regime for all directions except those for which the primary reflector itself provides shielding from the transmitter. In other words, more than half of the sky was lost.

It was later discovered that backend components were the first to be driven nonlinear, and as such it was possible to fit bandstop filters behind the LNA rather than ahead of them. These filters act in addition to the existing band-defining filters and their use necessarily degrades system performance at the ~10% level, which reduces the observatory’s effectiveness in its science mission.

The extent of the problem the 1452–1472 MHz transmissions caused can be summarised as follows:

1. The bandstop filters had to be custom-designed and could not be obtained off-the-shelf.

Consequently, the unit price was high and the lead time was long (~months).

Further, the cryostats for each receiver needed to be modified to accommodate the bandstop filters, which required extensive engineering time.

The total expenditure required to adapt the L-band systems to co-operate with the new transmissions in hardware and effort is estimated at ~5% of the observatory’s total operating budget, which is unsustainable for any potential future requirement to repeat this operation for other transmissions.

At the 100 m radio telescope at Effelsberg (Germany), similar experience was made with a very broad L-band receiver (called UBB). Originally, it was designed to offer frequencies between 600 MHz and 3 GHz, but strong interferers below 1 GHz drove the system into non-linearity and measurements were impossible. Later the receiver was significantly modified and now covers a range between about 1.2 and 5 GHz. Several well-known IMT bands, e.g., at 1800 and 2100 MHz are filtered out with stop-band filters. Fortunately, the LNA of the UBB is robust enough to allow placing these filters after LNA, such that the overall system performance is not heavily affected. However, since late 2022 a nearby IMT base station was upgraded to provide LTE/4G supplemental downlinks at 1452–1472 MHz, which causes intermodulation products throughout the observing bands. Even worse, for the Effelsberg “powerhorse”, the 21 cm 7-feed receiver, the system is completely saturated for more than half of the accessible sky. This receiver is used for more than 25% of the time, as it provides superb survey capabilities. Equipping this system with new filters would not only be very costly (as there are 14 independent channels), but it would also require enormous efforts to ensure that the calibration can be done consistently for old and newer data. Surveys can take several years, and it is usually not possible to change the system properties in the middle of the observations. It is currently not even clear whether stop-band filters after the LNAs would suffice. If filters needed to be put before the LNAs, this would have a huge impact on the sensitivity and require hundreds if not thousands of additional measurement hours to finish the surveys.

Some radio telescopes in Europe have suffered from the high-power signal received from radars very close to the radio telescopes. In Ny-Ålesund (Norway) and in Onsala (Sweden) observatories, the signal received from ship-borne radars close to the radio telescopes were so powerful that it drove the LNAs into their breaking point and, hence, were destroyed. In the RAEGE Santa María station (Azores, Portugal) a nearby radar makes it impossible to use the broadband receiver (VGOS radio telescope, 2-14 GHz) without any preventive measure to avoid driving the LNAs into the breaking point. Measurements were only possible after the installation of a filter in the cryostat at the input of the LNAs. This solution implies a degradation of 17% of its sensitivity. VGOS radio telescopes are planned to observe 4 different sub-bands of 1 GHz width in the 2-14 GHz range. With the installation of the filter, the lowest sub-band has been sacrificed to be able to observe in the three other ones.

[bookmark: _Toc129956005][bookmark: _Toc155274012]List of References



[bookmark: _Ref136617411][bookmark: _Ref131501303][bookmark: _Ref129083405][bookmark: _Ref213741794][Draft] ECC Recommendation(24)XX: "Receiver resilience to transmission on adjacent frequency ranges"

[bookmark: _Ref116468188]ERC Recommendation 74-01 “Unwanted emissions in the spurious domain”

ECC Report 310 “Evaluation of receiver parameters and the future role of receiver characteristics in spectrum management, including in sharing and compatibility studies”

[bookmark: _Ref131502280][bookmark: _Ref85105739]ETSI EN 303 340 V1.2.1: "Digital Terrestrial TV Broadcast Receivers; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU ";

[bookmark: _Ref131515720]ITU-R F.1094-2: “Maximum allowable error performance and availability degradations to digital fixed wireless systems arising from radio interference from emissions and radiations from other sources”

[bookmark: _Ref85105766]ITU-R F.758-7: “System parameters and considerations in the development of criteria for sharing or compatibility between digital fixed wireless systems in the fixed service and systems in other services and other sources of interference”

[bookmark: _Ref129084589]Recommendation ITU-R SM.329-12 “Unwanted emissions in the spurious domain”

[bookmark: _Ref129085360][bookmark: _Ref131502294]ETSI EN 301 908-14 V13.1.1: "IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 14: Evolved Universal Terrestrial Radio Access (E-UTRA) Base Stations (BS)”)";

[bookmark: _Ref129085793][bookmark: _Ref131502303][bookmark: _Ref129085379]ETSI EN 302 217-2 “V3.3.1: "Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 2: Digital systems operating in frequency bands from 1,3 GHz to 86 GHz; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU ;

[bookmark: _Ref129085448][bookmark: _Ref131502308]ETSI EN 301 502 V12.5.1: "Global System for Mobile communications (GSM); Base Station (BS) equipment; Harmonised Standard covering the essential requirements of article 3.2 of the Directive 2014/53/EU”"

[bookmark: _Ref129091936]Recommendation ITU-R SM. 1539-1 " Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU R SM.329"

[bookmark: _Ref131516418][bookmark: _Ref136601447]ETSI TS 136 141: "LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing (3GPP TS 36.141 version 15.4.0 Release 15)”

[bookmark: _Ref131515858]ETSI EN 301 390: " Fixed Radio Systems; Point-to-point and Multipoint Systems; Unwanted emissions in the spurious domain and receiver immunity limits at equipment/antenna port of Digital Fixed Radio Systems"

[bookmark: _Ref131516039][bookmark: _Ref136602213]ECC Report 146: "Compatibility between GSM MCBTS and other services (TRR, RSBN/PRMG, HC-SDMA, GSM-R, DME, MIDS, DECT) operating in the 900 and 1800 MHz frequency bands"

[bookmark: _Ref131516093]ECC Recommendation (02)01: "Specification of reference receiver performance parameters"

[bookmark: _Ref131516186]ECC Report 252: "SEAMCAT Handbook"

[bookmark: _Ref136603483]ECC Recommendation (02)05: "Unwanted emissions"

[bookmark: _Ref136610670]ECC Recommendation (18)01: "Radio frequency channel/block arrangements for Fixed Service systems operating in the bands 130-134 GHz, 141-148.5 GHz, 151.5-164 GHz and 167-174.8 GHz"

[bookmark: _Ref136610676]ECC Recommendation (18)02: "Radio frequency channel/block arrangements for Fixed Service systems operating in the bands 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz and 111.8-114.25 GHz"

[bookmark: _Ref136615705]Recommendation ITU-R RA.611: "Protection of the radio astronomy service from spurious emissions"

[bookmark: _Ref136615710]Report ITU-R RA.2259: "Characteristics of radio quiet zones"

[bookmark: _Ref136615862]Report ITU-R RA.2188: "Power flux-density and e.i.r.p. levels potentially damaging to radio astronomy receivers"

[bookmark: _Ref136616153]Recommendation ITU-R RA.769: "Protection criteria used for radio astronomical measurements"











image1.png

Relative attenuation (dB)

Victim and interfering signal amplitude (dBm)

Interfering signal received by the victim receiver (/,);
frequency offset = 30 MHz

10 for+Bi/2
= ([ sty ) ros
0 foi=B:/2
-10 ¢
(dB)
20 + Towe

d:(f)rif) /ILR

733 738 743 748 753 758 763 768 773 778 783

Frequency (MHz)

Victim receiver selectivity mask (relative attenuation)  sel,, (f)

Received victim signal spectrum  sd,, (f)

Interfering signal spectrum s, ()

Interfering signal power received by the victim receiver I, = I5o—c + In—cn

788 793







image2.emf

Step 2:


 


Calculate receiver 


desensitisation (M) 


Step 1:


 


Calculate Noise (N)


Step 3:


 


Determine ILR


Step 4:


 


Define the frequency offset selectivity 


(FOS)


Step 5:


 


Derive the receiver 


blocking level (I


blk


) or adjacent 


channel protection ratio (PR


adj


-


ch


) 


Yes


OOB or reciprical 


spurious domain?


Spurious


ILR considered 


infinite?


Yes


Calculate I


blk


 using 


Eq. 26


OOB


Calculate PR


adj-ch


 


using Eq. 27


No


Calculate I


blk


 using 


Eq. 25


Start


Is Noise Figure 


NF known?


Yes


Calculate N using 


Eq. 20


Calculate N using 


Eq. 21


No


Is M known?


Calculate M (Eq. 22)


No


Can M be derived 


from ETSI HS?


Yes


No


Operational 


environment 


driver?


Noise


Interference


Assume M = 15 dB Assume M = 3 dB ETSI HS


Is external info available on 


interfering system?


No


Define interfering signal 


and interference 


scenario based on ETSI 


HS


Yes


Define interfering signal and 


interference scenario based 


on external info


Info on existing/


planned deployments 


or technical reports ETSI HS


Choice of source of  interfering 


signal and interference scenario


ETSI HS


Annex 7


Define interfering signal and 


interference scenario using 


Annex 7


OOB or reciprocal 


spurious domain?


OOB


Choice of ILR 


source


External 


documents


Define ILR from 


external documents


Measurements


Define ILR from 


measurements 


EC/ECC Decisions, 


ECC/ITU-R 


Recommendations, 


CEPT/ECC Reports, 


ETSI HS/TS/TR


Spurious


Choice of ILR 


source


External 


documents


Define ILR from 


external documents


Annex 7


Define ILR from 


Annex 7


EC/ECC Decisions, 


ECC/ITU-R 


Recommendations, 


ESTI HS/TS/TR


Interfering signal and 


interference scenario based 


on ETSI HS?


No


Interfering 


signal type


Yes


Similar


 to wanted


Define X based on  


ETSI HS


ETSI HS


CW


Calculate FOS from 


Eq. 24


Define FOS 


according to Eq. 23


Define X based on  


maximum acceptable 


increase of the interfering 


signal power




Microsoft_Visio_Drawing.vsdx

Method for Receivers Resilience
Step 2: Calculate receiver desensitisation (M)
Step 1: Calculate Noise (N)
Step 3: Determine ILR
Step 4: Define the frequency offset selectivity (FOS)
Step 5: Derive the receiver blocking level (Iblk) or adjacent channel protection ratio (PRadj-ch)


Yes
OOB or reciprical spurious domain?
Spurious
ILR considered infinite?
Yes
Calculate Iblk using Eq. 26
OOB
Calculate PRadj-ch using Eq. 27
No
Calculate Iblk using Eq. 25
Start
Is Noise Figure NF known?
Yes
Calculate N using Eq. 20
Calculate N using Eq. 21
No
Is M known?
Calculate M (Eq. 22)
No
Can M be derived from ETSI HS?
Yes
No
Operational environment driver?
Noise
Interference
Assume M = 15 dB
Assume M = 3 dB
ETSI HS
Is external info available on interfering system?
No
Define interfering signal and interference scenario based on ETSI HS
Yes
Define interfering signal and interference scenario based on external info
Info on existing/planned deployments or technical reports
ETSI HS
Choice of source of  interfering signal and interference scenario
ETSI HS
Annex 7
Define interfering signal and interference scenario using Annex 7
OOB or reciprocal spurious domain?
OOB
Choice of ILR source
External 
documents
Define ILR from external documents
Measurements
Define ILR from measurements
EC/ECC Decisions, ECC/ITU-R Recommendations, CEPT/ECC Reports, ETSI HS/TS/TR
Spurious
Choice of ILR source
External 
documents
Define ILR from external documents
Annex 7
Define ILR from Annex 7
EC/ECC Decisions, ECC/ITU-R Recommendations, ESTI HS/TS/TR
Interfering signal and interference scenario based on ETSI HS?
No
Interfering signal type
Yes
Similar
 to wanted
Define X based on  ETSI HS
ETSI HS
CW
Calculate FOS from Eq. 24
Define FOS according to Eq. 23
Define X based on  maximum acceptable increase of the interfering signal power






image3.png

o (dBm)

-30

-40

-50

&
3

4
=]

-80

-90

-100

Variation of I, of a receiveras a function of M
129.43 dBm, Rx_sens=-90.43 dBm

6 9 12 15 18 21 24 27
M (dB)

Rx Iblk; N=-129.43 dBm; ACS=60 dB, ACLR=70 dB

Rx Iblk

Rx Iblk

- - - - Linear approximation of Iblk (dBm)

30

33






image4.png

-30

-40

& i
3 3

o (dBm)

4
=]

-80

-90

Variation of I, of a receiveras a function of M
N=-129.43 dBm, Rx_sens=-90.43 dBm

.

12 15 18 21 24 27
M (dB)
Rx Iblk
Linear approximation of Iblk (dBm)
Two Iblk values calculated at M=3 and M=15

30

33






image5.png

«+———— Unwanted Emissions

<« 0OBDomain — ¢ Spurious in 5
(OOB predominate) (Spurious predominate)

= Spurious Emissions

‘0OB Emissions

50 % 250 %

of the necessary bandwidth of the necessary bandwidth Frequency






image6.png

Cllagjcn (dB)

Comparison between MRR and NFD

0 3 6 9 12 15

18 21 24 27 30 33

M (dB)

——PRadj-ch (dB) calculated using MRR

——PRadj-ch (dB) calculated using NFD






image7.png

Spectrum

Ref Level -30.00 dem @ RBW 1kHz
SWT 3.55 VBW 10 kHz Mode Auto Sweep

-44.71 dBm
890.0000 MHz

-50 dBm—j

-60 dBm;

-70 dBm—j

-80 dBm—

-90 dBm—j

-100 dBm—

-120 dBm—

CF 890.0 MHz 1001 pts Span 35.0 MHz
Channel Power

Channel Bandwidth Offset Power
TX1 (Ref) 5.000 MHz -3.50 dBm
Tx Total -3.50 dBm
Channel Bandwidth Offset Lower Upper
Adj 5.000 MHz 5.000 MHz -46.65 dB -46.73 dB
Alt1 5.000 MHz 10.000 MHz -63.58 dB -63.86 dB

Measuring... LLLLLL







image8.png

Normalised amplitude (dBm/KHz)

5 MHz reference interfering signal spectrum mask for 30 MHz < f <1 GHz

-18 16 14 12 10 8 6 4 2 0 2 4 6 8

Frequency (MHz)

Norm Measured signal ~ —— Approximated Rl spectrum mask







image9.jpeg

Normalised amplitude (dBm/KHz)

5 MHz reference interfering signal spectrum mask for 1 GHz < f < =6 GHz

18

6 44 12 10 8 6 4 2 0 2 4 6 8 10 12 14 16
Frequency (MHz)

Measured signal spectrum  —— Approximated spectrum mask

18






image10.png

Normalised amplitude (dBm/KHz)

-20

-30

-40

-50

-60

-70

-80

Interference configuration for defining receivers resilience levels
2 MHz victim Rx and 5 MHz interfering Tx
(Interfering signal OOBD/SD boundary at + 12.5 MHz (2.5*5 MHz))

-18

16 14 12 10 8 6 4 2 0 2 4 6 8 10 12 14 16 18
Frequency (MHz)
——RIsignal spectrum for 30 MHz << 1 GHz ——RIsignal spectrum for 1 GHz <

Victim Rx - Interfering Tx freq offset = 3.5 MHz Victim Rx - Interfering Tx freq offset = 13.5 MHz.







image11.png

38,00 ai
700

1008 VD-25]

31210-10

BF-aT-20e1

a1
'S

148271 4B
53,0000 8

10000 48
35 305 )

500

haoo

300

900

800

G s 55000 -

Sop 000 de

000 ra
000

1003v0-25]

12 D-0105]

FaT2091

ST

3a 1780 a

50 dor|

000
000

000

000

1000

2000

000
000

5000
O st 55000 90

“Siop 35000 d6m






image12.emf














image13.emf

























ECC/REC/(YY)XX Page 4

ECC/REC/(YY)XX Page 5

+







ECC Recommendation (YY)XX































[bookmark: Text7]Receiver resilience to transmission on adjacent frequency ranges	 

DD Month YYYY (Arial 9pt bold)	

[last updated: DD Month YYYY) (Arial 9pt) [date of the latest update]]







[image: cept logo][image: ecc_logo][image: cept logo][image: ecc_logo]

[image: cept logo]SE21[image: cept logo]E21(23)XX



[image: ecc_logo][image: ecc_logo]



Draft ECC REPORT XXX









Page 3



Edition

Edition

[bookmark: _Hlk90367317]Introduction

For consideration at WG SE/ECC:”ETSI HARMONISED STANDARDS” need to be referred to simply as ”ETSI STANDARDS” or other more appropriate terminology.

This ECC Recommendation provides a framework for continuous improvement of receiver resilience in the spectrum domain for a wide range and category of receivers to facilitate the reliable functioning of receivers in spectrum that will become more densely occupied over time[footnoteRef:2]. The methodology in this Recommendation is intended to apply to all frequency bands in the radio spectrum. The initial recommendations for receiver resilience values for specific radio frequency ranges are listed in the individual annexes[footnoteRef:3].  [2:  The recommended receiver resilience levels defined in this Recommendation are for receivers which are covered by ETSI Harmonised Standards, or equivalent national provisions.]  [3:  The recommended receiver resilience levels defined in this first version of the Recommendation focus on the systems and frequencies in annexes 2 to 6 (these systems operate in certain bands between 30 MHz and 5725 MHz).] 


The increasing demand for existing and new services has directly led to a corresponding increase in demand for spectrum access and a need for administrations and the services to which the spectrum is allocated to use the scarce spectrum resource more efficiently. To facilitate the efficient use of spectrum, the characteristics and resilience of receivers are considered in assessing the possibilities for sharing and compatibility between radiocommunication devices, services, systems and applications.

Receiver characteristics are as important as transmitter characteristics for enabling efficient spectrum usage.  Receiver characteristics determine the resulting resilience and are also a basic requirement to carry out sharing and compatibility studies between services using the same frequency band (whether they overlap partially or completely), using adjacent frequency bands or frequency bands further away in the spurious domain. Improving receiver resilience will facilitate compatibility as the spectrum becomes more densely occupied.

This ECC Recommendation specifies values of receiver resilience to transmission on adjacent frequency ranges that should be considered by CEPT/ECC groups for use in compatibility studies. This ECC Recommendation additionally specifies recommended levels of receiver resilience which should be considered by ETSI when developing the Harmonised Standard.

ETSI Harmonised Standards limits are equipment minimum conformance requirements where the Harmonised Standard is used as the market access mechanism. In practice receivers have better levels of resilience to transmission on adjacent frequency ranges than the limits in the Harmonised Standards. 

[bookmark: _Ref127966081]When there is a difference between the receiver resilience limits defined in an ETSI Harmonised Standard and the recommended receiver resilience levels given in this ECC Recommendation, in particular if the limits in the standard are less stringent, the iterative, consultative procedure given in the ETSI/ECC Memorandum of Understanding[footnoteRef:4] should be followed. This procedure should consider the interaction between technical parameters, spectrum efficiency, regulatory and economic aspects. More specifically CEPT/ECC assumes the following procedure would be useful: [4:  https://cept.org/ecc/mous-and-lous-between-ceptecc-former-erc-and-other-organisations ] 


1. The relevant technical bodies in ETSI[footnoteRef:5] and CEPT/ECC[footnoteRef:6] can liaise with each other whenever they believe changes to this Recommendation or an ETSI Harmonised Standard, are required; [5:  Currently ETSI TC ERM.]  [6:  Currently CEPT/ECC WG SE.
] 


2. The ETSI liaison statement should be supported by appropriate technical justification and other relevant information. This should include information on economic and market related issues concerning the proposal. ETSI should also provide any information on system spectrum efficiency that they may have available to support their case; 

3. The CEPT/ECC liaison statement should include the implications of the proposal on spectrum engineering parameters such as: efficient use of the spectrum, requirements of existing services, sharing and compatibility, and other regulatory issues;

4. The proposal should be considered in the spirit of the ETSI/ECC MoU with dialogue, full consultation and an iterative process if necessary. Ideally, this process should be completed within six months. The conclusions should be mutually acceptable and neither party should feel that its views have been disregarded; 

5. When consensus is achieved the results should be recorded in a revision of this Recommendation and/or revision of the appropriate Harmonised Standard. 



This Recommendation should be reviewed regularly, typically every three years and at least not more than every five years, considering changing spectrum use and needs, technologies and regulatory requirements. This review should involve consultation with the relevant technical and Working Groups within CEPT/ECC and ETSI.

[bookmark: Text4]ECC recommendation of YY(No) on Receiver resilience to transmission on adjacent frequency ranges 

“The European Conference of Postal and Telecommunications Administrations, 

[bookmark: _Hlk45191033]considering 

that the demand for spectrum is increasing, with increasing demand for both existing and new services, systems and applications;

that the ECC Strategic Plan for the period 2020-2025 in 3.2 states that the ECC will develop or update ECC Recommendations on appropriate characterisations of both transmitters and receivers, and in undertaking this work, the ECC will seek to intensify its co-operation with ETSI in order to foster the development of efficient sharing conditions in the future;

that the RF working environment may change with the evolution of services and applications in use;

that receiver resilience means adjacent channel protection ratio (in the out-of-band domain) and blocking level (in the reciprocal spurious blocking domain (RSBD), defined later in these considerings) of the receiver;

that like transmitter characteristics, receiver resilience can directly affect the efficient use of spectrum and it is necessary to have resilient receivers to have high immunity against adjacent interference, not just co-channel interference;

that higher values of frequency offset selectivity (FOS) provide better receiver resilience to transmissions on adjacent frequency ranges when considered in an optimal balance with receiver sensitivity;

that to achieve outcomes that enable the efficient use of spectrum it is necessary to make realistic assumptions on receiver resilience for sharing and compatibility studies;

that efficient use of spectrum is enabled by technical requirements on receiver resilience documented in industry specifications, harmonised standards and in legal instruments permitting radio equipment to be made available on the market; 

that the receiver sensitivity of a system is linked with its associated transmitter power levels because both of these values contribute to the wanted signal link budget;

that the present version of Annexes 2 to 6 of this Recommendation defines recommended receiver resilience levels within the frequency range 30 MHz-5725 MHz.

that receivers operating outside their linear mode are not in the scope of this Recommendation;

that there is a need to consider the fundamental receiver characteristics defined by a set of inter-related parameters;

that the following inter-related parameters be used in the linear range (i.e. below the overloading threshold) of the receiver to define its resilience: “N”, “M”, “ILR” and “FOS” as described in Annex A1.1

that more sophisticated models may be developed expanding on this set of parameters in the future or if required on a case-by-case basis;

that, when receiver resilience levels are not defined in this Recommendation, the receiver resilience levels of some receivers may be found in ECC/CEPT deliverables, ETSI Harmonised Standards or ITU-R deliverables and can also be considered for compatibility studies;.

that Annex 1 of Appendix 3 of the Radio Regulations [1] and Recommendation ITU-R SM.1539 [3] defines the boundaries between the out-of-band and spurious domains of a transmitter (i.e. the spurious domain applies at a frequency separation of more than ±250% of the transmitter bandwidth from its centre frequency);

that in this Recommendation the out-of-band and spurious domains definitions, usually applied to transmitters, are also applied to the related receiver (i.e. the recommended levels are applied at a frequency separation of more than ±250% of the receiver channel bandwidth from its centre frequency when operating at the receiver operating band edge, see Figure 5 in Section A7.3.2);

that, for channelised systems, the concept of “reciprocal spurious blocking domain” implies that the victim receiver channel is in the spurious domain of the interfering transmitter and reciprocally the interfering transmitter channel is in the spurious domain of the wanted transmitter of the victim system. For non-channelised systems, the definition in Section A7.3.2 applies;

that ECC Report 310 [2] evaluated receiver parameters and the future role of receiver characteristics in spectrum management, including how they are used in sharing and compatibility studies;

that several Recommendations (ERC Recommendation 74-01 [4] and ECC Recommendation (19)02 [5]) and ECC Report 249 [6], were developed, based on assumptions about transmitter characteristics, to enable efficient spectrum usage;

that as noted in ECC Recommendation (19)02 [5], using ‘worst case’ or ‘minimum’ assumptions based on conformance limits for receivers in sharing and compatibility studies can lead to pessimistic results and consequently less efficient use of spectrum;

that typical receiver resilience, in practice, may be better than the values specified in harmonised standards, for example, adjacent channel protection ratios or the selectivity of a receiver. Therefore, measured receiver parameters may also be used as inputs to this Recommendation;

that measurements for sharing and compatibility studies should take into account a sufficient number and range of typical devices over their full expected working range of frequencies and signal levels as far as possible;

[bookmark: _Ref149638173]that additional values of receiver resilience, e.g. based on measurements, are intended to be provided in a dedicated annex in a future update of this ECC Recommendation;

that CEPT and ETSI have developed a Memorandum of Understanding describing the relative responsibilities of the two bodies3;

[bookmark: _Ref129073879]that the impact of pulse modulated, high peak to average power ratio transmissions, for example transmissions from radar systems, on receivers, is not in the scope of this Recommendation;

that in some cases, CEPT/ECC could decide not to offer protection for receivers possessing resilience below a certain level;

that limits of unwanted emissions in the spurious domain apply to frequencies given in several Recommendations (ERC Recommendation 74-01 [4] and ECC Recommendation (02)05 [23]);

that ETSI needs a transition period to implement this ECC Recommendation on future revisions of Harmonised Standards and on future devices for which new Harmonised Standards will be developed.






recommends 



1. that this ECC Recommendation should be used as a CEPT/ECC framework for continuous improvement of receiver resilience to transmission in adjacent frequency ranges to help enhance efficient use of spectrum;

2. that the methodology and levels of receiver resilience in this Recommendation should be used in compatibility studies.

3. that the general method defined in ANNEX 1: which has been used to derive the minimum levels of receiver resilience in the relevant annexes of this Recommendation, should also be used when  annexes are added or updated. 

4. that receivers which are covered by ETSI Harmonised Standards or equivalent national provisions, should be considered when adding to or updating the tables of Recommended receiver resilience values. 

5. [bookmark: _Ref148532005]that the recommended levels of receiver resilience only apply in the reciprocal spurious blocking domain and should be considered for ETSI Harmonised Standards developed or revised after the date at which this Recommendation has been approved;

[bookmark: _Hlk130835708]NOTE 1: in the case where these recommended levels are more stringent than the limits given in existing ETSI Harmonised Standards, a revision of those limits in the ETSI Harmonised Standards may need to be considered by ETSI;

NOTE 2: when CEPT/ECC or ETSI considers that the recommended levels are inappropriate for a particular service, then an agreement on alternative recommended levels should be reached in accordance with the MoU between CEPT/ECC and ETSI.;

NOTE 3: when an ETSI Harmonised Standard is updated with new limits for receiver resilience, then, under the ETSI/ECC MoU, ETSI is invited to inform CEPT/ECC of the relevant changes on a liaison basis. 

6. that the values of receiver resilience in the out-of-band domain specified in the relevant annexes, are for compatibility studies (Note 3 of recommends 5 also applies to this recommends);

7. that additional receiver resilience values, e.g. based on measurement data, can be used as alternative inputs for compatibility studies (also see considering x));

8. that when CEPT/ECC conducts sharing and compatibility studies and identifies that interference may occur and the receiver resilience is a limiting factor, then improving the receiver resilience should always be investigated as a possible mitigation in the study;

9. that, where an ECC deliverable or communication providing specific requirements from CEPT defining further constraints for a particular technology or application exists, the relevant values contained therein should take precedence over values in the present Recommendation;

10. that based on considering cc) this ECC Recommendation enters into force on [2nd June 2025] for new adopted ETSI work items on creation or revision of Harmonised Standards.	Comment by France: Period of transition. The date can be discussed.



[bookmark: _Ref534710302][bookmark: _Ref129075239]Table 1: List of annexes 

		Annex Title

		Annex Number



		Method for receiver resilience (MRR) used to derive blocking and selectivity levels 

		ANNEX 1:



		Receiver resilience values in the adjacent band (out-of-band domain) and recommended blocking levels (reciprocal spurious blocking domain) for:



		Broadcasting service

		ANNEX 2:



		Fixed service

		ANNEX 3:



		Land Mobile service

		ANNEX 4:



		Short Range Devices

		ANNEX 5:



		Wireless Access Systems (WAS)/Radio Local Area Networks (RLAN)

		ANNEX 6:



		Reference interfering signal and a single interference scenario to be used in the calculation of recommended receivers resilience levels using MRR

		ANNEX 7:



		Derivation of the method for receiver resilience (MRR)

		ANNEX 8:







Note: it is intended that this list of annexes will in the future contain an annex with additional levels of receiver resilience for use in compatibility studies.



Note: 

Please check the Office documentation database https://docdb.cept.org/ for the up to date position on the implementation of this and other ECC Recommendations.

[bookmark: _Ref127784900]Method for receiver resilience (MRR) used to derive blocking and selectivity levels

[bookmark: _Ref127784718]Basics of the method

 The definitions below, with the exception of “Receiver desensitisation” and " Reciprocal spurious blocking domain", (M)”, are taken from ECC Report 310 [2] with some small modifications and are used in this ECC Recommendation for receivers working in their linear range.



		Term

		Definition



		Blocking

		A measure of the receiver capability to receive a wanted signal without exceeding a given degradation due to the presence of an unwanted signal at any frequency other than those of the spurious responses or of the adjacent channels and it is defined as the maximum interfering signal level expressed in dBm reducing the specified receiver sensitivity by a certain number of dBs (desensitisation).



		Frequency Offset Selectivity (FOS)

		A measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted adjacent signal at a given frequency offset from the centre frequency of the assigned channel. In this context, it is defined as the ratio of the receiver filter attenuation on the offset frequency to the receiver filter attenuation on the assigned channel frequency (normally a positive number in dB).

FOS is of general use for any mixed wanted and unwanted signal situation.



		Interference Leakage Ratio (ILR)

		The ratio of the filtered mean power centred on the assigned channel frequency to the similarly filtered mean power centred on a given frequency offset.



		Receiver Interference Ratio (RIR)

		[bookmark: _Hlk147132415]The ratio of the in-channel interference power on a given frequency offset to the interference power received by the victim receiver.



		Receiver desensitisation (M)

		Reduction in the signal to noise ratio of the receiver or a reduction in the effective sensitivity in the presence of an interfering signal, given in dB. It corresponds to the ‘noise rise’ due to the interfering signal.



		Receiver noise floor (N)

		The total noise power at the receiver including the effect of thermal noise and the receiver noise figure.



		Reciprocal spurious blocking domain (RSBD)

		Implies that the victim receiver channel is in the spurious domain of the interfering transmitter and reciprocally the interfering transmitter channel is in the spurious domain of the victim receiver.







MRR derives the frequency offset interfering signal level (Iin-ch) at the receiver input and consequently the blocking level (Iblk) and the adjacent channel protection ratio (C/Iadj-ch) of the receiver, for given values of M, N, ILR and FOS. Iin-ch is referred as Iblk when calculating the blocking level and as Iadj-ch when calculating the adjacent channel protection ratio (PR), where:

Iin-ch: frequency offset interfering signal power measured in the interferer channel bandwidth at the receiver input before RF/IF/BB filtering. Note that when defining the resilience levels of a receiver, Iin-ch is often referred to as Iadj-ch or Iblk depending on the frequency offset between the useful and interfering signals.

Iblk: blocking interfering signal power measured in the interferer channel bandwidth at the receiver input before RF/IF/BB filtering;

Iadj-ch: adjacent channel interfering signal power measured in the interferer channel bandwidth at the receiver input before RF/IF/BB filtering.



The bandwidth of the wanted received signal and the bandwidth of the interfering signal need to be determined in advance. See Annex 7 for more information on defining receiver and transmitter bandwidth. The frequency offset is the offset between the centre frequency of the wanted received signal bandwidth and the centre frequency of the interfering signal bandwidth.

For the frequency offsets beyond the second adjacent channel, the blocking level :



		

		[bookmark: _Ref129955827][bookmark: _Ref131508314](1)





Or, for first and second adjacent channels frequency offsets, the carrier to interference ratio () as follows:



		



		[bookmark: _Ref129248597](2)





Where:



N: Noise floor of the victim receiver;

M: Victim receiver maximum acceptable desensitisation when the interferer is at a given frequency offset from the receiver;

FOS: Frequency offset selectivity of the victim receiver;

ILR: Leakage power ratio of the interfering signal at offset frequency;

C: useful signal level received by the victim receiver;

: receiver sensitivity level, specified for the victim receiver.



The definitions of ILR and FOS can be found at the beginning of this section. 

Application of the method

The steps below should be followed to derive the receiver blocking level (Iblk) or the adjacent channel protection ratio (PRadj-ch=C/Iadj-ch):

Step 1	Calculate the noise floor (N) of the receiver:

Every radio receiver is subject to a noise floor expressed in dBm that can be described using the following equation:



		



		[bookmark: _Ref155191548](3)





Where:

N: receiver noise floor (dBm);

k: Boltzmann constant in Joules per Kelvin (1.381×10-23);

T: temperature in degrees Kelvin (for common terrestrial radio receivers, 290 K can be used);

B: receiver bandwidth in Hertz;

kTB: receiver thermal noise in Watts;

NF: receiver Noise Figure (dB).



[bookmark: _Hlk155187469][bookmark: _Hlk147129371]In cases where the noise figure is not known, the noise floor may also be derived from the receiver sensitivity provided that the interference signal bandwidth is equal to that of the receiver signal bandwidth: 

		



		(4)







Step 2	Calculate the receiver desensitisation (M) (go to Step 3 if M is known):

M is often found in ETSI Harmonised Standards, either explicitly mentioned, or calculated from the required “Receiver sensitivity” (in absence of interference) and the “Minimum wanted received signal level” required in the blocking test through the equation:



		

		(5)







If it is not possible to find/calculate M from the ETSI Harmonised Standards as above, the following values: M=  3 dB and/or M = 15 dB should be assumed. The M = 3 dB value may be assumed for systems deployed in a “noise driven performance” operational environment, while the M = 15 dB may be assumed for systems deployed in an “interference driven performance” operational environment.

Note that the value of M should be chosen based on the expected operational range of the receiver.

Step 3	Determine the ILR value to be used in Step 4:

0. Identify the interfering signal and interference scenario:

The relevant interfering signal and interference scenario can be identified based on:

the existing or planned deployment of victim and interfering systems and the compatibility studies presented in various CEPT/ECC, ETSI and ITU-R technical reports;

or, if the information in the previous paragraph is not available, information may be sourced from either of the following:

[bookmark: _Hlk126585981]from the intra/inter-system interfering signals and interference scenarios defined in ETSI Harmonised Standards; or 

from the reference interfering signal[footnoteRef:7] and interference scenario defined in ANNEX 7:.  [7:  In this version of the Recommendation, the reference interfering signal has only been applied to the calculation of the recommended receiver resilience levels in the reciprocal spurious blocking domain] 


Determine the ILR value

Once the relevant interfering signal and interference scenario are identified, the following approach can be used to determine the appropriate ILR value:

If the victim receiver channel is fully or partially in the out-of-band (OOB) domain of the interfering signal (see A7.3.2) use either of the following:

the ILR (ACLR) value defined in existing EC/ECC decisions, ECC/ITU-R Recommendations, CEPT/ECC Reports and ETSI Harmonised Standards/technical specifications/reports;

the ILR value determined by measurements.

If the victim receiver channel is in the reciprocal spurious blocking domain (RSBD) (see A7.3.2) use either of the following: 

[bookmark: _Hlk147129526][bookmark: _Hlk147129575][bookmark: _Hlk147129605]the ILR value derived from the in-channel power of the interfering signal and the spurious emission levels defined in external documents including: the existing EC/ECC decisions, and ETSI Harmonised Standards/technical specifications/reports, ECC/ITU-R Recommendations (e.g. ERC Recommendation 74-01 [4], Rec. ITU-R SM.329-12 [7]); 

the ILR value derived from the reference interfering signal defined in ANNEX 7:



Step 4	 Define the frequency offset selectivity (FOS) of the receiver based on the following options:

Depending on the scenario, FOS can either be derived directly from the ETSI Harmonised Standard, or according to:

		

		[bookmark: _Ref131509239](6)







where the value of X depends on the maximum acceptable increase of the interfering signal power () at the receiver input due to  e.g.:

X = 0 dB would correspond to	

X = 10 dB would correspond to	



where:



: frequency offset interfering signal out-of-channel power received by the victim receiver;

: frequency offset interfering signal in-channel power received by the victim receiver.



The FOS is defined based on the scenario as follows:



If the victim receiver channel is in the OOB domain of the interfering signal: 



interfering signal and an interference scenario identified based on an ETSI harmonised standard (i.e. from Step 3):

if the interfering signal is an unmodulated continuous wave (CW) signal, the value of FOS can be determined based on the adjacent channel interfering signal limit defined in ETSI harmonised standards as follows, noting that this is a rearrangement of equation (1) and the value may have already been available:



		

		[bookmark: _Ref147493864][bookmark: _Ref131509237](7)







if the interfering signal is similar to the wanted signal, the value of FOS is determined according to equation (6) with the value of X defined according to the relevant ETSI Harmonised Standard;

In this interference configuration the impact of the interfering signal OOBE falling into the receiver channel, added to the interfering signal in-channel emission received by the receiver after filtering, is important. As the emissions falling into the receiver channel cannot be reduced by filtering, it is sensible to reduce as much as possible the interfering signal in-channel emission at the receiver input by filtering to reduce the impact of the interfering signal on the receiver performance. Therefore, setting X = 10 such that FOS=ILR + 10 dB would be an appropriate choice when deriving the adjacent-channel protection ratio of a receiver.



For all other cases (i.e. the reciprocal spurious blocking domain or interfering signal/scenario not based on an ETSI Harmonised Standard), the value of FOS is determined by equation (6) with the value of X set according to the maximum acceptable increase of the interfering signal power.



· [bookmark: _Hlk129954494]In most cases with the spurious emissions (SE) level of the interfering signal being low, X=0 dB such that FOS=ILR can be used in the calculation in order to avoid unnecessary constraint on the receiver. X=10 such that FOS=ILR+10 dB would be an appropriate choice if higher selectivity is required.



· If a CW interfering signal is used, instead of the RI signal, to calculate the blocking level of a victim receiver, the receiver FOS value should be equal to the FOS value specified for the blocking level calculation with the RI signal, since the ILR value of the CW signal is assumed to be very high or infinite (≥130 dB).



Step 5	Derive the receiver blocking level (Iblk) or adjacent channel protection ratio (PRadj-ch):

In the RSBD:



		



		[bookmark: _Ref131510215](8)





Or if the ILR of the interfering signal can be considered infinite (very high):



		

		(9)







In the OOB domain:

Calculate PRadj-ch (dB) as follows, with Iadj_ch evaluated from Equation (8) by replacing  with :

  by replacing  with :

		



		(10)







The method is illustrated in the following flowchart.







Figure 1: Flowchart describing the method for receiver resilience

[bookmark: _Ref127784908][bookmark: _Ref127785072][bookmark: _Toc280099658]resilience to transmission on adjacent frequency ranges for the broadcasting service

Digital Audio Broadcasting receivers OPERATING in the frequency range 174 – 240 MHz (VHF III)

Receiver resilience levels in the out-of-band domain 

Table 2: Receiver resilience to transmission in the out-of-band domain for DAB (1.712 MHz DAB VHF band III, wanted signal level = -70 dBm as defined in ETSI EN 303 345-4 [10])

		[bookmark: _Hlk124508105]Victim Rx type

		Interferer
Tx type

		Values used in calculations

		Values to be used in compatibility studies



		

		

		Freq offset
(MHz)

		N
(dBm)
(Note 1)

		Rx M
(dB)
(Note 2)

		ILR
(dB)
(Note 3)

		Rx FOS (dB)

		I (dBm)
(Note 5)

		C/I (dB)

		C/I (dB) defined in
ETSI EN 303 345-4 [10]



		DAB

		Wanted signal like

		1.712

		-106

		24

		55

		55
(Note 4)

		-30

		-40

		-35



		DAB

		Wanted signal like

		3.424

		-106

		24

		80

		55

		-27

		-43

		-40



		Note 1: Value calculated as shown in Table 4

Note 2: M (dB) = Wanted signal level (dBm) - Rx sens (dBm) = -70 dBm - (-94 dBm) = 24 dB

Note 3: Value calculated by numerical integration from the spectrum mask for DAB VHF transmitters in critical areas for adjacent channel interference in ETSI EN 300 401 [11]

Note 4: FOS = ILR is assumed

Note 5: Value calculated using MRR





Recommended receiver resilience levels in the reciprocal spurious blocking domain

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the blocking level of receivers in the reciprocal spurious blocking domain. An equivalent CW interfering signal blocking level has also been defined. For the purposes of conformance testing, in the reciprocal spurious blocking domain, only one of these signals is required for each test.

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the recommended blocking level of receivers in the reciprocal spurious blocking domain (RSBD). In addition, the blocking level when using a CW interferer has also been derived which provides an equivalent level of receiver blocking performance. ETSI should, based on its technical assessment, select only one of these signals when developing blocking level requirements in an individual Harmonised Standard.



’This updated text needs to be cascaded to the other annexes too.





Table 3: Recommended receiver resilience to transmission in the reciprocal spurious blocking domain for DAB (1.712 MHz DAB VHF band III, wanted signal level = -70 dBm as defined in ETSI EN 303 345-4 [10])

		[bookmark: _Hlk124508113]Victim Rx type

		Interferer
Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm) calculated from
ETSI EN 303 345-4 [10]



		

		

		Freq offset
(MHz)
(Note 1)

		N
(dBm)
(Note 2)

		Rx M
(dB)
(Note 3)

		ILR
(dB)
(Note 4)

		Rx FOS
(dB)
(Note 5)

		I (dBm)
Recommended
(Note 6)

		



		1.712 MHz DAB

		5 MHz RI

		≥13.4

		-106

		24

		75 

		75

		-10

		N/A



		1.712 MHz DAB

		CW
(Note 7) 

		≥13.4

		-106

		24

		130

		75

		-7
(Note 8)

		-30



		Note 1: Value calculated as described in Section A7.3.2

Note 2: Value calculated as shown in Table 4

Note 3: M (dB) = Wanted signal level (dBm) - Rx sens (dBm) = -70 dBm - (-94 dBm) = 24 dB

Note 4: Value calculated by numerical integration from the spectrum mask of the reference interfering signal. ILR=130 dB is assumed in the absence of spurious emissions

Note 5: FOS = ILR is assumed except when ILR=130 dB

Note 6: Value calculated using MRR

Note 7: The equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

Note 8: For DAB receivers, if an AM modulated 1 kHz sine wave signal with 80% modulation is used for blocking tests, the blocking level defined in this table should be reduced by 5 dB.

RI – Reference interfering signal





[bookmark: _Ref127786714]

Table 4: DAB VHF band III receiver noise floor, signal to noise ratio and sensitivity 

		Receiver parameter

		Value

		

		Comment



		Channel BW (MHz)

		1.712

		

		



		Effective BW (MHz)

		1.54

		

		



		Noise figure NF (dB)

		6

		

		Value defined in ETSI TR 101 758 [12]



		Noise power N (dBm)

		-106.10

		

		According to Equation (3)



		Sensitivity (Rx sens) (dBm)

		-94

		

		Value defined in ETSI EN 303 345-4 [10]



		Signal to noise ratio C/N (dB)

		12.10

		

		







Digital Terrestrial TV Broadcast receivers OPERATING in the frequency ranges 174 – 230 MHz (VHF III) and 470-694 MHz

Receiver resilience levels in the out-of-band domain

Table 5: Receiver resilience to transmission in the out-of-band domain for DTTB (8 MHz DTTB UHF band IV, wanted signal level - see Table 7)

		Victim Rx type

		Interferer Tx type

		Values used in calculations

		Values to be used in compatibility studies



		

		

		Freq offset (MHz)

		DVB-T/T2
N (dBm)
(Note 1)

		Rx M
(dB)
(Note 2)

		ILR
(dB)

		Rx FOS (dB)
(Note 3)

		I (dBm)
(Note 4)

		C/I (dB)

		C/I (dB) defined 
in ETSI EN 303 340 [13]



		DVB-T

		Wanted signal like

		8

		-101/-100

		27

		47

		47

		-30/-29

		-26

		-25



		DVB-T

		LTE700 UE

		18

		-101/-100

		19

		60

		60

		-25/-24

		-39

		-33 (Note 5) / -38



		Note 1: Value calculated as shown in Table 8

Note 2: M (dB) = Wanted signal level (dBm) - Rx sens (dBm), see Table 7 and Table 8

Note 3: FOS = ILR is assumed

Note 4: Value calculated using MRR

Note 5: ILR=51 dB is used in ETSI EN 303 340 [13] for DVB-T instead of ILR=60 dB





Recommended receiver resilience levels in the reciprocal spurious blocking domain

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the blocking level of receivers in the reciprocal spurious blocking domain. An equivalent CW interfering signal blocking level has also been defined. For the purposes of conformance testing, in the reciprocal spurious blocking domain, only one of these signals is required for each test.

Table 6: Recommended receiver resilience to transmission in the reciprocal spurious blocking domain for DTTB (8 MHz DTTB UHF band IV, wanted signal level - see Table 7)

		Victim Rx type

		Interferer
Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm) defined in ETSI EN 303 340 [13] 



		

		

		Freq offset
(MHz)

		DVB-T/T2
N (dBm)
(Note 2)

		Rx M (dB)
(Note 3)

		ILR (dB)
(Note 4)

		Rx FOS (dB)
(Note 5)

		I (dBm)
Recommended
(Note 6)

		



		8 MHz DTTB 

		5 MHz RI
(Note 7)

		≥22.5

		-101/-100

		13

		68

		68

		-23

		-25



		Note 1: Value calculated as described in Section A7.3.2

Note 2: Value calculated as shown in Table 8

Note 3: M (dB) = Wanted signal level (dBm) - Rx sens (dBm), see Table 7 and Table 8

Note 4: Value calculated by numerical integration from the spectrum mask of the reference interfering signal

Note 5: FOS = ILR is assumed

Note 6: Value calculated using MRR

Note 7:  CW interfering signal is not used in conformance tests for DTTB in the Harmonised Standard.

RI – Reference interfering signal







[bookmark: _Ref127787110]Table 7: 8 MHz DTTB wanted signal levels

		Frequency offset (MHz)

		DVB-T2 (dBm) (Note 1)

		DVB-T (dBm) (Note 2)



		± 8 

		-55

		-57



		18

		-63

		-65



		22.5 (Blocking)

		-69

		-71



		1. Level defined in ETSI HS 303 340 [13]

2. Level defined for DVB-T2 in ETSI HS 303 340 [13] decreased by 2 dB (see Table 8)







[bookmark: _Ref127787197]Table 8: DTTB receiver noise floor, signal to noise ratio and sensitivity

		Receiver parameter

		Value

		Value

		

		Comment



		Receiver type

		DVB-T

		DVB-T2

		

		



		Channel BW (MHz))

		8

		8

		

		



		Effective BW (MHz)

		7.61

		7.77

		

		



		Noise figure NF (dB)

		4

		5

		

		Value from measurements

(see ECC Report 310 [2])



		Noise power N (dBm)

		-101.16

		-100.07

		

		According to Equation (3)



		Signal to noise ratio C/N (dB)

		17

		18

		

		Value from measurements

(see ECC Report 310 [2])



		Sensitivity (Rx sens) (dBm)

		-84.16

		-82.07

		

		





[bookmark: _Ref127785068]Receiver resilience to transmission on adjacent frequency ranges for the Fixed service

FIXED SERVICE operating in the frequency range 1.4 GHz to 2.6 GHz

Receiver resilience levels in the out-of-band domain

Table 9: Receiver resilience to transmission in the out-of-band domain for the fixed service (DFRS Class 1, Class 2 and Class 4L, wanted signal level = Rx sens + Rx M)

		Victim
Rx type

		Interferer
Tx type

		Values used in calculations

		Values to be used in compatibility studies



		

		

		Freq offset (MHz)

		N
(dBm)

		Rx M
(dB)
(Note 1)

		ILR
(dB)
(Note 2)

		Rx FOS (dB)
(Note 3)

		I (dBm)
(Note 4)

		C/I (dB)

		C/I (dB) defined in ETSI EN 302 217-2 [14] 



		1.75 MHz
DFRS Class 1 / 2 / 4L

		Wanted
signal like

		1.75

		See Table 11

		1

		26 / 26 / 30
(Note 5)

		36 / 26 / 30
(Note 5)

		-86 / -90 / -85
(Note 5)

		0

		0



		1.75 MHz
DFRS Class 1 / 2 / 4L

		Wanted
signal like 

		1.75 

		See Table 11

		3

		26 / 26 / 30
(Note 5)

		36 / 26 / 30
(Note 5)

		-80 / -85 / -80
(Note 5)

		-4

		-4



		7 MHz
DFRS Class 2 / 4L

		Wanted
signal like

		7

		See Table 11

		1

		26 / 30
(Note 5)

		26 / 30
(Note 5)

		-87 / -80
(Note 5)

		-1

		0



		7 MHz
DFRS Class 2 / 4L

		Wanted
signal like

		7

		See Table 11

		3

		26 / 30
(Note 5)

		26 / 30
(Note 5)

		-81 / -74
(Note 5)

		-4

		-4



		14 MHz
DFRS Class 2 / 4L

		Wanted
signal like

		14

		See Table 11

		1

		26 / 30
(Note 5)

		26 / 30
(Note 5)

		- 81 / -76
(Note 5)

		-1

		0



		14 MHz
DFRS Class 2 / 4L

		Wanted
signal like

		14

		See Table 11

		3

		26 /30
(Note 5)

		26 / 30
(Note 5)

		- 76 / - 71
(Note 5)

		-4

		-4



		Note 1: Value defined in ETSI EN 302 217-2 [14]

Note 2: Value calculated by numerical integration from the transmitter (RF) spectrum masks defined in ETSI EN 302 217-2 [14] improved by 6 dB.

Note 3: FOS = ILR is assumed, except FOS = ILR+ 10 dB for DFRS Class 1 1.75 MHz

Note 4: Value calculated using MRR

Note 5: the „/“ is placed to differentiate the values between types of interferer. Example: ILR=26/26/30 dB leads to ILRDFRS Class 1 = 26 dB, ILRDFRS Class 2 = 26 dB and ILRDFRS Class 4L = 30 dB. Same type of logic will be applied in all cases.





 Recommended receiver resilience levels in the reciprocal spurious blocking domain

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the blocking level of receivers in the reciprocal spurious blocking domain. An equivalent CW interfering signal blocking level has also been defined. For the purposes of conformance testing, in the reciprocal spurious blocking domain, only one of these signals is required for each test.



[bookmark: _Hlk132230837]Table 10: Recommended receiver resilience to transmission in the reciprocal spurious blocking domain for the fixed service (DFRS Class 1, Class 2 and Class 4L, wanted signal level = Rx sens + Rx M)

		Victim Rx type

		Interferer
Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm) calculated
from ETSI EN 302 217-2 [14] 



		

		

		Freq offset (MHz)

		N
(dBm)

		Rx M
(dB)
(Note 1) (Note 6)

		ILR
(dB)
(Note 2)

		Rx FOS (dB)
(Note 3)

		I (dBm) Recommended
(Note 4)
(Note 6)

		



		Class 1



		1.75 MHz DFRS

		5 MHz RI

		≥13.4

		-105.5

		1

		78

		78

		-51

		N/A



		1.75 MHz DFRS

		CW
(Note 5)

		≥13.4

		-105.5

		1

		130

		78

		-48

		-56



		Class 2



		1.75 MHz DFRS

		5 MHz RI

		≥13.4

		-107.5

		1

		78

		78

		-53

		N/A



		1.75 MHz DFRS

		CW
(Note 5)

		≥13.4

		-107.5

		1

		130

		78

		-50

		-63



		7 MHz DFRS

		5 MHz RI

		≥20

		-101.5

		1

		71

		71

		-54

		N/A



		7 MHz DFRS

		CW
(Note 5)

		≥20

		-101.5

		1

		130

		71

		-51

		-57



		14 MHz DFRS

		5 MHz RI

		≥37.5

		-98.5

		1

		71

		71

		-51

		N/A



		14 MHz DFRS

		CW
(Note 5)

		≥37.5

		-98.5

		1

		130

		71

		-48

		-54



		Class 4L



		1.75 MHz DFRS

		5 MHz RI

		≥13.4

		-106.5

		1

		78

		78

		-52

		N/A



		1.75 MHz DFRS

		CW
(Note 5)

		≥13.4

		-106.5

		1

		130

		78

		-49

		-56



		7 MHz DFRS

		5 MHz RI

		≥20

		-106.5

		1

		71

		71

		-53

		N/A



		7 MHz DFRS

		CW
(Note 5)

		≥20

		-100.5

		1

		130

		71

		-50

		-50



		14 MHz DFRS

		5 MHz RI

		≥37.5

		-100.5

		1

		71

		71

		-50

		N/A



		14 MHz DFRS

		CW
(Note 5)

		≥37.5

		-100.5

		1

		130

		71

		-47

		-47



		Note 1: Value defined in ETSI EN 302 217-2 [14]

Note 2: Value calculated by numerical integration from the spectrum mask of the reference interfering signal. ILR=130 dB is assumed in the absence of spurious emissions.

Note 3: FOS = ILR is assumed except when ILR=130 dB

Note 4: Value calculated using MRR

Note 5: The equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

Note 6. For practicability M=1 dB is used for conformance tests in the related ETSI Harmonised Standard, however, it is not the value used as protection criterion in compatibility studies; therefore, M=0.041 dB is used in the calculations of I level to take into account the deployment of Fixed service in a noise limited operational environment. 

RI – Reference interfering signal







[bookmark: _Ref127787946]Table 11: Fixed service operating in the frequency range 1.4 GHz to 2.6 GHz receiver noise floor, signal to noise ratio and sensitivity for DFRS interference

		Receiver parameter

		DFRS Class 1

		DFRS Class 2

		DFRS Class 4L

		

		Comment



		Channel BW (MHz)

		1.75

		1.75

		7

		14

		1.75

		7

		14

		

		



		Effective BW  (MHz)

		1.75

		1.75

		7

		14

		1.75

		7

		14

		

		



		Noise figure NF (dB)

		6

		4

		4

		4

		5

		5

		5

		

		Value in line with those provided in ETSI TR 101 854 [15]



		Noise power N, (dB)

		-105.5

		-107.5

		-101.5

		-98.5

		-106.5

		-100.5

		-97.5

		

		According to Equation (3)



		Signal to noise ratio C/N (dB)

		18.5

		13.5

		13.5

		13.5

		19.5

		19.5

		19.5

		

		Value in line with those provided in ETSI TR 101 854 [15]



		Sensitivity (Rx sens (dBm))

		-87

		-94

		-88

		-85

		-87

		-81

		-78

		

		Value calculated as defined in ETSI EN 302 217-1 V3.2.2 [16]







[bookmark: _Ref156313128]Receiver resilience to transmission on adjacent frequency ranges for the LAND MOBILE SERVICE

MFCN IN BANDS HARMONISED FOR USE in cept in the frequency RANGE 410 mhZ to 5725 mhZ

Receiver resilience levels in the out-of-band domain for IMT BS

Table 12: Receiver resilience to transmission in the out-of-band domain for IMT (1.4, 3, 5, 10, 15 and 20 MHz E-UTRA BS channel bandwidths

		Victim Rx type

		Interferer Tx type (Note 1)

		Values used in calculations

		Values to be used in compatibility studies



		

		

		Freq offset (MHz)

		N (dBm)

		Rx M (dB) (Note 2)

		ILR (dB)

		Rx FOS (dB)

		I (dBm)
(Note 5)

		I (dBm) defined in ETSI EN 301 908-14 [17]



		Wide Area BS

		E-UTRA signal

		BWRx/2+ BWTx/2

		See 
Table 15

		11 (Note 3),
8 (Note 4)
and 6

		See Table 13 

		See Table 13 

		-52

		-52



		Local Area BS

		E-UTRA signal

		BWRx/2+ BWTx/2

		See 
Table 15

		11 (Note 3),
8 (Note 4)
 and 6

		See Table 13 

		See Table 13 

		-44

		-44



		Home BS

		E-UTRA signal

		BWRx/2+ BWTx/2

		See 
Table 15

		27 (Note 3),
24 (Note 4)
and 22

		See Table 13 

		See Table 13 

		-28

		-28



		Medium Range BS

		E-UTRA signal

		BWRx/2+ BWTx/2

		See 
Table 15

		11 (Note 3),
8 (Note 4)
and 6

		See Table 13 

		See Table 13  

		-47

		-47



		Note 1: Value calculated as described in Section A7.3.2

Note 2: M (dB) = Wanted signal level (dBm) - Rx sens (dBm)

Note 3: For BWRx=1.4 MHz

Note 4: For BWRx=3 MHz

Note 5: Value calculated using MRR







[bookmark: _Ref131248487]Table 13: FOS and ILR values used to define the IMT (E-UTRA) BS receiver selectivity levels (I) in the out-of-band domain (values defined in or derived from ETSI TS 136 141 [18])

		BWRx

		1.4 MHz

		3 MHz

		5 MHz

		10 MHz

		15 MHz

		20 MHz



		Interferer
Tx type

		ILR
(dB)

		FOS
(dB)

		ILR
(dB)

		FOS
(dB)

		ILR
(dB)

		FOS
(dB)

		ILR
(dB)

		FOS
(dB)

		ILR
(dB)

		FOS
(dB)

		ILR
(dB)

		FOS
(dB)



		E-UTRA signal (Selectivity)

		56

		46

		56

		46

		56

		46

		53

		43

		51

		41

		50

		40



		Note: According to ETSI TS 136 141 [18] the contribution from the test equipment ACLR is calculated to give a 0.4 dB additional rise in interference. This corresponds to a test equipment ACLR which is 10.2 dB better than the BS ACS (10 dB instead of 10.2 dB is used in this table)







Recommended levels of receiver resilience in the reciprocal spurious blocking domain for IMT BS

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the blocking level of receivers in the reciprocal spurious blocking domain. An equivalent CW interfering signal blocking level has also been defined. For the purposes of conformance testing, in the reciprocal spurious blocking domain, only one of these signals is required for each test.



Table 14: Recommended receiver resilience to transmission in the reciprocal spurious blocking domain for IMT (E-UTRA BS)

		Victim Rx type

		Interferer Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm) defined in ETSI EN 301 908-14 [17]



		

		

		Freq offset (MHz) (Note 1)

		N (dBm)

		Rx M (dB) (Note 2)

		ILR (dB) (Note 3)

		Rx FOS (dB) (Note 4)

		I (dBm) Recommended levels
(Note 5)

		



		

		

		

		

		

		

		

		Wide area BS

		Local area BS 

		Medium range BS

		Home BS

		



		BS (1.4, 3, 5 and 10 MHz) 

		5 MHz RI 

		≥23.2, ≥24, ≥25 and ≥27.5

		See Table 15

		6 / 14 (Note 7)

		See Table 15

		See Table 15

		-28

		-22

		-24

		-14

		N/A



		BS (1.4, 3, 5 and 10 MHz)

		CW

(Note 6)

		≥20.7, ≥21.5, ≥22.5≤ and ≥25

		See Table 15

		6 / 14 (Note 7)

		See Table 15

		See Table 15

		-18

		-18

		-18

		-11

		-15



		BS (15 MHz)

		5 MHz RI 

		≥30

		See Table 15

		6 / 14 (Note 7)

		See Table 15

		See Table 15

		-26

		-20

		-22

		-12

		N/A



		BS (15 MHz)

		CW

(Note 6)

		≥27.5

		See Table 15

		6 / 14 (Note 7)

		See Table 15

		See Table 15

		-16

		-16

		-16

		-9

		-15



		BS (20 MHz)

		5 MHz RI 

		≥32.5

		See Table 15

		6 / 14 (Note 7)

		See Table 15

		See Table 15 

		-25

		-19

		-21

		-11

		N/A



		BS (20 MHz)

		CW

(Note 6)

		≥30

		See Table 15

		6 / 14 (Note 7)

		See Table 15

		See Table 15

		-15

		-15

		-15

		-8

		-15



		Note 1:  BWRx (MHz) /2 + BWTx(RI) (MHz) /2 + 20 MHz in line with the IMT Harmonised Standard [17]

Note 2: M (dB) = Wanted signal level (dBm) – Rx sens (dBm)

Note 3: Value calculated by numerical integration from the spectrum mask of reference interfering signal. ILR=130 dB is assumed in the absence of spurious emissions

Note 4: FOS = ILR+10, +5, +2 and +0 are assumed for WA, MR, LA and H BS respectively, except when ILR=130 dB

Note 5: Value calculated using MRR

Note 6: The equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

Note 7 Only for Home BS







[bookmark: _Ref127788019][bookmark: _Ref131248625]Table 15: FOS and ILR values used to define the IMT (E-UTRA) BS receiver resilience levels (I) in the reciprocal spurious blocking domain

		FOS and ILR values used to define the IMT (E-UTRA) BS receiver blocking levels (I) in the adjacent band
ILR values derived from the 5 MHz RI signal



		Wide area BS; FOS = ILR + 10 dB (Note 1)



		BWRx

		1.4 MHz

		3 MHz

		5 MHz

		10 MHz

		15 MHz

		20 MHz



		Interferer Tx type

		ILR (dB)

		FOS (dB)

		ILR (dB)

		FOS (dB)

		ILR (dB)

		FOS (dB)

		ILR (dB)

		FOS (dB)

		ILR (dB)

		FOS (dB)

		ILR (dB)

		FOS (dB)



		5 MHz RI

		76

		86

		72

		82

		70

		80

		67

		77

		67

		77

		67

		77



		CW

		130

		86

		130

		82

		130

		80

		130

		77

		130

		77

		130

		77



		Medium range BS; FOS = ILR + 5 dB (Note 1)



		5 MHz RI

		76

		81

		72

		77

		70

		75

		67

		72

		67

		72

		67

		72



		CW

		130

		81

		130

		77

		130

		75

		130

		72

		130

		72

		130

		72



		Local area BS; FOS = ILR + 2 dB (Note 1)



		5 MHz RI

		76

		78

		72

		74

		70

		72

		67

		69

		67

		69

		67

		69



		CW

		130

		78

		130

		74

		130

		72

		130

		69

		130

		69

		130

		69



		Home BS; FOS = ILR (Note 2)



		5 MHz RI

		76

		76

		72

		72

		70

		70

		67

		67

		67

		67

		67

		67



		CW

		130

		76

		130

		72

		130

		70

		130

		67

		130

		67

		130

		67



		Note 1: Upper limit of FOS (FOS=ILR+10 dB) is used for “Wide range BS“ that has the highest selectivity according to ETSI EN 301 908-14 [17]. This value is reduced by :

- 5 dB for “Medium range BS“, which is the difference between the sensitivity of “Wide range BS“ and “Medium range BS“;

- 8 dB for “Local area BS“, which is the difference between the sensitivity of “Wide range BS“ and “Local area BS“.

Note 2: Lower limit of FOS (FOS=ILR)) is used for “Home BS“ that has the lowest selectivity according to ETSI EN 301 908-14 [17].







[bookmark: _Ref127799734]Table 16: IMT BS noise floor, signal to noise ratio and sensitivity

		IMT BS receiver N, SNR and sensitivity



		Receiver parameter

		Value

		

		Comment



		Channel BW (MHz)

		1.40

		3

		5

		10

		15

		20

		

		



		Number of RB used

		6

		15

		25

		50

		75

		100

		

		



		RB BW (MHz)

		0.18

		0.18

		0.18

		0.18

		0.18

		0.18

		

		



		Carrier offset (MHz)

		0.02

		0.02

		0.02

		0.02

		0.02

		0.02

		

		



		Effective BW (MHz)

		1.095

		2.715

		4.515

		9.015

		13.515

		18.015

		

		



		

		



		Noise figure NF (dB)

		5

		5

		5

		5

		5

		5

		

		Value defined in ETSI TS 136 141 [18]



		Noise power N (dBm)

		-108.58

		-104.64

		-102.43

		-99.43

		-97.67

		-96.42

		

		According to Equation (3)



		Signal to noise ratio C/N (dB)

		2.5

		2.3

		1.6

		-1.4

		-3.1

		-4.4

		

		Calculated 
(Rx sens - N)



		Sensitivity (Rx sens) (dBm)

		-106.08

		-102.34

		-100.83

		-100.83

		-100.77

		-100.82

		

		Values defined in ETSI EN 301 908-14 [17]



		

		



		Noise figure NF (dB)

		13

		13

		13

		13

		13

		13

		

		Calculated from the Wide Area BS reference NF of 5 dB according to the relative sensitivity degradation



		Noise power N (dBm)

		-100.58

		-96.64

		-94.43

		-91.43

		-89.67

		-88.42

		

		According to Equation (3)



		Signal to noise ratio C/N (dB)

		2.5

		2.3

		1.6

		-1.4

		-3.1

		-4.4

		

		Calculated 
(Rx sens - N)



		Sensitivity (Rx sens) (dBm)

		-98.08

		-94.34

		-92.83

		-92.83

		-92.77

		-92.82

		

		Values defined in ETSI EN 301 908-14 [17]



		

		



		Noise power N (dBm)

		-103.58

		-99.64

		-97.43

		-94.43

		-92.67

		-91.42

		

		According to Equation (3)



		Signal to noise ratio C/N (dB)

		2.5

		2.3

		1.6

		-1.4

		-3.1

		-4.4

		

		Calculated 
(Rx sens - N)



		Sensitivity (Rx sens) (dBm)

		-101.08

		-97.34

		-95.83

		-95.83

		-95.77

		-95.82

		

		Values defined in ETSI EN 301 908-14 [17]







Global System for Mobile communications receivers operating in the frequency RANGE 873 TO 1880 MHz

The receiver resilience and recommended blocking levels below are applicable to GSM 900, ER-GSM 900 and DCS 1800 Normal BTS, but are not applicable to Multi-carrier BTS (MCBTS) or Multi-Standard Radio (MSR) BS supporting GSM.

Receiver resilience levels in the out-of-band domain for GSM BTS

Table 17: Receiver resilience to transmission in the out-of-band domain for GSM BTS (GSM 900 and ER-GSM 900 Normal BTS, circuit switched channels (GMSK))

		Victim Rx type

		Interferer Tx type

		Values used in calculations

		Values to be used in compatibility studies 



		

		

		Freq offset (MHz)

		N (dBm)

		Rx M (dB) (Note 1)

		ILR (dB) (Note 2)

		Rx FOS (dB) (Note 3)

		C/I (dB)
(Note 4)

		C/I (dB) defined in ETSI EN 301 502 [19]



		Normal and Micro-BTS (M1, M2 and M3)

		GMSK modulated

		0.2

		See Table 21

		20

		21

		21

		-9

		-9



		Normal and Micro-BTS (M1, M2 and M3)

		GMSK modulated

		0.4

		See Table 21

		20

		53

		53

		-41

		-41



		Pico-BTS (P1)

		GMSK modulated

		0.2

		See Table 21

		20

		21

		21

		-9

		-5



		Pico-BTS (P1)

		GMSK modulated

		0.4

		See Table 21

		24

		53

		53

		-41

		-37



		Note 1: M (dB) = Wanted signal level (dBm) - Rx sens (dBm)

Note 2: Values defined in ECC Report 146 [8] increased by 3 dB

Note 3: FOS = ILR is assumed

Note 4: Value calculated using MRR







Table 18: Receiver resilience to transmission in the out-of-band domain for GSM BTS (GSM 900 and ER-GSM 900 Normal BTS GSM - Packet switched channels (8-PSK) - TU 50 (No FH))

		Victim Rx type

		Interferer Tx type

		Values used in calculations

		Values to be used in compatibility studies 



		

		

		Freq offset (MHz)

		N (dBm) (See Table 21)

		Rx M (dB) (Note 1)

		ILR (dB) (Note 2)

		Rx FOS (dB) (Note 3)

		I (dBm) (Note 4)

		C/I (dB)

		C/I (dB) defined in ETSI EN 301 502 [19]



		PDTCH/MCS-5

		GMSK modulated

		0.2

		-113

		20

		21

		21

		-75

		-2.5

		-2



		PDTCH/MCS-5

		GMSK modulated

		0.4

		-113

		20

		53

		53

		-43

		-34.5

		-34.5



		PDTCH/MCS-6

		GMSK modulated

		0.2

		-113

		20

		21

		21

		-75

		0

		1



		PDTCH/MCS-6

		GMSK modulated

		0.4

		-113

		20

		53

		53

		-43

		-32

		-32



		PDTCH/MCS-7

		GMSK modulated

		0.2

		-113

		20

		21

		21

		-75

		6

		8.5



		PDTCH/MCS-7

		GMSK modulated

		0.4

		-113

		20

		53

		53

		-43

		-26

		-26



		PDTCH/MCS-8

		GMSK modulated

		0.2

		-113

		20

		21

		21

		-75

		12

		9



		PDTCH/MCS-8

		GMSK modulated

		0.4

		-113

		20

		53

		53

		-43

		-20

		-20



		PDTCH/MCS-9

		GMSK modulated

		0.2

		-113

		20

		21

		21

		-75

		15

		13.5



		PDTCH/MCS-9

		GMSK modulated

		0.4

		-113

		20

		53

		53

		-43

		-17

		-17



		Note 1: M (dB) = Wanted signal level (dBm) - Rx sens (dBm)

Note 2: Values defined in ECC Report 146 [7] increased by 3 dB

Note 3: FOS = ILR is assumed

Note 4: Value calculated using MRR







Table 19: Receiver resilience to transmission in the out-of-band domain for GSM BTS (DCS 1800 Normal BTS- Packet switched channels (8-PSK) - TU 50 (No FH))

		Victim Rx type

		Interferer Tx type

		Values used in calculations

		Values to be used in compatibility studies



		

		

		Freq offset (MHz)

		N (dBm) 
(See Table 23)

		Rx M (dB) (Note 1)

		ILR (dB) (Note 2)

		Rx FOS (dB) (Note 3)

		I (dBm) (Note 4)

		C/I (dB)

		C/I (dB) defined in ETSI EN 301 502 [19]



		PDTCH
/MCS-5

		GMSK
modulated

		0.2

		-113

		20

		21

		21

		-75

		-3

		-2



		PDTCH
/MCS-5

		GMSK
modulated

		0.4

		-113

		20

		53

		53

		-43

		-35

		-35



		PDTCH
/MCS-6

		GMSK
modulated

		0.2

		-113

		20

		21

		21

		-75

		0

		1.5



		PDTCH
/MCS-6

		GMSK
modulated

		0.4

		-113

		20

		53

		53

		-43

		-32.5

		-32.5



		PDTCH
/MCS-7

		GMSK
modulated

		0.2

		-113

		20

		21

		21

		-75

		8

		10.5



		PDTCH
/MCS-7

		GMSK
modulated

		0.4

		-113

		20

		53

		53

		-43

		-24

		-24



		PDTCH
/MCS-8

		GMSK
modulated

		0.2

		-113

		20

		21

		21

		-75

		7

		10



		PDTCH
/MCS-8

		GMSK
modulated

		0.4

		-113

		20

		53

		53

		-43

		-25

		-25



		PDTCH
/MCS-9

		GMSK
modulated

		0.2

		-113

		20

		21

		21

		-75

		11

		16



		PDTCH
/MCS-9

		GMSK
modulated

		0.4

		-113

		20

		53

		53

		-43

		-20

		-21



		Note 1: M (dB) = Wanted signal level (dBm) - Rx sens (dBm)

Note 2: Values defined in ECC Report 146 [8] increased by 3 dB

Note 3: FOS = ILR is assumed

Note 4: Value calculated using MRR





Recommended levels of receiver resilience in the reciprocal spurious blocking domain for GSM BTS

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the blocking level of receivers in the reciprocal spurious blocking domain. An equivalent CW interfering signal blocking level has also been defined. For the purposes of conformance testing, in the reciprocal spurious blocking domain, only one of these signals is required for each test.



Table 20: Recommended receiver resilience values (dBm) to transmission in the reciprocal spurious blocking domain for GSM BTS (GSM 900, ER-GSM 900 and DCS 1800 Normal Micro/Pico BTS (TCH/FS))

		Victim Rx type

		Interferer Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm) defined in ETSI EN 301 502 [19]



		

		

		Freq offset (MHz)
(Note 1)

		N (dBm) (See Table 21)

		Rx M (dB) (Note 2)

		ILR (dB) (Note 3)

		Rx FOS (dB) (Note 4)

		I (dBm) Recommended value (Note 5)

		



		GSM 900 and ER-GSM 900

		5 MHz RI

		≥12.6

		-113

		3

		84

		94

		-29

		N/A



		GSM 900 and ER-GSM 900

		CW
(Note 6)

		≥12.6

		-113

		3

		130

		94

		-19

		8



		DCS 1800

		5 MHz RI

		≥12.6

		-113

		3

		88

		98

		-25

		N/A



		DCS 1800

		CW
(Note 6)

		≥12.6

		-113

		3

		130

		98

		-15

		0



		GSM 900 and ER-GSM 900 Micro (M1)

		5 MHz RI

		≥12.6

		-106

		3

		84

		94

		-22

		N/A



		GSM 900 and ER-GSM 900 Micro (M1)

		CW
(Note 6)

		≥12.6

		-106

		3

		130

		94

		-12

		-8



		DCS 1800 Micro (M2)

		5 MHz RI

		≥12.6

		-106

		3

		88

		98

		-18

		N/A



		DCS 1800 Micro (M2)

		CW
(Note 6)

		≥12.6

		-106

		3

		130

		98

		-8

		-0



		GSM 900 and ER-GSM 900 Micro (M2)

		5 MHz RI

		≥12.6

		-101

		3

		84

		94

		-17

		N/A



		GSM 900 and ER-GSM 900 Micro (M2)

		CW
(Note 6)

		≥12.6

		-101

		3

		130

		94

		-7

		8



		DCS 1800 Micro (M3)

		5 MHz RI

		≥12.6

		-101

		3

		88

		98

		-13

		N/A



		DCS 1800 Micro (M3)

		CW
(Note 6)

		≥12.6

		-101

		3

		130

		98

		-3

		0



		GSM 900 and ER-GSM 900 Micro (M3)

		5 MHz RI

		≥12.6

		-96

		3

		84

		94

		-12

		N/A



		GSM 900 and ER-GSM 900 Micro (M3)

		CW
(Note 6)

		≥12.6

		-96

		3

		130

		94

		-2

		8



		DCS 1800 Micro (M1)

		5 MHz RI

		≥12.6

		-111

		3

		88

		98

		-23

		N/A



		DCS 1800 Micro (M1)

		CW
(Note 6)

		≥12.6

		-111

		3

		130

		98

		-13

		0



		GSM 900 and ER-GSM 900 Pico (P1)

		5 MHz RI

		≥12.6

		-97

		3

		84

		94

		-13

		N/A



		GSM 900 and ER-GSM 900 Pico (P1)

		CW
(Note 6)

		≥12.6

		-97

		3

		130

		94

		-3

		8



		DCS 1800 Pico (P1)

		5 MHz RI

		≥12.6

		-104

		3

		88

		98

		-16

		N/A



		DCS 1800 Pico (P1)

		CW
(Note 6)

		≥12.6

		-104

		3

		130

		98

		-6

		0



		Note 1: 2.5BWTx (MHz) + BWRx (MHz)/2 for BWTx ≥ BWRx

Note 2: M (dB) = Wanted signal level (dBm) - Rx sens (dBm)

Note 3: Value calculated by numerical integration from the spectrum mask of reference interfering signal. ILR=130 dB is assumed in the absence of spurious emissions

Note 4: FOS = ILR + 10 dB is assumed except when ILR=130 dB

Note 5: Value calculated using MRR

Note 6: The equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

RI – Reference interfering signal







[bookmark: _Ref127788157]Table 21: GSM BTS noise floor, signal to noise ratio and sensitivity (GSM 900/ER-GSM 900/DCS 1800 Normal BTS (CS))

		Receiver parameter

		GSM 900/
DCS 1800 BTS

		GSM 900 micro‑BTS M1 and DCS 1800 micro‑BTS M2

		GSM 900 micro‑BTS M2 and DCS 1800 micro‑BTS M3

		GSM 900 micro‑BTS M3

		GSM 900 pico‑BTS P1

		DCS 1800 micro‑BTS M1

		DCS 1800 pico-BTS P1

		

		Comment



		Channel BW (MHz)

		0.20

		0.20

		0.20

		0.20

		0.20

		0.20

		0.20

		

		



		Effective BW (MHz)

		0.18

		0.18

		0.18

		0.18

		0.18

		0.18

		0.18

		

		



		Noise figure NF (dB)

		8

		15

		20

		25

		24

		10

		17

		

		Derived from Equation (3)



		Noise power (N) (dBm)

		-113

		-106

		-101

		-96

		-97

		-111

		-104

		

		Calculated

(Rx sens – C/N)



		Signal to noise ratio C/N (dB)

		9

		9

		9

		9

		9

		9

		9

		

		C/N=C/Ico-ch is assumed. C/Ico-ch values are defined in ETSI TS 145 005 [27]



		Sensitivity (Rx sens) (dBm)

		-104

		-97

		-92

		-87

		-88

		-102

		-95

		

		Values defined in ETSI EN 301 502 [19]







Table 22: GSM BTS noise floor, signal to noise ratio and sensitivity (GSM 900 and ER-GSM 900 Normal BTS (PS) TU 50 (No FH))

		Receiver parameter

		PDTCH/
MCS -5
TU 50 (No FH)

		PDTCH/
MCS -5
TU 50 (No FH)

		PDTCH/
MCS -5
TU 50 (No FH)

		PDTCH/
MCS -5
TU 50 (No FH)

		PDTCH/
MCS -5
TU 50 (No FH)

		

		Comment



		Channel BW (MHz)

		0.20

		0.20

		0.20

		0.20

		0.20

		

		



		Effective BW (MHz)

		0.18

		0.18

		0.18

		0.18

		0.18

		

		



		Noise figure NF (dB)

		8

		8

		8

		8

		8

		

		GSM 900/ DCS 1800 reference NF of 8 dB



		Noise power N (dBm)

		-113

		-113

		-113

		-113

		-113

		

		According to Equation (3)



		Signal to noise ratio C/N (dB)

		15.5

		18

		24

		30

		33

		

		C/N=C/Ico-ch is assumed. C/Ico-ch values are defined in ETSI EN 301 502 [19]



		Sensitivity (Rx sens) (dBm)

		-98

		-95

		-89

		-83

		-80

		

		Calculated 









[bookmark: _Ref127788233]Table 23: DCS noise floor, signal to noise ratio and sensitivity (TU 50 (No FH))

		Receiver parameter

		PDTCH/
MCS -5
TU 50 (No FH)

		PDTCH/
MCS -5
TU 50 (No FH)

		PDTCH/
MCS -5
TU 50 (No FH)

		PDTCH/
MCS -5
TU 50 (No FH)

		PDTCH/
MCS -5
TU 50 (No FH)

		

		Comment



		Channel BW (MHz)

		0.20

		0.20

		0.20

		0.20

		0.20

		

		



		Effective BW (MHz)

		0.18

		0.18

		0.18

		0.18

		0.218

		

		



		Noise figure NF (dB)

		8

		8

		8

		8

		8

		

		GSM 900/ DCS 1800 reference NF of 8 dB



		Noise power N (dBm)

		-113

		-113

		-113

		-113

		-113

		

		

According to Equation (3)



		Signal to noise ratio C/N (dB)

		15

		17.5

		26

		25

		29

		

		C/N=C/Ico-ch is assumed. C/Ico-ch values are defined in ETSI EN 301 502 [19]



		Sensitivity (Rx sens) (dBm)

		-98

		-95

		-87

		-88

		-84

		

		Calculated








[bookmark: _Ref127785083][bookmark: _Ref147136539]Receiver resilience to transmission on adjacent frequency ranges for SHORT RANGE DEVICES 

Short Range Devices (SRD) operating in the frequency range 30 MHz to 1000 MHz

Receiver resilience levels in the out of band domain for SRDs

Table 24: Receiver resilience to transmission in the out-of-band domain for Category 1 SRDs (Wanted signal level = Rx sens + Rx M)

		Victim Rx type (Note 1)

		Interferer Tx type

		Values used in calculations

		Values to be used in compatibility studies



		

		

		Freq offset (MHz)

		N (dBm) (Note 2)

		Rx M (dB) (Note 3)

		ILR (dB) (Note 4)

		Rx FOS (dB) (Note 5)

		I (dBm) (Note 5)

		C/I (dB) 

		I (dBm) defined in ETSI 
EN 300 220-1 [20]



		0.01 MHz SRD

		CW

		0.01

		-129

		3

		130

		79

		-50

		-54

		-50



		0.025 MHz SRD

		CW

		0.025

		-125

		3

		130

		75

		-50

		-50

		-50



		0.05 MHz SRD

		CW

		0.05

		-122

		3

		130

		78

		-44

		-53

		-44



		0.1 MHz SRD

		CW

		0.1

		-119

		3

		130

		75

		-44

		-50

		-44



		0.5 MHz SRD

		CW

		0.5

		-112

		3

		130

		68

		-44

		-43

		-44



		1.7 MHz SRD

		CW

		1.7

		-107

		3

		130

		63

		-44

		-38

		-44



		Note 1: The channel bandwidths listed above are not exhaustive. More bandwidths could be added in a future revision of this ECC Recommendation.

Note 2: Value calculated as shown in Table 27

Note 3: Value defined in ETSI EN 300 220-1 [20]

Note 4: Value assumed for CW

Note 5: Value calculated using MRR







Recommended receiver resilience levels in the reciprocal spurious blocking domain

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the blocking level of receivers in the reciprocal spurious blocking domain. An equivalent CW interfering signal blocking level has also been defined. For the purposes of conformance testing, in the reciprocal spurious blocking domain, only one of these signals is required for each test.



Table 25: Recommended receiver resilience to transmission in the reciprocal spurious blocking domain for Short Range Devices Category 1 (Wanted signal level = Rx sens + Rx M)

		Victim Rx type (Note 1)

		Interferer Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm) defined in ETSI EN 300 220-1 [20]



		

		

		Freq offset (MHz)

		N (dBm) (Note 2)

		Rx M (dB) (Note 3)

		ILR (dB) (Note 4)

		Rx FOS (dB)
(Note 5)

		I (dBm) Recommended
(Note 6)

		



		0.01 MHz SRD

		5 MHz RI

		≥12.505

		-129

		3

		97

		107.00

		-32

		N/A



		0.01 MHz SRD

		CW (Note 7)

		≥12.505

		-129

		3

		130

		107.00

		-22

		-20



		0.025 MHz SRD

		5 MHz RI

		≥12.5125

		-125

		3

		93

		103.00

		-32

		N/A



		0.025 MHz SRD

		CW (Note 7)

		≥12.5125

		-125

		3

		130

		103.00

		-22

		-20



		0.05 MHz SRD

		5 MHz RI

		≥12.525

		-122

		3

		90

		100

		-33

		N/A



		0.05 MHz SRD

		CW (Note 7)

		≥12.525

		-122

		3

		130

		100

		-22

		-20



		0.1 MHz SRD

		5 MHz RI

		≥12.55

		-119

		3

		87

		97

		-33

		N/A



		0.1 MHz SRD

		CW (Note 7)

		≥12.55

		-119

		3

		130

		97

		-22

		-20



		0.5 MHz SRD

		5 MHz RI

		≥12.75

		-112

		3

		80

		90

		-33

		N/A



		0.5 MHz SRD

		CW (Note 7)

		≥12.75

		-112

		3

		130

		90

		-22

		-20



		1.7 MHz SRD

		5 MHz RI

		≥13.35

		-107

		3

		74

		84

		-33

		N/A



		1.7 MHz SRD

		CW (Note 7)

		≥13.35

		-107

		3

		130

		84

		-22

		-20



		Note 1: The channel bandwidths listed above are not exhaustive. More bandwidths could be added in a future revision of this ECC Recommendation

Note 2: Value calculated as shown in Table 27

Note 3: Value defined in ETSI EN 300 220-1 [20]

Note 4: Value calculated by numerical integration from the spectrum mask of reference interfering signal. ILR=130 dB is assumed in the absence of spurious emissions, see ANNEX 7:

Note 5: FOS = ILR + 10 dB is assumed except when ILR≥130 dB

Note 6: Value calculated using MRR

Note 7: The equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

RI – Reference interfering signal







Table 26: Recommended receiver resilience to transmission in the reciprocal spurious blocking domain for Short Range Devices Categories 1.5 and 2 (Note 8) (Wanted signal level = Rx sens + Rx M)

		Victim Rx type (Note 1)

		Interferer
Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm) defined
in ETSI EN 300 220-1 [20] 



		

		

		Freq offset (MHz)

		N
(dBm)
(Note 2)

		Rx M
(dB)
(Note 3)

		ILR
(dB)
(Note 4)

		Rx FOS (dB)
(Note 5)

		I (dBm)
Recommended
(Note 6)

		



		0.01 MHz SRD
Category 1.5/2

		5 MHz RI

		≥12.505

		-129

		3

		97

		97

		-35

		N/A



		0.01 MHz SRD
Category 1.5/2

		CW
(Note 7)

		≥12.505

		-129

		3

		130

		97 / 85

		-32 / -44
(Note 8)

		-33 / -44(Note 7)



		0.025 MHz SRD
Category 1.5/2

		5 MHz RI

		≥12.5125

		-125

		3

		93

		93

		-35

		N/A



		0.025 MHz SRD
Category 1.5/2

		CW
(Note 7)

		≥12.5125

		-125

		3

		130

		93 / 81

		-32 / -44
(Note 8)

		-33 / -44
(Note 7)



		0.05 MHz SRD
Category 1.5/2

		5 MHz RI

		≥12.525

		-122

		3

		90

		90

		-35

		N/A



		0.05 MHz SRD
Category 1.5/2

		CW
(Note 7)

		≥12.525

		-122

		3

		130

		90 / 78

		-32 / -44
(Note 8)

		-33 / -44
(Note 7)



		0.1 MHz SRD
Category 1.5/2

		5 MHz RI

		≥12.55

		-119

		3

		87

		87

		-35

		N/A



		0.1 MHz SRD
Category 1.5/2

		CW
(Note 7)

		≥12.55

		-119

		3

		130

		87 / 75

		-32 / -44
(Note 8)

		-33 / -44
(Note 7)



		0.5 MHz SRD
Category 1.5/2

		5 MHz RI

		≥12.75

		-112

		3

		80

		80

		-35

		N/A



		0.5 MHz SRD
Category 1.5/2

		CW
(Note 7)

		≥12.75

		-112

		3

		130

		80 / 68

		-35 / -44
(Note 8)

		-33 / -44
(Note 7)



		1.7 MHz SRD
Category 1.5/2

		5 MHz RI

		≥13.35

		-107

		3

		74

		74

		-35

		N/A



		1.7 MHz SRD
Category 1.5/2

		CW
(Note 7)

		≥13.35

		-107

		3

		130

		74 / 62

		-32 / -44
(Note 8)

		-33 / -44
(Note 7)



		Note 1: The channel bandwidths listed above are not exhaustive. More bandwidths could be added in a future revision of this ECC Recommendation

Note 2: Value calculated as shown in Table 27

Note :3 Value defined in ETSI EN 300 220-1 [20]

Note :4 Value calculated by numerical integration from the spectrum mask of reference interfering signal. ILR=130 dB is assumed in the absence of spurious emissions, see ANNEX 7:

Note :5 FOS = ILR is assumed except when ILR= 130 dB

Note :6: Value calculated using MRR

Note 7: The equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

Note 8: Blocking level determined in line with ECC Report 207 [24]. FOS value is calculated using Equation (7). For compatibility studies, the blocking levels defined for SRD Category 1.5  should [also be considered] | [be applied] for SRD category 2.

RI – Reference interfering signal









[bookmark: _Ref127800289][bookmark: _Ref127800391]Table 27: SRD in the frequency range 30 MHz to 1000 MHz receiver noise floor, signal to noise ratio and sensitivity

		Receiver Parameter

		Value

		Comment



		Channel BW (MHz)

		0.01

		0.025

		0.05

		0.10

		0.50

		

		1.70

		The bandwidths shown are not an exhaustive list of what is used on the market. More bandwidths could be added in a future revision of this ECC Recommendation



		Effective BW (MHz)

		0.01

		0.025

		0.05

		0.10

		0.50

		

		1.70

		



		Noise figure NF (dB)

		5

		5

		5

		5

		5

		

		5

		Value considered



		Noise power N (dBm)

		-129

		-125

		-122

		-119

		-112

		

		-107

		 According to Equation (3)



		Sensitivity (Rx sens) (dBm)

		-107

		-103

		-100

		-97

		-90

		

		-85

		Value calculated as defined in ETSI EN 300 220-1 [20]



		Signal to noise ratio C/N (dB)

		22

		22

		22

		22

		22

		

		22

		Rx sens (dBm) – N (dBm)







[bookmark: _Ref127784913]Receiver resilience to transmission on adjacent frequency ranges for Wireless Access Systems (wAS) / Radio Local Area Networks (RLAN)

WAS/RLAN operating in the 2400 TO 2483.5 mhZ band 

Recommended receiver resilience levels in the reciprocal spurious blocking domain

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the blocking level of receivers in the reciprocal spurious blocking domain. An equivalent CW interfering signal blocking level has also been defined. For the purposes of conformance testing, in the reciprocal spurious blocking domain, only one of these signals is required for each test.

Table 28: Recommended receiver resilience to transmission in the reciprocal spurious blocking domain for 2.4 GHz WAS/RLAN (Wanted signal level = Rx sens + Rx M; Bandwidth = 20 MHz (Note 1))

		Victim Rx type

		Interferer Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm)
defined in ETSI EN 300 328 [21]



		

		

		Freq offset (MHz)

		N
(dBm)
(Note 2)

		Rx M
(dB)
(Note 3)

		ILR
(dB)
(Note 4)

		Rx FOS (dB)
(Note 5)

		I (dBm)
Recommended
(Note 6)

		



		Category 1 

		5 MHz RI

		≥ 52.5

		-91

		6

		71

		71

		-19

		N/A



		Category 1 

		CW (Note 7)

		≥ 52.5

		-91

		6

		130

		71

		-16

		-34
(Note 8)



		Category 2 

		5 MHz RI

		≥ 52.5

		-91

		6

		71

		71

		-19

		N/A



		Category 2 

		CW (Note 7)

		≥ 52.5

		-91

		6

		130

		71

		-16

		-34

(Note 9)



		Category 3 

		5 MHz RI

		≥ 52.5

		-91

		6

		71

		71

		-19

		N/A



		Category 3 

		CW (Note 7)

		≥ 52.5

		-91

		6

		130

		71

		-16

		-34

(Note 10)



		Note 1: The channel bandwidths listed above are not exhaustive. More bandwidths could be added in a future revision of this ECC Recommendation

Note 2: Value calculated as shown in Table 29

Note 3: Value defined in ETSI EN 301 893 [22] (see also notes 8 – 10)

Note 4: Value calculated by numerical integration from the spectrum mask of reference interfering signal. ILR=130 dB is assumed in the absence of spurious emissions

Note 5: FOS = ILR is assumed except when ILR= 130 dB

Note 6: Value calculated using MRR

Note 7: The equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

RI – Reference interfering signal

Note 8: This blocking level is provided from the HS considering a maximum allowable desensitisation of 26 / 20 dB. 

EN 300 328 Notes for Cat1 for different frequency offsets: NOTE 2 for 20 MHz frequency offset from the band edge: In case of radiated measurements using a companion device and the level of the wanted signal from the companion device cannot be determined, a relative test may be performed using a wanted signal up to Pmin + 26 dB where Pmin is the minimum level of wanted signal required to meet the minimum performance criteria as defined in clause 4.3.1.12.3 in the absence of any blocking signal. NOTE 3 for ≥40 MHz frequency offset from the band edge: In case of radiated measurements using a companion device and the level of the wanted signal from the companion device cannot be determined, a relative test may be performed using a wanted signal up to Pmin + 20 dB where Pmin is the minimum level of wanted signal required to meet the minimum performance criteria as defined in clause 4.3.1.12.3 in the absence of any blocking signal.

Note 9: This blocking level is provided from the HS considering a maximum allowable desensitisation of 26 dB.

EN 300 328 Notes for Cat2: NOTE 2: In case of radiated measurements using a companion device and the level of the wanted signal from the companion device cannot be determined, a relative test may be performed using a wanted signal up to Pmin + 26 dB where Pmin is the minimum level of wanted signal required to meet the minimum performance criteria as defined in clause 4.3.1.12.3 in the absence of any blocking signal.

Note 10: This blocking level is provided from the HS considering a maximum allowable desensitisation of 30 dB. 

EN 300 328 Notes for Note for Cat3: NOTE 2: In case of radiated measurements using a companion device and the level of the wanted signal from the companion device cannot be determined, a relative the test may be performed using a wanted signal up to Pmin + 30 dB where Pmin is the minimum level of wanted signal required to meet the minimum performance criteria as defined in clause 4.3.1.12.3 in the absence of any blocking signal.









[bookmark: _Ref127788623]Table 29: WAS/RLAN operating in the 2.4 GHz receiver noise floor, signal to noise ratio and sensitivity

		Receiver parameter

		2.4 GHz WAS/RLAN FHS - 1 Mbits/s

		

		Comment



		Channel BW (MHz)

		20

		

		The bandwidths shown are not an exhaustive list of what is used on the market. More bandwidths could be added in a future revision of this ECC Recommendation



		Effective BW (MHz)

		18

		

		



		Noise figure NF (dB)

		10

		

		 ECC Report 325 [9]



		Noise power N (dBm)

		-91

		

		According to Equation (3)



		Signal to noise ratio C/N (dB)

		11

		

		



		Sensitivity (Rx sens) (dB)

		-80

		

		ECC Report 325 [9]



		Wanted signal level C -Category 1 to 3 (dBm)

		-74

		

		(-139 dBm + 10 × log10(OCBW)) or -74 dBm whichever is lower







WAS/RLAN operating in the frequency ranges 5150 MHz - 5350 MHz and 5470 MHz - 5725 MHz 

Receiver resilience levels in the out-of-band domain

’EN 301 893 is currently being updated and will include blocking levels in the out-of-band domain. There may be an opportunity to update this Recommendation before publication by replacing the reference to IEEE 802.11 in Table 30 with a reference to EN 301 893.

Table 30: Receiver resilience to transmission in the out-of-band domain for 5 GHz RLAN (Wanted signal level = Rx sens + Rx M; Bandwidth = 20 MHz)

		Victim Rx type

		Interferer
Tx type

		Values used in calculations

		Values to be used in compatibility studies 



		

		

		Freq offset (MHz)

		N
(dBm)
(Note 1)

		Rx M
(dB)
(Note 2)

		ILR
(dB)
(Note 3)

		Rx FOS (dB)
(Note 4)

		I (dBm)
(Note 5)

		C/I (dB)

		C/I (dB) defined in IEEE Std 802.11-2020 [26]



		RLAN
BPSK (1/2)

		Wanted signal like

		20

		-87.00

		6

		26

		26

		-59

		-17

		-16



		RLAN
QPSK (1/2)

		Wanted signal like

		20

		-86.50

		6

		26

		26

		-59

		-14

		-13



		RLAN
QPSK (3/4)

		Wanted signal like

		20

		-87.00

		6

		26

		26

		-59

		-12

		-11



		RLAN
16-QAM (1/2)

		Wanted signal like

		20

		-86.50

		6

		26

		26

		-59

		-9

		-8



		RLAN
16-QAM (3/4)

		Wanted signal like

		20

		-85.00

		6

		26

		26

		-57

		-7

		-4



		RLAN
64-QAM (2/3)

		Wanted signal like

		20

		-83.50

		6

		26

		26

		-56

		-4

		0



		RLAN
64-QAM (3/4)

		Wanted signal like

		20

		-85.00

		6

		26

		26

		-57

		-2

		1



		Note 1: Value calculated as shown in  Table 31

Note 2: Value defined in ETSI EN 301 893 [22]

Note 3: Value calculated by numerical integration from the spectrum mask for RLAN for interference in ETSI EN 301 893 [22].

Note 4: FOS = ILR is assumed

Note 5: Value calculated using MRR







Recommended receiver resilience levels in the reciprocal spurious blocking domain

A 5 MHz OFDM interfering signal (RI signal) has been used in this ECC Recommendation to define the blocking level of receivers in the reciprocal spurious blocking domain. An equivalent CW interfering signal blocking level has also been defined. For the purposes of conformance testing, in the reciprocal spurious blocking domain, only one of these signals is required for each test.

[bookmark: _Ref147488322]Table 31: Recommended receiver resilience to transmission in the reciprocal spurious blocking domain for 5 GHz RLAN (Wanted signal level = Rx sens + Rx M; Bandwidth = 20 MHz (Note 1))

		Victim Rx type

		Interferer Tx type

		Values used in calculations

		Values recommended for Harmonised Standard

		I (dBm) defined
in ETSI EN 301 893 [22]



		

		

		Freq offset (MHz)

		N
(dBm)
(Note 2)

		Rx M
(dB)
(Note 3)

		ILR
(dB)
(Note 4)

		Rx FOS (dB)
(Note 5)

		I (dBm)
Recommended
(Note 6)

		



		RLAN
BPSK (1/2) 

		5 MHz RI

		≥52.25

		-87

		6

		71

		71

		-14

		N/A



		RLAN
BPSK (1/2)

		CW
(Note 7)

		≥52.25

		-87

		6

		130

		71

		-11

		-53



		RLAN
QPSK (1/2)

		5 MHz RI

		≥52.25

		-86.5

		6

		71

		71

		-14

		N/A



		RLAN
QPSK (1/2)

		CW
(Note 7)

		≥52.25

		-86.5

		6

		130

		71

		-11

		-53



		RLAN
QPSK (3/4)

		5 MHz RI

		≥52.25

		-87

		6

		71

		71

		-14

		N/A



		RLAN
QPSK (3/4)

		CW
(Note 7)

		≥52.25

		-87

		6

		130

		71

		-11

		-53



		RLAN
16-QAM (1/2)

		5 MHz RI

		≥52.25

		-86.5

		6

		71

		71

		-14

		N/A



		RLAN
16-QAM (1/2)

		CW
(Note 7)

		≥52.25

		-86.5

		6

		130

		71

		-11

		-53



		RLAN
16-QAM (3/4)

		5 MHz RI

		≥52.25

		-85

		6

		71

		71

		-12

		N/A



		RLAN
16-QAM (3/4)

		CW
(Note 7)

		≥52.25

		-85

		6

		130

		71

		-9

		-53



		RLAN
64-QAM (2/3)

		5 MHz RI

		≥52.25

		-83.5

		6

		71

		71

		-11

		N/A



		RLAN
64-QAM (2/3)

		CW
(Note 7)

		≥52.25

		-83.5

		6

		130

		71

		-8

		-53



		RLAN
64-QAM (3/4)

		5 MHz RI

		≥52.25

		-85

		6

		71

		71

		-12

		N/A



		RLAN
64-QAM (3/4)

		CW
(Note 7)

		≥52.25

		-85

		6

		130

		71

		-9

		-53



		Note 1: The channel bandwidths listed above are not exhaustive. More bandwidths could be added in a future revision of this ECC Recommendation

Note 2: Value calculated as shown in Table 32

Note 3: Value defined in ETSI EN 301 893 [22]

Note 4: Value calculated by numerical integration from the spectrum mask of reference interfering signal. ILR=130 dB is assumed in the absence of spurious emissions

Note 5: FOS = ILR is assumed except when ILR= 130 dB

Note 6: Value calculated using MRR

Note 7: The equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

RI – Reference interfering signal







[bookmark: _Ref127788766]Table 32: 5 GHz RLAN receiver noise floor, signal to noise ratio and sensitivity

		Receiver parameter

		Value

		

		Comment



		Modulation

		BPSK

		QPSK

		QPSK

		16-QAM

		16-QAM

		64-QAM

		64-QAM

		

		From ECC Report 325 [9]



		Coding

		1/2

		1/2

		3/2

		1/2

		3/4

		2/3

		3/4

		

		From ECC Report 325 [9]



		Channel BW (MHz)

		20

		20

		20

		20

		20

		20

		20

		

		The bandwidths shown are not an exhaustive list of what is used on the market. More bandwidths could be added in a future revision of this ECC Recommendation



		Effective BW (MHz)

		18

		18

		18

		18

		18

		18

		18

		

		From ECC Report 325 [9]



		Noise figure NF dB

		14.4

		14.9

		14.4

		14.9

		16.4

		17.9

		16.4

		

		Calculated



		Noise power N (dBm)

		-87

		-86.5

		-87

		-86.5

		-85

		-83.5

		-85

		

		

From ECC Report 325 [9]



		Signal to noise ratio C/N (dB)

		5

		7.5

		10

		12.5

		15

		17.5

		20

		

		From ECC Report 325 [9]



		Sensitivity (Rx sens) (dBm)

		-82

		-79

		-77

		-74

		-70

		-66

		-65

		

		Value defined in ECC Report 325 [9] and IEEE Std 802.11-2016 [26]







[bookmark: _Ref127785092]Reference interfering signal and a single interference scenario to be used in the calculation of recommended receiver resilience levels using MRR

Introduction

When using MRR to define the adjacent channel protection ratio (PR) and the blocking level of receivers, the relevant interfering signal and interference scenario are determined from existing or planned deployment of victim and interfering systems and the compatibility studies presented in various CEPT/ECC, ETSI and ITU-R technical reports, or, if the information in the previous reports is not available, from the intra/inter-system interfering signals and interference scenarios defined in ETSI Harmonised Standards or from the reference interfering signal and interference scenario defined in this annex.

The blocking levels defined in this Recommendation are calculated using MRR and based on the reference interfering signal (RI) described in this section. Consequently, the RI signal can be used to check the conformity of receivers with the related blocking levels. Additionally, the equivalent blocking levels for a CW interfering signal are derived in the Recommendation to allow the conformance tests to be done with a CW signal as an alternative.

[bookmark: _Ref147139462]Approach used to define a REFERENCE interfering signal

In nearly all Harmonised Standards (HS), a wanted signal like modulated adjacent channel interfering signal is used to define receiver resilience levels for intra-system compatibility.

On the other hand, for large frequency offsets going beyond the operating band of a given system/service, in most cases an unmodulated continuous wave (CW) interfering signal is used to define receiver resilience levels for inter-system compatibility based on the assumption that beyond at a large frequency offset the victim receiver FOS is constant, thus independent of the interfering signal bandwidth. This approach allows to get around the difficulty to identify case by case the most relevant interfering signal, which is not an easy task due to the more and more densely used radio spectrum.

As most HS use a CW test signal, a representative interfering signal is necessary to define receiver resilience levels for inter-system/service compatibility and to calculate receiver resilience levels using MRR.

Note: In this version of the ECC Recommendation, in order to have a consistent approach, the 5 MHz OFDM interfering signal has been used to define the receiver resilience levels in the reciprocal spurious blocking domain in Annexes 2-6.

[bookmark: _Ref127801201]Proposed reference interfering signal

The proposed reference interfering signal has the following characteristics:

Signal type: 3GPP LTE OFDM DL. , whichOFDM is the most common modulation type (e.g., IMT (LTE), DVB-T, DAB, DRM, Wireless LAN, WiMax, etc.). 

Due to the very wide range of OFDM signal generators available on the market, such a signal can easily be generated in laboratory for test purposes.	Comment by France: Alignment with the ECC Report

Bandwidth: 5 MHz: This is a compromise between potential narrowband and wideband OFDM interfering signals for the systems covered in this ECC Recommendation.

Transmission bandwidth configuration: highest transmission bandwidth allowed for uplink or downlink in a given channel bandwidth, expressed in MHz (See BWconfig and Figure 3.1-1, ETSI EN 301 908-14 V15.1.1 (2021-09)).

OOBD/SD boundary: at ± 12.5 MHz offset from the centre frequency of the interfering signal (± 250% of the interfering signal bandwidth).

OOBE level joins spurious emission (SE) level at the OOBD/SD boundary.

ILR/out-of-band (OOB) domain:

1st adjacent channel ILR (ACLR) = 48 dB/5 MHz.

Value approximated for IMT BS (44 dB) in ETSI EN 301 908-14 [17] + 3 dB margin (see Figure 3).

2nd adjacent channel ILR = 67 dB/5 MHz and 68 dB/5MHz, for frequency ranges of 9 kHz  f  1 GHz and 1 GHz < f <  6 GHz respectively.

Value obtained by the method described in Section A7.2.2.

1st adjacent channel ILR to OOBD/SD boundary = 70 dB/5 MHz and 74 dB/5 MHz, for frequency ranges of 9 kHz  f  1 GHz and 1 GHz < f <  6 GHz respectively.

Value obtained by the method described in Section A7.2.2 based on the SE (spurious emissions) limits defined in ERC Recommendation 74-01 (Annex 2, Table 6, line 2.1.1) [4]– 3 dB margin.

Spurious emission (SE) domain: 

-39 dBm/100 kHz for 9 kHz  f  1 GHz, and

-33 dBm MHz for 1 GHz < f <  6 GHz

as defined in ERC Recommendation 74-01 – 3 dB margin, (ERC Recommendation 74-01, Annex 2, Table 6, 2.1.1).

These limits are the most common limits to radio transmitters including IMT base stations. Note that the maximum SE BW is limited to 2 times of the BW of the reference interfering signal (2*BWRI) to prevent from calculating non-realistic spurious emission levels for a 5 MHz interference signal over a wide range frequency.

[bookmark: _Ref127801141]Derivation of the 5 MHz bandwidth reference interference signal spectrum mask

The spectrum mask of the reference interfering signal described in Section A7.2.1 has been derived according to the following steps:

[bookmark: _Hlk129957066]Step 1: A 5 MHz OFDM signal having a 1st adj-ch ILR (ACLR) of 47 dB/5 MHz was generated using an OFDM signal generator. The value of 47 dB/5 MHz is the IMT BS 1st adjacent ILR (ACLR) defined in ETSI EN 301 908-14 [17] increased by 3 dB. The 2nd adj-ch ILR of this signal is 64 dB/5 MHz as shown in Figure 2.



[bookmark: _Hlk147145798][bookmark: _Hlk147145849]Nevertheless, for measurement purpose, if it is difficult to generate an OFDM signal having exactly the above ILR values, it would be acceptable to generate an OFDM signal having slightly higher ILR values (max 2 dB/5 MHz). In all the cases, the ILR value measured for a given receiver channel bandwidth should be equal to the ILR value calculated from the reference interfering signal spectrum mask ±1 dB.



[image: ]

[bookmark: _Ref127788849]Figure 2: 5 MHz OFDM signal (QPSK/4-QAM modulation, PAPR = 11 dB, Spectrum analyser noise floor = -117.5 dBm/kHz)

A numerical integration method based on the trapezoidal rule has been used to calculate the ILR of the spectrum mask of the reference interfering signal as follows:

		

		[bookmark: _Ref129080741](11)





Where:

: frequency offset interfering signal in-channel power at the receiver input;

: victim receiver bandwidth;

: centre frequency of the victim receiver channel;

: frequency offset interfering signal spectral density in the linear domain.



		

		(12)







Where:

		

		(13)







The validity of the integration method was checked as presented in Table 33.

[bookmark: _Ref127788866]Table 33: Comparison of measurements and numerical integration results
(Measurement BW=5 MHz / Integration BW (4.97 MHz)) 

		Offset type

		Integration BW (MHz)

		ILR obtained
by measurement
(dB)

		ILR obtained
by numerical
integration (dB)

		Difference (dB)



		Co-channel

		4.97 MHz

		0

		0

		0



		1st adjacent channel

		4.97 MHz

		46.73

		47.27

		0.54



		2nd adjacent channel

		4.97 MHz

		63.86

		63.80

		-0.06



		2nd adjacent channel <

		4.97 MHz

		70

		70.28

		0.28







Step 2 (for 9 kHz < f ≤ 1 GHz): A reference interfering (RI) signal has been derived from the measured 5 MHz OFDM signal spectrum using linear interpolation to join the OOBE level to SE level of -36 dBm/100 kHz decreased by 3 dB at the OOBD/SD boundary. The derived spectrum mask has four break points as shown in Figure 3. The ILR of the RI signal has been calculated by numerical integration of its power (W) in a BW of 5 MHz, as described above, at three different frequency offsets as shown in Table 34.

[image: ]

[bookmark: _Ref127788930][bookmark: _Hlk129080466][bookmark: _Hlk129957147]Figure 3: Approximation of the 5 MHz OFDM signal to a reference interfering signal for 30 MHz < f < 1 GHz

[bookmark: _Ref127788918]Table 34: ILR of the reference interfering signal for 30 MHz < f < 1 GHz (60 W/5 MHz conducted Tx power normalised to 0 dBm/kHz; FNorm=11.25 dB)

		CH BW (MHz)

		Power per CH BW (W)

		Power per CH BW (dBm)

		Offset type

		ILR of the RI signal (dB)

		ILR of the measured signal (dB)



		4.97

		4.54E+00

		36.57

		Co-channel

		0

		0



		4.97

		6.61E-05

		-11.80

		1st adjacent channel

		48

		47



		4.97

		9.25E-07

		-30.34

		2nd adjacent channel

		67

		64



		4.97

		4.69E-07

		-33.29 (Note 1)

		1st adjacent channel to OOBD/SD boundary in the SD

		70

		70



		Note 1 : -33.29 dBm/5MHz+11.25 dB =-39dBm/100kHz, which is the spurious limit of -36dBm/100kHz minus a 3 dB margin







[bookmark: _Hlk129957191]Step 3 (for 1 GHz < f≤ 6 GHz): A reference interfering signal has been derived from the measured 5 MHz OFDM signal spectrum using linear interpolation to join the OOBE level to SE level of -30 dBm/MHz decreased by 3 dB at the OOBD/SD boundary. The derived spectrum mask has three break points as shown in Figure 4. The ILR of the RI signal has been calculated by numerical integration of its power (W) in its bandwidth (BW) of 5 MHz at three different frequency offsets as shown in Table 35.



[image: ]

[bookmark: _Ref127791492][bookmark: _Ref127791517]Figure 4: Approximation of the 5 MHz OFDM signal to a reference interfering signal for 1 GHz < f ≤ 6 GHz



[bookmark: _Ref148447575]Table 35: ILR of the reference interfering signal for 1 GHz < f ≤ 6 GHz (60 W/5 MHz conducted Tx power normalised to 0 dBm/kHz; FNorm=11.25 dB)

		CH BW (MHz)

		Power per CH BW (W)

		Power per CH BW (dBm)

		Offset type

		ILR of the RI signal (dB)

		ILR of the measured signal (dB)



		4.97

		4.54E+00

		36.57

		Co-channel

		0

		0



		4.97

		6.61E-05

		-11.80

		1st adjacent channel

		48

		47



		4.97

		7.22E-07

		-31.41

		2nd adjacent channel

		68

		64



		4.97

		1.87E-07

		-37.29
(Note 1)

		1st adjacent channel to OOBD/SD boundary in the SD

		74

		70



		Note 1: -37.29 dBm/5MHz+11.25 dB =-33dBm/ 1 MHz, which is the spurious limit of -30dBm/1 MHz minus a 3 dB margin





Approach used to define a single interference Scenario

[bookmark: _Ref127791675]Rationale

The rationale behind the idea to have a single interference scenario is:

To treat all the systems equally;

To define the recommended receiver resilience levels only for the most relevant frequency offsets, thus to keep the size of the future Recommendation on receivers reasonable.

[bookmark: _Ref127947152]Detailed description of the proposed interference scenario

According to the rationale presented in Section A7.3.1, it is proposed to define the receiver resilience level for different frequency offsets between the interfering transmitter and the victim receiver:

In the OOB domain, the first adjacent channel to the interfering signal, for the purpose of receiver adjacent channel PR calculation/measurement; 

In the RSBD of the interfering signal, provided that the interfering signal is beyond the second adjacent channel of the victim receiver, for the purpose of recommended receiver blocking level calculation/measurement.



The frequency offsets between the reference interfering signal (RI) centre frequency and the victim receiver (Rx) centre frequency are calculated as follows:

In the OOB domain:

Frequency offset (MHz) = BWRI/2 (MHz) + BWRx/2 (MHz)

In the RSBD:

If BWTX > BWRX 

[bookmark: _Hlk147146319]	Frequency offset (MHz) ≥ 2.5BWRI (MHz) + BWRx/2 (MHz)

Else

	Frequency offset (MHz) ≥ 2.5BWRx (MHz) + BWRI/2 (MHz)



[bookmark: _Hlk147146389][bookmark: _Hlk129957373][bookmark: _Hlk147146446]For channelised systems, the above calculations define that the victim receiver channel is in the spurious domain of the interfering transmitter and reciprocally the interfering transmitter channel is in the spurious domain of the wanted transmitter of the victim system, whatever their channel bandwidth (BW) is. For single carrier and multi-carrier receivers the bottom channel at the lower edge and the top channel at the upper edge of the receiver operating band is to be defined and tested. For receivers supporting multiple channel bandwidths, the smallest and the largest channel bandwidths should be defined and tested. For non-channelised systems the occupied bandwidth is considered as a single channel. Figure 5 illustrates this for both channelised and non-channelised systems.



Victim receiver operating band

Frequency offset

Frequency

Frequency offset

fc

fc

fc

fc

Victim receiver channel

Victim receiver channel

Reference interfering signal

Reference interfering signal



[bookmark: _Ref131520765]Figure 5: Channel of the receiver to be defined and tested

This interference scenario including the reference interferer (RI) and the two offset frequencies is depicted in Figure 6 for the example of a victim receiver having a bandwidth of 2 MHz (BWRI > BWRx).

[image: ]

[bookmark: _Ref127791762][bookmark: _Hlk129957391]Figure 6: Proposed single interference scenario to be used for all systems/services when calculating receiver resilience levels using MRR

The resulting offset frequencies from the centre of the RI to the centre frequency of the victim receiver are for the 2 MHz receiver:

3.5 MHz: 2.5 MHz (BWRI/2)) + 1 MHz (BWRx/2);

13.5 MHz: 12.5 MHz (2.5xBWRI)) + 1 MHz (BWRx/2).



The ILR values for the two offset frequencies are then to be derived by numerical integration from the spectrum masks defined in Table 36 depending on the victim receiver bandwidth according to Equation (11). Note that the maximum SE bandwidth (BW) of the RI signal is assumed to be 10 MHz (2BWTx). Consequently, for the victim receivers having a BW greater than 10 MHz the SE level of the RI should be reduced by the factor of 10log(BWRx (MHz)/10 (MHz)) when calculating/measuring ILR.

[bookmark: _Ref129080588]Table 36: Reference interfering signal relative spectrum mask

		Offset MHz

		RI signal relative spectrum mask for frequencies of 30 MHz  f  1 GHz (dB/kHz)

		RI signal relative spectrum mask for frequencies of 1 GHz < f <  6 GHz (dB/kHz)



		-17.5

		-70.25

		-74.25



		-12.5

		-70.25

		-74.25



		-10

		-70.25

		-72



		-7.5

		-62

		-62



		-2.5

		-42

		-42



		-2.25

		0

		0



		+2.25

		0

		0



		+2.5

		-42

		-42



		+7.5

		-62

		-62



		+10

		-70.25

		-72



		+12.5

		-70.25

		-74.25



		+17.5

		-70.25

		-74.25







[bookmark: _Hlk129249505]For measurement purpose, the reference interfering signal can be generated using any OFDM RF signal generator available on the market. The recommended modulation schemes are QPSK/4- QAM (max data rate  8 Mbps, sub-carrier spacing  15 kHz), with preferably a PAPR =11 ± 3 dB at the RF generator output. Higher constellations and data rates can also be used provided that the generated signal meets the interfering signal spectrum mask defined in this table as well as the PAPR value. Nevertheless, for measurement purpose, if it is difficult to generate an OFDM signal having exactly the above ILR values, it would be acceptable to generate an OFDM signal having slightly higher ILR values (max 2 dB/5 MHz). In all the cases, the ILR value measured for a given receiver channel bandwidth should be equal (± 1 dB) to the ILR value calculated from the reference interfering signal spectrum mask defined in the table. The ILR value of the generated reference interfering signal can be adjusted to the required ILR by adjusting the generator output power. If the required ILR value is not reached an external filter can be used to improve the value.

Some calculation examples are provided in the following section.

Derivation of the reference interfering signal ILR to be used in the calculation of receiver resilience levels using MRR

The ILR values of the reference interfering signal to be used to calculate the receiver resilience levels for some systems/applications have been derived by using numerical integration. These systems are DAB, DTT, IMT, Fixed service, GSM, 2.4 GHz and 5 GHz RLAN and 30 MHz-1GHz SRD. The results obtained are presented for information in Section A7.3.5.

Choice of the FOS value

Based on the definition of the reference interfering signal, it is proposed to choose the value of FOS to be used in the calculation, when using MRR to calculate the receiver resilience levels, as follows:

FOS = ILR + 10 dB, when the victim receiver channel is adjacent to the interfering transmitter channel (see Figure 6).

[bookmark: _Hlk129957460]Reason: This configuration is critical since the interfering transmitter channel is very close to the victim receiver channel resulting in a high OOB emission level in the receiver input, which cannot be reduced by filtering. Consequently, a good adjacent channel filtering would be necessary to minimise the interfering signal in-block power at the receiver input.

[bookmark: _Hlk129957468][bookmark: _Hlk147146850]FOS = ILR dB, when the victim receiver channel is adjacent to the OOBD/SD boundary of the interfering signal in its SD (see Figure 6). If a CW interfering signal is used, instead of the RI signal, to calculate the blocking level of a victim receiver, the receiver FOS value should be equal to the FOS value specified for the blocking level calculation with the RI signal, since the ILR value of the CW signal is assumed to be very high or infinite (≥130 dB).

Reason: This configuration is less critical compared to the first configuration, since the interfering transmitter channel is far away from to the victim receiver channel resulting in a lower OOB emission level in the receiver input. Consequently, it is sensible to put less constraint on the receiver FOS.

[bookmark: _Ref127791844]Derived reference interfering signal ILR to be used in the calculation of receiver resilience levels

Table 37: ILR values for systems/applications as a function of the victim channel bandwidth and frequency offset 30 MHz < f < 1 GHz

		System

		Victim channel BW (MHz) 

		Power (W)

		Power (dBm)

		Offset

		ILR (dB)



		RI

		4.97

		4.54

		36.57

		Co-channel

		0



		DAB (1.54)

		1.54

		5.05E-05

		-12.97

		1st adjacent channel

		50



		 DAB (1.54)

		1.54

		1.45E-07

		-38.37

		1st adjacent channel to OOBD/SD boundary in the SD

		75



		DTT (8 MHz)

		7.60

		5.70E-05

		-12.44

		1st adjacent channel

		49



		DTT (8 MHz) 

		7.60

		7.17E-07

		-31.44

		1st adjacent channel to OOBD/SD boundary in the SD

		68



		IMT (1.4 MHz)

		1.09

		4.21E-05

		-13.76

		1st adjacent channel

		50



		IMT (1.4 MHz)

		1.09

		1.02E-07

		-39.90

		1st adjacent channel to OOBD/SD boundary in the SD

		76



		IMT (3 MHz)

		2.70

		6.12E-05

		-12.13

		1st adjacent channel

		49



		 IMT (3 MHz)

		2.70

		2.54E-07

		-35.94

		1st adjacent channel to OOBD/SD boundary in the SD

		73



		IMT (5 MHz)

		4.52

		2.91E-05

		-15.36

		1st adjacent channel

		52



		IMT (5 MHz) 

		4.52

		4.26E-07

		-33.70

		1st adjacent channel to OOBD/SD boundary in the SD

		70



		IMT (10 MHz)

		9.00

		4.43E-05

		-13.54

		1st adjacent channel

		50



		IMT (10 MHz) 

		9.00

		8.49E-07

		-30.71

		1st adjacent channel to OOBD/SD boundary in the SD

		67



		IMT (15 MHz)

		13.51

		3.26E-05

		-14.86

		1st adjacent channel

		51



		IMT (15 MHz) 

		10.01

		9.45E-07

		-30.25

		1st adjacent channel to OOBD/SD boundary in the SD

		67



		IMT (20 MHz)

		18.03

		2.92E-05

		-15.34

		1st adjacent channel

		52



		IMT (20 MHz) 

		10.01

		9.45E-07

		-30.25

		1st adjacent channel to OOBD/SD boundary in the SD

		67



		GSM (0.2 MHz)

		0.18

		9.88E-06

		-20.05

		1st adjacent channel

		57



		 GSM (0.2 MHz)

		0.18

		1.65E-08

		-47.82

		1st adjacent channel to OOBD/SD boundary in the SD

		84



		SRD (0.1 MHz)

		0.11

		6.12E-06

		-22.13

		1st adjacent channel

		59



		SRD (0.1 MHz) 

		0.11

		9.91E-09

		-50.04

		1st adjacent channel to OOBD/SD boundary in the SD

		87



		SRD (0.5 MHz)

		0.49

		2.41E-05

		-16.17

		1st adjacent channel

		53



		 SRD (0.5 MHz)

		0.49

		4.63E-08

		-43.35

		1st adjacent channel to OOBD/SD boundary in the SD

		80



		SRD (1.7 MHz)

		1.68

		5.25E-05

		-12.80

		1st adjacent channel

		49



		SRD (1.7 MHz) 

		1.68

		1.59E-07

		-38.00

		1st adjacent channel to OOBD/SD boundary in the SD

		75



		SRD (0.05 MHz)

		0.04

		2.63E-06

		-26.77

		1st adjacent channel

		62



		SRD (0.05 MHz) 

		0.04

		4.13E-09

		-54.81

		1st adjacent channel to OOBD/SD boundary in the SD

		90



		SRD (0.025 MHz)

		0.04

		1.32E-06

		-28.81

		1st adjacent channel

		65



		 SRD (0.025 MHz)

		0.04

		2.06E-09

		-56.85

		1st adjacent channel to OOBD/SD boundary in the SD

		93







Table 38: ILR values for systems/applications as a function of the victim channel bandwidth and frequency offset 1 GHz < f ≤ 6 GHz

		System

		Victim channel BW (MHz) 

		Power (W)

		Power (dBm)

		Offset

		ILR (dB)



		RI

		4.97

		4.54

		36.57

		Co-channel

		0



		DAB (1.54 MHz)

		1.54

		6.61E-05

		-11.80

		1st adjacent channel

		50



		DAB (1.54 MHz)

		1.54

		1.87E-07

		-37.29

		1st adjacent channel to OOBD/SD boundary in the SD

		79



		DTT (8 MHz)

		7.60

		5.05E-05

		-12.97

		1st adjacent channel

		49



		DTT (8 MHz) 

		7.60

		5.79E-08

		-42.37

		1st adjacent channel to OOBD/SD boundary in the SD

		72



		IMT (1.4 MHz)

		1.09

		5.69E-05

		-12.45

		1st adjacent channel

		50



		IMT (1.4 MHz)

		1.09

		2.85E-07

		-35.44

		1st adjacent channel to OOBD/SD boundary in the SD

		80



		IMT (3 MHz)

		2.70

		4.21E-05

		-13.76

		1st adjacent channel

		49



		IMT (3 MHz)

		2.70

		4.08E-08

		-43.90

		1st adjacent channel to OOBD/SD boundary in the SD

		77



		IMT (5 MHz)

		4.52

		6.12E-05

		-12.13

		1st adjacent channel

		52



		IMT (5 MHz)

		4.52

		1.01E-07

		-39.94

		1st adjacent channel to OOBD/SD boundary in the SD

		74



		IMT (10 MHz)

		9.00

		2.91E-05

		-15.36

		1st adjacent channel

		50



		IMT (10 MHz) 

		9.00

		1.70E-07

		-37.70

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		IMT (15 MHz)

		13.51

		4.41E-05

		-13.56

		1st adjacent channel

		51



		IMT (15 MHz)

		10.01

		3.38E-07

		-34.71

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		IMT (20 MHz)

		18.03

		3.22E-05

		-14.92

		1st adjacent channel

		52



		IMT (20 MHz)

		10.01

		3.76E-07

		-34.25

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		FS (1.4 MHz)

		1.75

		2.85E-05

		-15.45

		1st adjacent channel

		49



		FS (1.4 MHz) 

		1.75

		3.76E-07

		-34.25

		1st adjacent channel to OOBD/SD boundary in the SD

		78



		FS (1.4 MHz)

		7.00

		5.34E-05

		-12.73

		1st adjacent channel

		48



		FS (1.4 MHz)

		7.00

		6.58E-08

		-41.82

		1st adjacent channel to OOBD/SD boundary in the SD

		72



		FS (1.4 MHz)

		14.00

		6.67E-05

		-11.76

		1st adjacent channel

		48



		FS (1.4 MHz)

		10.01

		2.63E-07

		-35.80

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		SRD (0.1 MHz)

		0.11

		6.70E-05

		-11.74

		1st adjacent channel

		59



		SRD (0.1 MHz)

		0.11

		3.76E-07

		-34.25

		1st adjacent channel to OOBD/SD boundary in the SD

		91



		WLAN (20 MHz)

		19.99

		6.12E-06

		-22.13

		1st adjacent channel

		50



		 WLAN (20 MHz)

		10.01

		3.95E-09

		-54.04

		1st adjacent channel to OOBD/SD boundary in the SD

		71



		DCS (0.2 MHz)

		0.18

		4.45E-05

		-13.52

		1st adjacent channel

		57



		DCS (0.2 MHz)

		0.18

		3.78E-07

		-34.23

		1st adjacent channel to OOBD/SD boundary in the SD

		88







[bookmark: _Ref147762018][bookmark: _Ref127785101] Derivation of the method for receiver resilience (MRR)  

The basic equation of MRR shown below allows to calculate the receiver blocking level as a function of N, M, ILR and FOS:

		

		(14)







This equation has been derived through the following steps. Note that the derivation is carried out in the linear domain and only the final result is transformed from the linear domain to the logarithmic domain.

The victim receiver receives both the interfering signal in-channel emissions (Iin-ch) attenuated by FOS and the interfering signal out-of-channel emissions (Ioo-ch) falling into its channel, which are defined by Iin-ch/ILR. 

		



		[bookmark: _Ref128993822][bookmark: _Ref131436789](15)





[bookmark: _Hlk129956051]where:

: frequency offset interfering signal power received by the victim receiver;

: frequency offset interfering signal power measured in its channel bandwidth at the receiver input before RF/IF/BB filtering;

: frequency offset interfering signal out-of-channel power received by the victim receiver;

: frequency offset interfering signal in-channel power received by the victim receiver. Note that when defining the resilience levels of a receiver, Iin-ch is often referred to as Iadj-ch or Iblk depending on the frequency offset between the useful and interfering signals.



The received interfering signal power causing a receiver desensitisation of M can be calculated as follows, where N is the noise floor of the receiver:

		

		[bookmark: _Ref127801507](16)







By equating Equations (15)and (16):

		

		(17)







The above equation can be rewritten as:

		

		[bookmark: _Ref127804169](18)







and then:

		

		(19) 







Transforming from the linear domain to the logarithmic domain results in:

		

		(20) 







As the values of N, M, ILR and FOS are always defined in the logarithmic domain, the above equation can be rewritten as:

		

		(21) 







which is the basic equation of MRR.

Note that the receiver frequency offset selectivity can also be expressed as a function of N, M, Iin-ch and ILR from Equation (18):

		

		(22) 







and

		

		(23) 







then

		

		(24) 







Transforming from the linear domain to the logarithmic domain results in:

		

		(25) 







and finally:

		

		[bookmark: _Toc280099660](26) 







[bookmark: _Hlk129956244]Note that when defining the resilience levels of a receiver, Iin-ch is often referred as Iadj-ch or Iblk depending on the frequency offset between the useful and interfering signals.

List of references

[bookmark: _Ref131432882][bookmark: _Ref147761346][bookmark: _Ref127784759][bookmark: _Ref213741794]ITU Radio Regulations, Edition of 2020

[bookmark: _Ref131510464][bookmark: _Ref129077747]ECC Report 310 “Evaluation of receiver parameters and the future role of receiver characteristics in spectrum management, including in sharing and compatibility studies”

[bookmark: _Ref129333943]Recommendation ITU-R SM. 1539-1 “Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU-R SM.329”

[bookmark: _Ref127784771]ERC Recommendation 74-01 “Unwanted emissions in the spurious domain”

[bookmark: _Ref127784780]ECC/REC/(19)02 “Guidance and methodologies when considering typical unwanted emissions in sharing/compatibility studies”

[bookmark: _Ref127784786]ECC Report 249 “Unwanted emissions of common radio systems: measurements and use in sharing/compatibility studies”

[bookmark: _Ref127784738]Recommendation ITU-R SM. 329-12 “Unwanted emissions in the spurious domain”

[bookmark: _Ref127800076]ECC Report 146 “Compatibility between GSM MCBTS and other services (TRR, RSBN/PRMG, HC-SDMA, GSM-R, DME, MIDS, DECT) operating in the 900 and 1800 MHz frequency bands”

[bookmark: _Ref127801053]ECC Report 325 “Compatibility and technical feasibility of coexistence studies for the potential introduction of new terrestrial applications operating in the 2483.5-2500 MHz frequency band with existing services / applications in the same band and adjacent bands”

[bookmark: _Ref127786596]ETSI EN 303 345-4 V1.1.1 “Broadcast Sound Receivers; Part 4: DAB broadcast sound service; Harmonised Standard for access to radio spectrum”

[bookmark: _Ref127786782]ETSI EN 300 401 V2.1.1 “Radio Broadcasting Systems; Digital Audio Broadcasting (DAB) to mobile, portable and fixed receivers”

[bookmark: _Ref127799304]ETSI TR 101 758 V2.1.1“Digital Audio Broadcasting (DAB); Signal strengths and receiver parameters; Targets for typical operation”

[bookmark: _Ref127799333]ETSI EN 303 340 V1.2.1“Digital Terrestrial TV Broadcast Receivers; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

[bookmark: _Ref127799513]ETSI EN 302 217-2 V3.3.1 “Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 2: Digital systems operating in frequency bands from 1,3 GHz to 86 GHz; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

[bookmark: _Ref127799635]ETSI TR 101 854 V2.1.1"Fixed Radio Systems; Point-to-point equipment; Derivation of receiver interference parameters useful for planning fixed service point-to-point systems operating different equipment classes and/or capacities”

[bookmark: _Ref127799650]ETSI EN 302 217-1 V3.2.2“Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 1: Overview, common characteristics and system-dependent requirements”

[bookmark: _Ref127799671]ETSI EN 301 908-14 V15.1.1"IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 14: Evolved Universal Terrestrial Radio Access (E-UTRA) Base Stations (BS)”

[bookmark: _Ref127799927]ETSI TS 136 141 V12.12.0"LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing (3GPP TS 36.141 version 15.4.0 Release 15)”

[bookmark: _Ref127800030]ETSI EN 301 502 V12.5.2 "Global System for Mobile communications (GSM); Base Station (BS) equipment; Harmonised Standard covering the essential requirements of article 3.2 of the Directive 2014/53/EU”

[bookmark: _Ref127800339]ETSI EN 300 220-1 V3.1.1 “Short Range Devices (SRD) operating in the frequency range 25 MHz to 1 000 MHz; Part 1: Technical characteristics and methods of measurement”

[bookmark: _Ref127800521]ETSI EN 300 328 V2.2.2“Wideband transmission systems; Data transmission equipment operating in the 2,4 GHz band; Harmonised Standard for access to radio spectrum”

[bookmark: _Ref127800954]ETSI EN 301 893 V2.1.1"5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

[bookmark: _Ref127785017]ECC Recommendation (02)05 “Unwanted emissions”

[bookmark: _Ref146803419]ECC Report 207 “Adjacent band co-existence of SRDs in the band 863-870 MHz in light of the LTE usage below 862 MHz”

[bookmark: _Ref147148575]EN 301 908-18 “IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 18: E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS) Release 15”

[bookmark: _Ref147296820][bookmark: _Ref147509205]IEEE Std 802.11-2020: Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

[bookmark: _Ref148085830]ETSI TS 145 005 V 16.1.0 (2020-11) “Digital cellular telecommunications system (Phase 2+) (GSM); GSM/EDGE Radio transmission and reception (3GPP TS 45.005 version 16.1.0 Release 16)

image4.emf

Step 2:


 


Calculate receiver 


desensitisation (M) 


Step 1:


 


Calculate Noise (N)


Step 3:


 


Determine ILR


Step 4:


 


Define the frequency offset selectivity 


(FOS)


Step 5:


 


Derive the receiver 


blocking level (I


blk


) or adjacent 


channel protection ratio (PR


adj


-


ch


) 


Yes


OOB or reciprical 


spurious domain?


Spurious


ILR considered 


infinite?


Yes


Calculate I


blk


 using 


Eq. 9


OOB


Calculate PR


adj-ch


 


using Eq. 10


No


Calculate I


blk


 using 


Eq. 8


Start


Is Noise Figure 


NF known?


Yes


Calculate N using 


Eq. 3


Calculate N using 


Eq. 4


No


Is M known?


Calculate M (Eq. 5)


No


Can M be derived 


from ETSI HS?


Yes


No


Operational 


environment 


driver?


Noise


Interference


Assume M = 15 dB Assume M = 3 dB ETSI HS


Is external info available on 


interfering system?


No


Define interfering signal 


and interference 


scenario based on ETSI 


HS


Yes


Define interfering signal and 


interference scenario based 


on external info


Info on existing/


planned deployments 


or technical reports ETSI HS


Choice of source of  interfering 


signal and interference scenario


ETSI HS


Annex 7


Define interfering signal and 


interference scenario using 


Annex 7


OOB or reciprocal 


spurious domain?


OOB


Choice of ILR 


source


External 


documents


Define ILR from 


external documents


Measurements


Define ILR from 


measurements 


EC/ECC Decisions, 


ECC/ITU-R 


Recommendations, 


CEPT/ECC Reports, 


ETSI HS/TS/TR


Spurious


Choice of ILR 


source


External 


documents


Define ILR from 


external documents


Annex 7


Define ILR from 


Annex 7


EC/ECC Decisions, 


ECC/ITU-R 


Recommendations, 


ESTI HS/TS/TR


Interfering signal and 


interference scenario based 


on ETSI HS?


No


Interfering 


signal type


Yes


Similar


 to wanted


Define X based on  


ETSI HS


ETSI HS


CW


Calculate FOS from 


Eq. 7


Define FOS 


according to Eq. 6


Define X based on  


maximum acceptable 


increase of the interfering 


signal power




Microsoft_Visio_Drawing.vsdx

Method for Receivers Resilience
Step 2: Calculate receiver desensitisation (M)
Step 1: Calculate Noise (N)
Step 3: Determine ILR
Step 4: Define the frequency offset selectivity (FOS)
Step 5: Derive the receiver blocking level (Iblk) or adjacent channel protection ratio (PRadj-ch)


Yes
OOB or reciprical spurious domain?
Spurious
ILR considered infinite?
Yes
Calculate Iblk using Eq. 9
OOB
Calculate PRadj-ch using Eq. 10
No
Calculate Iblk using Eq. 8
Start
Is Noise Figure NF known?
Yes
Calculate N using Eq. 3
Calculate N using Eq. 4
No
Is M known?
Calculate M (Eq. 5)
No
Can M be derived from ETSI HS?
Yes
No
Operational environment driver?
Noise
Interference
Assume M = 15 dB
Assume M = 3 dB
ETSI HS
Is external info available on interfering system?
No
Define interfering signal and interference scenario based on ETSI HS
Yes
Define interfering signal and interference scenario based on external info
Info on existing/planned deployments or technical reports
ETSI HS
Choice of source of  interfering signal and interference scenario
ETSI HS
Annex 7
Define interfering signal and interference scenario using Annex 7
OOB or reciprocal spurious domain?
OOB
Choice of ILR source
External 
documents
Define ILR from external documents
Measurements
Define ILR from measurements
EC/ECC Decisions, ECC/ITU-R Recommendations, CEPT/ECC Reports, ETSI HS/TS/TR
Spurious
Choice of ILR source
External 
documents
Define ILR from external documents
Annex 7
Define ILR from Annex 7
EC/ECC Decisions, ECC/ITU-R Recommendations, ESTI HS/TS/TR
Interfering signal and interference scenario based on ETSI HS?
No
Interfering signal type
Yes
Similar
 to wanted
Define X based on  ETSI HS
ETSI HS
CW
Calculate FOS from Eq. 7
Define FOS according to Eq. 6
Define X based on  maximum acceptable increase of the interfering signal power






image5.png

Spectrum

Ref Level -30.00 dem @ RBW 1kHz
SWT 3.55 VBW 10 kHz Mode Auto Sweep

-44.71 dBm
890.0000 MHz

-50 dBm—j

-60 dBm;

-70 dBm—j

-80 dBm—

-90 dBm—j

-100 dBm—

-120 dBm—

CF 890.0 MHz 1001 pts Span 35.0 MHz
Channel Power

Channel Bandwidth Offset Power
TX1 (Ref) 5.000 MHz -3.50 dBm
Tx Total -3.50 dBm
Channel Bandwidth Offset Lower Upper
Adj 5.000 MHz 5.000 MHz -46.65 dB -46.73 dB
Alt1 5.000 MHz 10.000 MHz -63.58 dB -63.86 dB

Measuring... LLLLLL







image6.png

Normalised amplitude (dBm/KHz)

5 MHz reference interfering signal spectrum mask for 30 MHz < f <1 GHz

-18 16 14 12 10 8 6 4 2 0 2 4 6 8

Frequency (MHz)

Norm Measured signal ~ —— Approximated Rl spectrum mask







image7.jpeg

Normalised amplitude (dBm/KHz)

5 MHz reference interfering signal spectrum mask for 1 GHz < f < =6 GHz

18

6 44 12 10 8 6 4 2 0 2 4 6 8 10 12 14 16
Frequency (MHz)

Measured signal spectrum  —— Approximated spectrum mask

18






image8.png

Normalised amplitude (dBm/KHz)

-20

-30

-40

-50

-60

-70

-80

Interference configuration for defining receivers resilience levels
2 MHz victim Rx and 5 MHz interfering Tx
(Interfering signal OOBD/SD boundary at + 12.5 MHz (2.5*5 MHz))

-18

16 14 12 10 8 6 4 2 0 2 4 6 8 10 12 14 16 18
Frequency (MHz)
——RIsignal spectrum for 30 MHz << 1 GHz ——RIsignal spectrum for 1 GHz <

Victim Rx - Interfering Tx freq offset = 3.5 MHz Victim Rx - Interfering Tx freq offset = 13.5 MHz.







image2.emf














image3.emf















